| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0176-v5 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0105 (AHG10: An improved VVC rate control scheme) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Yang Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | wangyang.cs@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the crosscheck results of JVET-Y0105 [1], which proposes some improvements based on the current R-lambda rate control algorithm. In JVET-Y0105, three methods are tested. Method 1 is the CTU-level bit allocation for skip and non-skip CTU. Method 2 is to set smooth window size based on the GOP size and Intra Period. In Method 3, the quality dependency factor (QDF) based rate control was extended to low frame rate. The simulation results match with those provided in JVET-Y0105.

Experimental results

JVET-Y0105 was implemented on top of VTM-14.0 [2]. The simulations were conducted under Random Access and Low Delay B configurations. The crosscheck results are shown in Table 1.

Table 1. Results of test #1

|  | Random Access Main 10 (GOP size = 32) |  |  |  |  | Low delay B Main10 |  |  |  |  | Random Access Main 10 (GOP size = 16) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.81% | -2.86% | -3.89% | 101% | 101% |  |  |  |  |  | -0.67% | 0.43% | -1.77% | 101% | 100% |
| Class A2 | -1.07% | -2.52% | -2.19% | 99% | 101% |  |  |  |  |  | -0.61% | -2.03% | -1.77% | 99% | 100% |
| Class B | -1.02% | -1.45% | -1.22% | 100% | 100% | -0.29% | -0.56% | -0.45% | 100% | 100% | -0.52% | -1.36% | -1.25% | 100% | 100% |
| Class C | -0.23% | -0.13% | -0.03% | 99% | 101% | -0.25% | -0.35% | -0.43% | 99% | 100% | 0.16% | -0.25% | -0.02% | 99% | 100% |
| Class E |  |  |  |  |  | -1.78% | -1.21% | -0.73% | 101% | 100% |  |  |  |  |  |
| Overall | -1.38% | -1.59% | -1.63% | 100% | 100% | -0.65% | -0.65% | -0.51% | 100% | 100% | -0.39% | -0.84% | -1.13% | 100% | 100% |
| Class D | -0.29% | -0.05% | -0.33% | 99% | 100% | -0.38% | -0.03% | -0.08% | 99% | 100% | -0.17% | 0.34% | 0.46% | 99% | 100% |
| Class F (Optional) | 0.79% | -1.74% | -2.09% | 102% | 100% | -2.44% | -1.33% | -2.33% | 100% | 100% | -0.57% | -0.23% | -0.02% | 101% | 100% |

Table 2. Results of test #2

|  | Random Access Main 10 (GOP size = 32) |  |  |  |  | Low delay B Main10 |  |  |  |  | Random Access Main 10 (GOP size = 16) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.20% | 0.70% | -0.83% | 101% | 100% |  |  |  |  |  | 0.19% | 0.73% | -1.33% | 101% | 100% |
| Class A2 | 0.03% | -1.63% | -1.37% | 100% | 100% |  |  |  |  |  | -0.04% | -2.13% | -1.72% | 99% | 100% |
| Class B | -0.13% | -0.88% | -0.67% | 100% | 100% | -0.08% | -0.43% | -0.38% | 100% | 100% | -0.15% | -1.28% | -1.17% | 100% | 100% |
| Class C | -0.02% | -0.61% | -0.47% | 100% | 100% | -0.03% | -0.46% | -0.34% | 100% | 100% | 0.11% | -0.28% | -0.20% | 100% | 100% |
| Class E |  |  |  |  |  | -0.39% | -0.95% | -0.18% | 101% | 101% |  |  |  |  |  |
| Overall | 0.00% | -0.64% | -0.79% | 100% | 100% | -0.14% | -0.57% | -0.31% | 100% | 100% | 0.01% | -0.78% | -1.05% | 100% | 100% |
| Class D | -0.07% | -0.33% | -0.73% | 99% | 100% | -0.24% | 0.01% | -0.24% | 99% | 101% | 0.00% | 0.03% | 0.40% | 99% | 101% |
| Class F (Optional) | -0.10% | -0.21% | -0.22% | 100% | 100% | -2.40% | -2.34% | -2.79% | 99% | 100% | 0.40% | -0.03% | 0.13% | 100% | 100% |

Table 3. Results of test #3

|  | Random Access Main 10 (GOP size = 32) |  |  |  |  | Low delay B Main10 |  |  |  |  | Random Access Main 10 (GOP size = 16) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.53% | -1.18% | -1.23% | 100% | 100% |  |  |  |  |  | -0.75% | -0.22% | -0.50% | 100% | 100% |
| Class A2 | -0.90% | -0.89% | -0.66% | 100% | 100% |  |  |  |  |  | -0.67% | -0.39% | -0.44% | 100% | 100% |
| Class B | -0.99% | -0.44% | -0.40% | 100% | 100% | -0.21% | -0.03% | 0.16% | 100% | 100% | -0.48% | -0.64% | -0.58% | 100% | 100% |
| Class C | -0.49% | -0.86% | -0.62% | 99% | 100% | -0.13% | -0.08% | -0.09% | 100% | 100% | 0.00% | 0.21% | 0.24% | 100% | 100% |
| Class E |  |  |  |  |  | -1.49% | 0.12% | -0.54% | 100% | 100% |  |  |  |  |  |
| Overall | -0.95% | -0.79% | -0.68% | 100% | 100% | -0.50% | -0.01% | -0.10% | 100% | 100% | -0.44% | -0.28% | -0.32% | 100% | 100% |
| Class D | -0.64% | -0.40% | -0.44% | 99% | 100% | -0.37% | -0.28% | -0.52% | 100% | 101% | -0.24% | 0.36% | 0.15% | 100% | 101% |
| Class F (Optional) | 3.65% | 1.26% | 1.32% | 103% | 101% | -4.14% | -3.01% | -3.49% | 100% | 100% | -1.97% | -1.61% | -1.62% | 100% | 100% |

Table 4. Results of test #4

|  | Random Access Main 10 (GOP size = 32) |  |  |  |  | Low delay B Main10 |  |  |  |  | Random Access Main 10 (GOP size = 16) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.35% | -0.47% | -2.02% | 100% | 100% |  |  |  |  |  | -0.67% | 0.43% | -1.77% | 101% | 100% |
| Class A2 | -1.07% | -2.52% | -2.19% | 99% | 101% |  |  |  |  |  | -0.61% | -2.03% | -1.77% | 99% | 100% |
| Class B | -1.02% | -1.45% | -1.22% | 100% | 100% | -0.29% | -0.56% | -0.45% | 100% | 100% | -0.52% | -1.36% | -1.25% | 100% | 100% |
| Class C | -0.47% | -1.12% | -0.90% | 99% | 100% | -0.25% | -0.35% | -0.43% | 99% | 100% | 0.16% | -0.25% | -0.02% | 99% | 100% |
| Class E |  |  |  |  |  | -1.78% | -1.21% | -0.73% | 101% | 100% |  |  |  |  |  |
| Overall | -0.95% | -1.38% | -1.49% | 99% | 100% | -0.65% | -0.65% | -0.51% | 100% | 100% | -0.39% | -0.84% | -1.13% | 100% | 100% |
| Class D | -0.60% | -1.06% | -1.09% | 99% | 100% | -0.38% | -0.03% | -0.08% | 99% | 100% | -0.17% | 0.34% | 0.46% | 99% | 100% |
| Class F (Optional) | 5.11% | 2.22% | 2.31% | 102% | 101% | -2.44% | -1.33% | -2.33% | 100% | 100% | -1.73% | -1.37% | -1.19% | 100% | 100% |

Table 5. Results of test #5

|  | Random Access Main 10 VTM-14.0-RC Over VTM-14.0 |  |  |  |  | Random Access Main 10 Extension of DQF Over VTM-14.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Campfire | 22.22% | 21.20% | 16.50% |  |  | 11.95% | 11.26% | 9.15% |  |  |
| RaceHorsesC | 2.60% | 1.02% | 0.85% |  |  | 2.52% | 3.18% | 3.54% |  |  |
| RaceHorses | 4.00% | 2.59% | 4.59% |  |  | 4.29% | 6.01% | 7.90% |  |  |
| SlideEditing | 19.21% | 11.77% | 10.67% |  |  | 18.42% | 12.08% | 11.10% |  |  |
| SlideShow | 33.38% | 19.00% | 23.06% |  |  | 29.12% | 12.91% | 14.84% |  |  |
| SlideShow (GOP16) | 57.73% | 35.13% | 35.71% |  |  | 66.79% | 40.87% | 40.50% |  |  |

Table 6. Results of test #6

|  | Random Access Main 10 (GOP size = 32) |  |  |  |  | Low delay B Main10 |  |  |  |  | Random Access Main 10 (GOP size = 16) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.03% | -3.72% | -2.91% | 100% | 101% |  |  |  |  |  | -0.75% | -0.22% | -0.50% | 100% | 100% |
| Class A2 | -0.90% | -0.89% | -0.66% | 100% | 100% |  |  |  |  |  | -0.67% | -0.39% | -0.44% | 100% | 100% |
| Class B | -0.99% | -0.44% | -0.40% | 100% | 100% | -0.21% | -0.03% | 0.16% | 100% | 100% | -0.48% | -0.64% | -0.58% | 100% | 100% |
| Class C | -0.24% | -0.13% | 0.19% | 100% | 100% | -0.13% | -0.08% | -0.09% | 100% | 100% | 0.00% | 0.21% | 0.24% | 100% | 100% |
| Class E |  |  |  |  |  | -1.49% | 0.12% | -0.54% | 100% | 100% |  |  |  |  |  |
| Overall | -1.38% | -1.10% | -0.80% | 100% | 100% | -0.50% | -0.01% | -0.10% | 100% | 100% | -0.44% | -0.28% | -0.32% | 100% | 100% |
| Class D | -0.38% | 0.51% | 0.32% | 100% | 100% | -0.37% | -0.28% | -0.52% | 100% | 101% | -0.24% | 0.36% | 0.15% | 100% | 101% |
| Class F (Optional) | 1.24% | -0.92% | -1.10% | 102% | 100% | -4.14% | -3.01% | -3.49% | 100% | 100% | -1.59% | -0.90% | -0.70% | 101% | 100% |

References

G. Ren, J. Jia, J. Wang, Z. Chen, and Z. Liu, “AHG10: An improved VVC rate control scheme,” Document of Joint Video Experts Team, JVET-Y0105, by teleconference, January 2022.

VTM-14.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-14.0.