| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0177-v1 |
| --- | --- |

| Title: | AHG 10: Enhanced deblocking settings for VTM CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn, R. Sjöberg, J. Ström, L. Litwic | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

Deblocking can provide a significant subjective benefit for VVC. This contribution proposes setting the deblocking parameter beta offset to -2 in the CTC configuration files. The beta offset controls the amount of spatial activity that is allowed until deblocking is turned off. It is asserted by the proponents that this change improves the objective performance of the deblocking filtering while maintaining the subjective quality benefit.

The objective benefit of the approach is as follows:

AI:    -0.28%Y,	-0.04%U, -0.03%V

RA:   -0.25%Y, -0.09%U, -0.12%V

LDB: -0.43%Y,	-0.03%U, -0.07%V

LDP: -0.34%Y,	0.29%U, -0.13%V

No change in encoding or decoding time.

# Introduction

Deblocking in VVC allow for control of the deblocking strength by the tC parameter and control of how much spatial activity can be present for deblocking to be applied by the beta parameter. The tC parameter mainly controls the clipping which determines how much a deblocked sample can vary from the sample value before deblocking. The beta parameter is mainly used as a basis for thresholds that spatial activities are compared to and deblocking can only be applied if the spatial activities are below thresholds based on the beta parameter.

# Proposal

We propose to make the following changes to the VTM config files for AI, RA, LDB and LDP [1]:

DeblockingFilterBetaOffset_div2: -2 (this reduces beta parameter for luma)

DeblockingFilterCbBetaOffset_div2: -2 (this reduces beta parameter for chroma component Cb)

DeblockingFilterCrBetaOffset_div2: -2 (this reduces beta parameter for chroma component Cr)

The reason to reduce the beta parameter is to make the deblocking a bit more careful to not remove true edges. We propose to not change the tC parameter to still allow for strong deblocking when needed, e.g. for more flat areas.

# Results

We provide results for the VTM CTC [2]. A significant improvement to the objective performance can be noted. The encoding and decoding time numbers provided are not accurate but expected to be similar to the anchor.

We have also verified the subjective quality on a wide range of sequences. Sometimes we see some slight benefit but mostly the proposal looks similar to the anchor.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.07% | 0.01% | 92% | 98% |
| Class A2 | -0.34% | -0.11% | -0.02% | 93% | 95% |
| Class B | -0.33% | -0.05% | -0.13% | 90% | 93% |
| Class C | -0.22% | -0.03% | -0.03% | 84% | 85% |
| Class E | -0.21% | 0.03% | 0.07% | 88% | 89% |
| Overall | -0.28% | -0.04% | -0.03% | 89% | 92% |
| Class D | -0.17% | 0.05% | 0.01% | 85% | 84% |
| Class F | -0.13% | 0.00% | 0.01% | 89% | 89% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | -0.06% | -0.13% | 91% | 92% |
| Class A2 | -0.33% | -0.02% | -0.08% | 94% | 91% |
| Class B | -0.27% | -0.17% | -0.28% | 100% | 108% |
| Class C | -0.19% | -0.05% | 0.05% | 101% | 111% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.09% | -0.12% | 97% | 102% |
| Class D | -0.17% | 0.06% | -0.04% | 101% | 114% |
| Class F | -0.17% | -0.08% | -0.20% | 100% | 113% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | 0.12% | -0.14% | 91% | 92% |
| Class C | -0.24% | -0.31% | 0.07% | 95% | 102% |
| Class E | -0.75% | 0.10% | -0.14% | 94% | 94% |
| Overall | -0.43% | -0.03% | -0.07% | 93% | 96% |
| Class D | -0.11% | 0.51% | 0.66% | 84% | 82% |
| Class F | -0.30% | -0.06% | -0.19% | 92% | 99% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.32% | 0.08% | -0.53% | 93% | 99% |
| Class C | -0.19% | 0.29% | -0.25% | 88% | 93% |
| Class E | -0.57% | 0.61% | 0.71% | 87% | 94% |
| Overall | -0.34% | 0.29% | -0.13% | 90% | 96% |
| Class D | -0.09% | 0.29% | 0.61% | 88% | 88% |
| Class F | -0.15% | -0.50% | -0.38% | 89% | 93% |

# Conclusion

It this contribution we have shown that it can be beneficial to set the deblocking beta offsets config parameters to -2.

We therefore propose to set the beta offset parameters equal to -2 in the VTM CTC configuration files.

# References

[1] VTM-15.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, January 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).