| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0181-v1 |
| --- | --- |

| Title: | AHG12: CABAC initialization from previous inter slice |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Dong Nan Hu Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com jiedong@qti.qualcomm.com nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports test results for CABAC initialization for inter slices by using probabilities stored from the center CTU of the previous slice having the same type and slice QP.

# Introduction

Utilizing previous coded slices for CABAC initialization was used in JEM, proposed in JVET-K0379 and studied in VVC CE5.

Specifically, if the current slice type is a B or P, the probability state of each context model is first obtained after coding CTUs up to a specified location and stored. Then, the stored probability state will be used as the initial probability state for the corresponding context model in the next B- or P-slice coded with the same quantization parameter (QP). The CTU location for storing probability states is computed using the following formula:

CTU location = min ((W + C)/2 + 1, C).

where W denote the number of CTUs in a CTU line, and C is the total number of CTUs in a slice.

# Results

The proposed method was implemented on top of ECM-3.1 and tested under ECM CTC. Run time is inaccurate as anchor and test were run on heterogeneous cluster.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | 0.22% | 0.18% | 84% | 85% |
| Class C | -0.17% | 0.08% | 0.28% | 77% | 83% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.37% | 0.14% | 0.20% | 74% | 78% |
| Class F | -0.23% | -0.20% | -0.19% | 79% | 82% |
| Class TGM | -0.52% | -0.58% | -0.56% | 85% | 86% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.30% | -0.07% | 0.89% | 83% | 86% |
| Class E | -0.13% | 0.52% | 0.28% | 80% | 85% |
| Overall |  |  |  |  |  |
| Class D | -0.50% | 1.32% | 1.56% | 75% | 87% |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).