| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0181-v3 |
| --- | --- |

| Title: | AHG12: CABAC initialization from previous inter slice |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Dong Nan Hu Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com jiedong@qti.qualcomm.com nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution first reports test results for CABAC initialization for inter slices by using probabilities stored from the center CTU of the previous slice having the same type and slice QP. This contribution also proposes a method to adjusting the window size for each context model, such that the probability estimation adapts better to the local statistics. Experimental results show that this method working together with temporal CABAC reduces BD-rate -0.09% AI, -0.29% RA, and -0.32%* LDB.

# Introduction

## Temporal CABAC

Utilizing previous coded slices for CABAC initialization was used in JEM, proposed in JVET-K0379 and studied in VVC CE5.

Specifically, if the current slice type is a B or P, the probability state of each context model is first obtained after coding CTUs up to a specified location and stored. Then, the stored probability state will be used as the initial probability state for the corresponding context model in the next B- or P-slice coded with the same quantization parameter (QP). The CTU location for storing probability states is computed using the following formula:

CTU location = min ((W + C)/2 + 1, C).

where W denote the number of CTUs in a CTU line, and C is the total number of CTUs in a slice.

## Adaptive Window Size Adjustment

The CABAC used in ECM employs two probability states that are updated with a short and a long window size, respectively. The window sizes, pre-defined for each context model, are not optimal for varying statistics in different regions. This contribution proposes to adjust the window sizes according to the preceding M bins. Specifically, as shown in Fig. 1, given a current bin to be coded in regular mode (in blue) and the context model used for the bin, the preceding M bins that are processed by the same context model and stored before were retrieved.

Fig. 1. M-bin memory used to determine the window size adjustment for the current bin

Then, the integer of which the binary representation is exactly the M-bin memory is used as the index to search a pre-defined look-up table (LUT) to get the adjustment parameters: delta0 for short window and delta1 for the long window. Denote the original short and long window sizes defined for the context model as shift0 and shift1, respectively. The actual window sizes used to code the current bin after adjustment is (shift0+delta0) and (shift1+delta1). Since there are 2M possible combinations for M bins, the size of the LUT for a context model is 2M. In this contribution, M is equal to 12, so the size of the LUT is 4096. Considering there are 492 context models used in the current version of ECM and each context model has its own LUT, we store a 2-D LUT (see Fig. 2), which uses the M-bin memory and the context model index to search for the right window size adjustment parameter.

Fig. 2-D LUT for searching window size adjustment parameters

The ranges of delta0 and delta1 are both from -7 to 7, and the actual window sizes (shift0+delta0) and (shift1+delta1) are guaranteed to be no less than 2. delta0 and delta1 each has 15 valid values, so each can be represented by a 4-bit fixed length code. Then delta0 and delta1 are packed into one byte as shown in Fig. 2 and stored in one entry of the 2-D LUT.

# Results

The proposed method was implemented on top of ECM-3.1 and tested under ECM CTC. Run time is inaccurate as anchor and test 2 were run on heterogeneous cluster.

Table 1 shows the BD-rate reduction for temporal CABAC, and Table 2 shows the BD-rate reduction for the joint use of temporal CABAC and adaptive window size adjustment.

Table 1 BD-rate reduction for temporal CABAC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | 0.10% | -0.22% | 100% | 99% |
| Class A2 | -0.06% | 0.03% | -0.06% | 99% | 98% |
| Class B | -0.19% | 0.22% | 0.18% | 100% | 100% |
| Class C | -0.17% | 0.08% | 0.28% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | 0.12% | 0.08% | 99% | 100% |
| Class D | -0.37% | 0.14% | 0.20% | 100% | 101% |
| Class F | -0.23% | -0.20% | -0.19% | 98% | 100% |
| Class TGM | -0.52% | -0.58% | -0.56% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.26% | 0.27% | 0.56% | 102% | 101% |
| Class C | -0.30% | -0.07% | 0.89% | 101% | 100% |
| Class E | -0.13% | 0.52% | 0.28% | 103% | 101% |
| Overall | -0.24% | 0.22% | 0.60% | 102% | 101% |
| Class D | -0.50% | 1.32% | 1.56% | 99% | 104% |
| Class F | -0.43% | -0.05% | -0.41% | 99% | 99% |
| Class TGM | -0.88% | -1.03% | -1.08% | 96% | 99% |

Table 2 BD-rate reduction for the joint use of temporal CABAC and adaptive window size adjustment

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.01% | 0.07% | 88% | 91% |
| Class A2 | -0.07% | 0.07% | 0.08% | 92% | 92% |
| Class B | -0.09% | -0.01% | -0.11% | 94% | 81% |
| Class C | -0.09% | -0.14% | -0.06% | 97% | 87% |
| Class E | -0.12% | -0.15% | -0.12% | 103% | 88% |
| Overall | -0.09% | -0.05% | -0.04% | 95% | 87% |
| Class D | -0.06% | -0.02% | -0.15% | 103% | 91% |
| Class F | -0.20% | -0.25% | -0.19% | 93% | 81% |
|  | Random access Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | 0.10% | -0.17% | 92% | 90% |
| Class A2 | -0.21% | -0.05% | 0.02% | 94% | 91% |
| Class B | -0.31% | 0.18% | 0.26% | 89% | 82% |
| Class C | -0.28% | 0.10% | 0.53% | 90% | 86% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | 0.09% | 0.20% | 91% | 86% |
| Class D | -0.46% | 0.47% | 0.24% | 103% | 92% |
| Class F | -0.34% | -0.36% | -0.45% | 96% | 85% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.38% | -0.18% | 0.99% | 78% | 78% |
| Class E | -0.24% | 0.98% | -0.42% | 72% | 75% |
| Overall |  |  |  |  |  |
| Class D | -0.67% | 0.94% | 2.32% | 77% | 81% |
| Class F | -0.40% | -1.10% | -1.00% | 80% | 82% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).