| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0191 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0130: “EE2-related: Unification of availability check for intra mode coding” |  |  |
| --- | --- | --- | --- |
| Status: | Crosscheck Report |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mehdi Salehifar 8910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | mehdi.salehifar@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results of JVET-Y0130. The simulation results reportedly match those provided by the proponents.

# Verification

In JVET-Y0130, the unification of usage for neighbor intra modes above the CTU boundary is suggested. Currently, in ECM, the neighbor intra modes can be used in MPM and TIMD process. In VVC, neighboring intra modes above the CTU boundary are not exploited for MPM process to avoid the necessity of storing the intra modes in line buffer memory. However, in ECM, the above mentioned restriction still remains in some parts, but above neighbor intra modes are used as the candidates of the current block for some processes, such as TIMD and in the case of the intra mode propagation from the inter blocks (IPM). Therefore, it is proposed to unify the usage for neighboring intra modes above the CTU boundary. Two unified methods are introduced. In Method 1, the above intra modes in the different CTUs are always restricted as VVC. Method 2 allows to use the intra modes or intra modes from the IPM buffer from the above, above-right, or above-left neighbor blocks, regardless of the CTU boundaries.

The coding results under the ECM CTC for methods 1 and 2 are summarized in Table 1 and 2 respectively.

Table 1: Performance of proposed method 1 over ECM-3.1

Table 2: Performance of proposed method 2 over ECM-3.1

The BD rate differences match with the results from the proposal [1]. Note that timing results are not accurate in this report, since the crosscheck tests and anchor test were launched in different time slots with different simulation environments.

# References

[1] S. Yoo, H. Jang, J. Nam, J. Choi, J. Lim, S. Kim, “EE2-related : Unification of availability check for intra mode coding”, JVET-Y0130, January 2022.