| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0200-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0163 (AHG 7: GDR without encoder constraints for ECM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Tangi Poirier Karam Naser | Email: | tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the cross of JVET-Y0163 (AHG 7: GDR without encoder constraints for ECM).
It is reported that the results in JVET-Y0163 are confirmed.

# Introduction

JVET-Y0163 [1] propose a new implementation for GDR (Gradual Decoding Refresh), the previous GDR implementation needs to check for each mode that no information from a dirty region is used to decode a CU in a clean region. This implies a lot of development for the new tools from ECM. In this contribution it is proposed to set the pixels in dirty areas as “not available” for CUs in clean area, and if needed they are set to 	2BD-1  , where BD is bit depth, which will give prediction for CUs in clean area of the current picture more freedom over the current picture and reference pictures. For CUs in clean area of a current picture, coding information in dirty areas of the current picture and reference pictures are considered “not available”, which will prevent CUs in clean area of the current picture from using coding information in dirty area of the current picture and reference pictures.

# Analysis

The new GDR implementation doesn’t need to check the validity of intra modes or motion vectors used by the clean region by setting the pixels in dirty area as unavailable at both encoder and decoder. It also keeps 2 version of reference frames, one used by the dirty region where all information is available, as shown in Figure 1 and one for clean region where the dirty region as been set to a default value, Figure 2.

Figure 1: reference frame used by dirty region

Figure 2: reference frame used by clean region

The proposed method is faster to implement code-wise but needs more buffer for the prediction in the current implementation, in an alternative implementation more checks are needed at the decoder. Whereas VTM-like implementation from JVET-Y1062, doesn’t need checks at decoder but need to adapt each tools individually.

# Results

Table 1: Simulation results of the proposed method over ECM-base

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1_All_off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.27% | 4.76% | 6.71% | 104% | 113% |
| Class C | 1.18% | 3.43% | 3.82% | 103% | 113% |
| Class E | 5.28% | 12.26% | 13.27% | 101% | 118% |
| Overall | 2.24% | 6.19% | 7.39% | 103% | 114% |
| Class D | 1.69% | 5.88% | 5.86% | 103% | 117% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class TGM | 2.76% | 3.19% | 2.83% | 100% | 117% |

Table 2: Simulation results of proposed mehod over ECM-3.1 with 7 tools disabled

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1_7_off |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | 1.31% | 4.36% | 4.35% | 101% | 111% |
| Class E | 6.00% | 12.97% | 13.79% | 101% | 118% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 2.02% | 6.88% | 6.91% | 99% | 114% |
| Class F | 1.37% | 4.46% | 6.57% | 107% | 120% |
| Class TGM | 2.29% | 4.87% | 4.62% | 84% | 118% |

The results match those provided in JVET-Y0163.

# Conclusion

Results of  JVET-Y0163 have been confirmed.

# Reference

[1] L. Wang, S. Hong, K. Panusopone, M. M. Hannuksela, J. Lainema (Nokia), “AHG 7: GDR without encoder constraints for ECM”, JVET-Y0163, Jan. 2022

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).