| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0212-v1 |
| --- | --- |

| Title: | AHG4: REV Result for AHG11/EE1 and AHG10/Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Wien, RWTH Aachen University | Tel: Email: | +49-241-80-27867 wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Abstract

Remote expert viewing tests were conducted during the 25th JVET meeting for the exploration activity on EE1/DNN-based coding tools and deblocking filter tuning. The tests were performed using mp4 files provided by the proponents with a total of 60 comparison for EE1 and 36 comparisons in the deblocking category. Sequences of 720p and 1080p contents were evaluated for both EE1 and deblocking, and an additional set of UHD sequences was evaluated for EE1 only. Calls for participation in the subjective viewing were issued on the JVET reflector. Overall, 25 experts volunteered in participating in the tests. The testing procedure followed the draft SC29/AG5 guidelines for remote experts viewing, providing an A/B comparison of sequences under test.

In the deblocking filter experiment the figures indicate that the visual performance impact of the proposal is negligible for both, the VTM and the ECM test cases. In the ECM case, two visually significant differences are reported on the ArenaOfValour sequence, with the indication of a significant benefit on the one QP point and the indication of the significant disadvantage on the other QP point. It is noted that JVET-Y0085 reports REV results performed by the proponents themselves. It is found that there is a significant congruency between the two experiments.

For the EE1-1.1 experiment, a significant visual benefit is reported for 7 out of 18 test cases. In one additional case (KristenAndSara QP34), the confidence interval is only very slightly touching the zero line. In all other cases, the zero line is included in the CI and therefore, comparable visual performance is suggested.

In the EE1-1.2 experiment, significant visual benefit is reported for 10 out of 26 test cases. The benefits seem to be more prominent on the UHD sequences. In two cases (Kimono1 QP37 and FourPeople QP42), a significant disadvantage is observed.

In the super-resolution category (EE1-2.x), the largest visual benefits are observed compared to the base line VTM. The gains appear to be higher by tendency for the NN-based method.

# Introduction

Remote expert viewing tests were conducted during the 25th JVET meeting for the exploration activity on EE1/DNN-based coding tools [2] and deblocking filter tuning [3]. The tests were performed using mp4 files provided by the proponents with a total of 60 comparison for EE1 and 36 comparisons in the deblocking category. Sequences of 720p and 1080p contents were evaluated for both EE1 and deblocking, and an additional set of UHD sequences was evaluated for EE1 only.

Calls for participation in the subjective viewing were issued on the JVET reflector. Overall, 25 experts volunteered in participating in the tests. The testing procedure followed the draft SC29/AG5 guidelines for remote experts viewing, providing an A/B comparison of sequences under test [1]. Tests were conducted on Monday, 2022-01-10 (13 participants) and Tuesday, 2022-01-11 (12 participants).

It is noted that document JVET-Y0085 includes results for an internal remote experts viewing (REV) conducted by the proponents [4]. The results are found to be consistent to a high degree.

# Test arrangement

Sequences of 5s duration were used for evaluation. For all but the Campfire sequence, the first 5s were assessed. For the Tango sequence, the duration is slightly below 5s.

The following EE1 tests were suggested to be included into viewing:

Table 1: EE1 tests suggested for the viewing

| EE1 Test | Proposal | Viewing categories |
| --- | --- | --- |
| 1.1 | JVET-X0055 | UHD, 1080p, 720p |
| 1.2.1 | JVET-X0066 | UHD, 1080p, 720p |
| 2.1 | JVET-X0117 | UHD |
| 2.3.1 | JVET-X0064 | UHD |

The sequence compared in the EE1 category are listed below:

Table 2: EE1 Pairs of mp4 files assessed in the visual test

| ./EE1-anchor/FourPeople_1280x720_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.1/FourPeople_1280x720_60fps_8bit_420_qp34.mp4 |
| --- | --- |
| ./EE1-anchor/FourPeople_1280x720_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.1/FourPeople_1280x720_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/Jets_1280x720_60fps_bt709_420_0_5_qp34.mp4 | ./EE1-1.1/Jets_1280x720_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/Jets_1280x720_60fps_bt709_420_0_5_qp39.mp4 | ./EE1-1.1/Jets_1280x720_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/Spincalendar_1280x720_60fps_bt709_420_0_5_qp34.mp4 | ./EE1-1.1/Spincalendar_1280x720_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/Spincalendar_1280x720_60fps_bt709_420_0_5_qp39.mp4 | ./EE1-1.1/Spincalendar_1280x720_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/KristenAndSara_1280x720_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.1/KristenAndSara_1280x720_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/KristenAndSara_1280x720_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.1/KristenAndSara_1280x720_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/Johnny_1280x720_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.1/Johnny_1280x720_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/Johnny_1280x720_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.1/Johnny_1280x720_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/Spincalendar_1280x720_60fps_bt709_420_0_5_qp34.mp4 | ./EE1-1.2.1/Spincalendar_1280x720_60fps_bt709_420_qp34.mp4 |
| ./EE1-anchor/Spincalendar_1280x720_60fps_bt709_420_0_5_qp39.mp4 | ./EE1-1.2.1/Spincalendar_1280x720_60fps_bt709_420_qp39.mp4 |
| ./EE1-anchor/Jets_1280x720_60fps_bt709_420_0_5_qp34.mp4 | ./EE1-1.2.1/Jets_1280x720_60fps_bt709_420_qp34.mp4 |
| ./EE1-anchor/Jets_1280x720_60fps_bt709_420_0_5_qp39.mp4 | ./EE1-1.2.1/Jets_1280x720_60fps_bt709_420_qp39.mp4 |
| ./EE1-anchor/FourPeople_1280x720_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.2.1/FourPeople_1280x720_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/FourPeople_1280x720_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.2.1/FourPeople_1280x720_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/KristenAndSara_1280x720_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.2.1/KristenAndSara_1280x720_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/KristenAndSara_1280x720_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.2.1/KristenAndSara_1280x720_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/Johnny_1280x720_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.2.1/Johnny_1280x720_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/Johnny_1280x720_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.2.1/Johnny_1280x720_60fps_8bit_420_qp39.mp4 |

| ./EE1-anchor/redkayak_1920x1080_30Hz_8bit_420_0_5_qp34.mp4 | ./EE1-1.1/redkayak_1920x1080_30fps_8bit_420_qp34.mp4 |
| --- | --- |
| ./EE1-anchor/redkayak_1920x1080_30Hz_8bit_420_0_5_qp39.mp4 | ./EE1-1.1/redkayak_1920x1080_30fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/ArenaOfValor_1920x1080_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.1/ArenaOfValor_1920x1080_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/ArenaOfValor_1920x1080_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.1/ArenaOfValor_1920x1080_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/BQTerrace_1920x1080_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.1/BQTerrace_1920x1080_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/BQTerrace_1920x1080_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.1/BQTerrace_1920x1080_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/Kimono1_1920x1080_24_0_5_qp34.mp4 | ./EE1-1.1/Kimono1_1920x1080_30fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/Kimono1_1920x1080_24_0_5_qp39.mp4 | ./EE1-1.1/Kimono1_1920x1080_30fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/redkayak_1920x1080_30Hz_8bit_420_0_5_qp34.mp4 | ./EE1-1.2.1/redkayak_1920x1080_30fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/redkayak_1920x1080_30Hz_8bit_420_0_5_qp39.mp4 | ./EE1-1.2.1/redkayak_1920x1080_30fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/BQTerrace_1920x1080_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.2.1/BQTerrace_1920x1080_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/BQTerrace_1920x1080_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.2.1/BQTerrace_1920x1080_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/ArenaOfValor_1920x1080_60fps_8bit_420_0_5_qp34.mp4 | ./EE1-1.2.1/ArenaOfValor_1920x1080_60fps_8bit_420_qp34.mp4 |
| ./EE1-anchor/ArenaOfValor_1920x1080_60fps_8bit_420_0_5_qp39.mp4 | ./EE1-1.2.1/ArenaOfValor_1920x1080_60fps_8bit_420_qp39.mp4 |
| ./EE1-anchor/Kimono1_1920x1080_24_0_5_qp34.mp4 | ./EE1-1.2.1/Kimono1_1920x1080_24_qp34.mp4 |
| ./EE1-anchor/Kimono1_1920x1080_24_0_5_qp39.mp4 | ./EE1-1.2.1/Kimono1_1920x1080_24_qp39.mp4 |

| ./EE1-anchor/Tango_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-1.2.1/Tango_3840x2160_60fps_10bit_420_qp39.mp4 |
| --- | --- |
| ./EE1-anchor/Tango_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-1.2.1/Tango_3840x2160_60fps_10bit_420_qp42.mp4 |
| ./EE1-anchor/DaylightRoad2_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-1.2.1/DaylightRoad2_3840x2160_60fps_10bit_420_qp39.mp4 |
| ./EE1-anchor/DaylightRoad2_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-1.2.1/DaylightRoad2_3840x2160_60fps_10bit_420_qp42.mp4 |
| ./EE1-anchor/CatRobot_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-1.2.1/CatRobot_3840x2160_60fps_10bit_420_qp39.mp4 |
| ./EE1-anchor/CatRobot_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-1.2.1/CatRobot_3840x2160_60fps_10bit_420_qp42.mp4 |
| ./EE1-anchor/Campfire_3840x2160_30fps_10bit_420_qp36.mp4 | ./EE1-1.2.1/Campfire_3840x2160_30fps_10bit_420_qp36.mp4 |
| ./EE1-anchor/Campfire_3840x2160_30fps_10bit_420_qp39.mp4 | ./EE1-1.2.1/Campfire_3840x2160_30fps_10bit_420_qp39.mp4 |
| ./EE1-anchor/Campfire_3840x2160_30fps_10bit_420_qp36.mp4 | ./EE1-2.1/Campfire_3840x2160_30fps_10bit_420_qp28.mp4 |
| ./EE1-anchor/Campfire_3840x2160_30fps_10bit_420_qp39.mp4 | ./EE1-2.1/Campfire_3840x2160_30fps_10bit_420_qp32.mp4 |
| ./EE1-anchor/CatRobot_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-2.1/CatRobot_3840x2160_60fps_10bit_420_qp33.mp4 |
| ./EE1-anchor/CatRobot_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-2.1/CatRobot_3840x2160_60fps_10bit_420_qp36.mp4 |
| ./EE1-anchor/Tango_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-2.1/Tango_3840x2160_60fps_10bit_420_qp32.mp4 |
| ./EE1-anchor/Tango_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-2.1/Tango_3840x2160_60fps_10bit_420_qp35.mp4 |
| ./EE1-anchor/DaylightRoad2_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-2.1/DaylightRoad2_3840x2160_60fps_10bit_420_qp33.mp4 |
| ./EE1-anchor/DaylightRoad2_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-2.1/DaylightRoad2_3840x2160_60fps_10bit_420_qp36.mp4 |
| ./EE1-anchor/Campfire_3840x2160_30fps_10bit_420_qp36.mp4 | ./EE1-2.3.1/Campfire_3840x2160_30fps_10bit_420_qp28.mp4 |
| ./EE1-anchor/Campfire_3840x2160_30fps_10bit_420_qp39.mp4 | ./EE1-2.3.1/Campfire_3840x2160_30fps_10bit_420_qp32.mp4 |
| ./EE1-anchor/CatRobot_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-2.3.1/CatRobot_3840x2160_60fps_10bit_420_qp33.mp4 |
| ./EE1-anchor/CatRobot_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-2.3.1/CatRobot_3840x2160_60fps_10bit_420_qp36.mp4 |
| ./EE1-anchor/Tango_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-2.3.1/Tango_3840x2160_60fps_10bit_420_qp32.mp4 |
| ./EE1-anchor/Tango_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-2.3.1/Tango_3840x2160_60fps_10bit_420_qp35.mp4 |
| ./EE1-anchor/DaylightRoad2_3840x2160_60fps_10bit_420_qp39.mp4 | ./EE1-2.3.1/DaylightRoad2_3840x2160_60fps_10bit_420_qp33.mp4 |
| ./EE1-anchor/DaylightRoad2_3840x2160_60fps_10bit_420_qp42.mp4 | ./EE1-2.3.1/DaylightRoad2_3840x2160_60fps_10bit_420_qp36.mp4 |

In the deblocking filter category, a visual investigation of the proposal was requested for JVET-X0063 [3]. The compared mp4 files are listed below:

Table 3: Deblocking pairs of mp4 files assessed in the visual test

| ./JVET-Y0085_ECM_DBToff/Johnny_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/Johnny_1280x720_60fps_10bit_420_qp37.mp4 |
| --- | --- |
| ./JVET-Y0085_ECM_DBToff/Johnny_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/Johnny_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_ECM_DBToff/Spincalendar_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/Spincalendar_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_ECM_DBToff/Spincalendar_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/Spincalendar_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_ECM_DBToff/FourPeople_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/FourPeople_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_ECM_DBToff/FourPeople_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/FourPeople_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_ECM_DBToff/KristenAndSara_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/KristenAndSara_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_ECM_DBToff/KristenAndSara_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/KristenAndSara_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_ECM_DBToff/Jets_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/Jets_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_ECM_DBToff/Jets_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/Jets_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/Spincalendar_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/Spincalendar_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/Spincalendar_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/Spincalendar_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/Jets_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/Jets_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/Jets_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/Jets_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/FourPeople_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/FourPeople_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/FourPeople_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/FourPeople_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/KristenAndSara_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/KristenAndSara_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/KristenAndSara_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/KristenAndSara_1280x720_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/Johnny_1280x720_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/Johnny_1280x720_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/Johnny_1280x720_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/Johnny_1280x720_60fps_10bit_420_qp42.mp4 |

| ./JVET-Y0085_ECM_DBToff/ArenaOfValor_1920x1080_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/ArenaOfValor_1920x1080_60fps_10bit_420_qp37.mp4 |
| --- | --- |
| ./JVET-Y0085_ECM_DBToff/ArenaOfValor_1920x1080_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/ArenaOfValor_1920x1080_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_ECM_DBToff/redkayak_1920x1080_30fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/redkayak_1920x1080_30fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_ECM_DBToff/redkayak_1920x1080_30fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/redkayak_1920x1080_30fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_ECM_DBToff/Kimono1_1920x1080_24fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/Kimono1_1920x1080_24fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_ECM_DBToff/Kimono1_1920x1080_24fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/Kimono1_1920x1080_24fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_ECM_DBToff/BQTerrace_1920x1080_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_ECM_DBTon/BQTerrace_1920x1080_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_ECM_DBToff/BQTerrace_1920x1080_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_ECM_DBTon/BQTerrace_1920x1080_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/BQTerrace_1920x1080_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/BQTerrace_1920x1080_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/BQTerrace_1920x1080_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/BQTerrace_1920x1080_60fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/Kimono1_1920x1080_24fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/Kimono1_1920x1080_24fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/Kimono1_1920x1080_24fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/Kimono1_1920x1080_24fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/redkayak_1920x1080_30fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/redkayak_1920x1080_30fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/redkayak_1920x1080_30fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/redkayak_1920x1080_30fps_10bit_420_qp42.mp4 |
| ./JVET-Y0085_VTM_DBToff/ArenaOfValor_1920x1080_60fps_10bit_420_qp37.mp4 | ./JVET-Y0085_VTM_DBTon/ArenaOfValor_1920x1080_60fps_10bit_420_qp37.mp4 |
| ./JVET-Y0085_VTM_DBToff/ArenaOfValor_1920x1080_60fps_10bit_420_qp42.mp4 | ./JVET-Y0085_VTM_DBTon/ArenaOfValor_1920x1080_60fps_10bit_420_qp42.mp4 |

# Test procedure

## Test session setup

The viewing compared the reconstruction quality of the proposals under test to the reconstruction quality of the VTM anchor. It was held as an online session via Zoom with the participants having downloaded the (anonymized) mp4 files beforehand in a password protected zip file. The password was provided to the participants before the start of the viewing.

In the following, video A and video B denote either the anchor or the proposal from the pairs of mp4 files listed in Table 1. The assignment of anchor and proposal to A or B was randomized and not revealed to the test subjects. The set of tests was split into five viewing sessions (1× UHD with 28 votes, 2× 1080p with 20 votes, and 2× 720p with 24 votes).

Each basic test cell (BTC) was constructed as follows:

- “Original” (on a mid-gray background, 1 s presentation)

- Original Video

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “Vote N” (on a mid-gray background, 5 s presentation)

The test subjects were asked to rate the video sequences presented in each test cell as follows:

- “A was much better than B”

- “A was better than B”

- “B was better than A”

- “B was much better than A”

The used voting sheet is presented in the figure below. The participants were supposed to fill the voting sheet and enter their results into an online form after completion of the session. The participants were also asked to enter the used screen size and any specific observations in a comment field of the online form.

The voting sheets were used in order to simplify the workload for the participants during the session.

Figure 1: Voting sheet

Each viewing session contained a stabilization phase of BTCs to be discarded and an additional test point comparing the original sequence to a coded version.

The viewers were advised to view the UHD content at 1.5H and the HD content at 3H, with H representing the screen height.

Before the start of the test sessions, the viewers were provided a demo playlist in order to familiarize themselves with the procedure of the test. The viewers were verbally instructed about the procedure and the rating scale in the introduction of the session. They were asked not to stop the video during the test sessions but to use the assigned voting time to record their judgement.

The test sessions were run in the order of decreasing length, i.e. UHD, 720p, and 1080p.

## Voting scores

For evaluation, the votes of the test subjects were mapped to MOS numbers as follows:

- +3	: “Proposal much better than anchor”

- +1	: “Proposal better than anchor”

- -1	: “Anchor better than proposal”

- -3	: “Anchor much better than proposal”

From these values, the MOS and the 95% confidence intervals were calculated. A resulting MOS value of zero indicates that anchor and proposal were rated to have the same visual quality. The scores of the four participants with the lowest Pearson correlation were removed from the evaluation.

## Participants comments

Some participants commented that it was difficult to see differences. One participant commented that the length of 5s would not be sufficient esp. for the ultra-high-resolution content.

# Test results

The MOS values and the corresponding confidence intervals (CI) resulting from the evaluation are reported in Table 4. According to the assignment of voting scores, a positive number indicates a visual benefit for the proposal over the anchor while a negative number indicates the proposal to be inferior to the anchor.

The scores of the 25 participants have not been further processed with the following exemptions: The votes of participants voting the original inferior to the coded version were not regarded in the final evaluation; two single votes were excluded as they were obvious outliers; due to a difficulty with one 720p playlist, no voting results were available for a single BTC for the first 13 participants (Spincalendar QP37).

In the table, the ECM and VTM tests refer to the deblocking filter experiment. The EE1-* tests denote the DNN-related results. The ‘orig’ test represents the assessment of the original against a coded version.

Table 4: MOS results for the A/B comparisons under test

| Test | Name | MOS | CI |  | Test | Name | MOS | CI |  | Test | Name | MOS | CI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ECM | ArenaOfValor QP37 | 0.71 | 0.49 |  | EE1-1.1 | ArenaOfValor QP34 | 0.62 | 0.58 |  | EE1-2.1 | Campfire QP36 | 1.38 | 0.64 |
| ECM | ArenaOfValor QP42 | -0.52 | 0.37 |  | EE1-1.1 | ArenaOfValor QP39 | 0.71 | 0.49 |  | EE1-2.1 | Campfire QP39 | 1.76 | 0.50 |
| ECM | BQTerrace QP37 | -0.05 | 0.51 |  | EE1-1.1 | BQTerrace QP34 | 0.62 | 0.44 |  | EE1-2.1 | CatRobot QP39 | 0.05 | 0.44 |
| ECM | BQTerrace QP42 | 0.05 | 0.44 |  | EE1-1.1 | BQTerrace QP39 | 0.52 | 0.46 |  | EE1-2.1 | CatRobot QP42 | 0.33 | 0.49 |
| ECM | FourPeople QP37 | -0.24 | 0.57 |  | EE1-1.1 | FourPeople QP34 | 0.43 | 0.55 |  | EE1-2.1 | DaylightRoad2 QP39 | -0.43 | 0.48 |
| ECM | FourPeople QP42 | -0.20 | 0.52 |  | EE1-1.1 | FourPeople QP39 | -0.40 | 0.70 |  | EE1-2.1 | DaylightRoad2 QP42 | 0.33 | 0.49 |
| ECM | Jets QP37 | -0.30 | 0.51 |  | EE1-1.1 | Jets QP34 | 0.14 | 0.58 |  | EE1-2.1 | Tango QP39 | 0.43 | 0.55 |
| ECM | Jets QP42 | -0.43 | 0.55 |  | EE1-1.1 | Jets QP39 | 0.05 | 0.75 |  | EE1-2.1 | Tango QP42 | 1.10 | 0.50 |
| ECM | Johnny QP37 | 0.05 | 0.58 |  | EE1-1.1 | Johnny QP34 | 0.60 | 0.46 |  | EE1-2.3.1 | Campfire QP36 | 1.76 | 0.57 |
| ECM | Johnny QP42 | 0.00 | 0.53 |  | EE1-1.1 | Johnny QP39 | 0.05 | 0.58 |  | EE1-2.3.1 | Campfire QP39 | 1.76 | 0.57 |
| ECM | Kimono1 QP37 | 0.05 | 0.51 |  | EE1-1.1 | Kimono1 QP34 | 0.33 | 0.62 |  | EE1-2.3.1 | CatRobot QP39 | 0.90 | 0.50 |
| ECM | Kimono1 QP42 | -0.24 | 0.50 |  | EE1-1.1 | Kimono1 QP39 | 0.62 | 0.44 |  | EE1-2.3.1 | CatRobot QP42 | 0.90 | 0.50 |
| ECM | KristenAndSara QP37 | -0.33 | 0.49 |  | EE1-1.1 | KristenAndSara QP34 | 0.60 | 0.61 |  | EE1-2.3.1 | DaylightRoad2 QP39 | 0.62 | 0.58 |
| ECM | KristenAndSara QP42 | 0.14 | 0.51 |  | EE1-1.1 | KristenAndSara QP39 | 0.30 | 0.51 |  | EE1-2.3.1 | DaylightRoad2 QP42 | 0.14 | 0.64 |
| ECM | redkayak QP37 | -0.24 | 0.50 |  | EE1-1.1 | redkayak QP34 | 0.14 | 0.43 |  | EE1-2.3.1 | Tango QP39 | 0.62 | 0.51 |
| ECM | redkayak QP42 | -0.05 | 0.44 |  | EE1-1.1 | redkayak QP39 | 0.71 | 0.49 |  | EE1-2.3.1 | Tango QP42 | 1.10 | 0.50 |
| ECM | Spincalendar QP37 | 0.00 | 0.53 |  | EE1-1.1 | Spincalendar QP34 | -0.33 | 0.41 |  |  |  |  |  |
| ECM | Spincalendar QP42 | -0.24 | 0.57 |  | EE1-1.1 | Spincalendar QP39 | -0.05 | 0.51 |  |  |  |  |  |
| VTM | ArenaOfValor QP37 | 0.33 | 0.56 |  | EE1-1.2.1 | ArenaOfValor QP34 | 0.33 | 0.56 |  | Test | Name | MOS | CI |
| VTM | ArenaOfValor QP42 | -0.43 | 0.55 |  | EE1-1.2.1 | ArenaOfValor QP39 | 1.00 | 0.47 |  | Orig | ArenaOfValor QP0 | -2.43 | 0.40 |
| VTM | BQTerrace QP37 | -0.05 | 0.58 |  | EE1-1.2.1 | BQTerrace QP34 | 0.05 | 0.51 |  | Orig | BQTerrace QP0 | -2.43 | 0.40 |
| VTM | BQTerrace QP42 | -0.14 | 0.58 |  | EE1-1.2.1 | BQTerrace QP39 | 0.52 | 0.66 |  | Orig | DaylightRoad2 QP0 | -2.24 | 0.50 |
| VTM | FourPeople QP37 | 0.24 | 0.50 |  | EE1-1.2.1 | Campfire QP36 | 1.38 | 0.64 |  | Orig | FourPeople QP0 | -2.71 | 0.31 |
| VTM | FourPeople QP42 | -0.40 | 0.41 |  | EE1-1.2.1 | Campfire QP39 | 1.57 | 0.55 |  | Orig | Johnny QP0 | -2.68 | 0.34 |
| VTM | Jets QP37 | 0.24 | 0.50 |  | EE1-1.2.1 | CatRobot QP39 | 0.90 | 0.57 |  |  |  |  |  |
| VTM | Jets QP42 | -0.40 | 0.41 |  | EE1-1.2.1 | CatRobot QP42 | 1.00 | 0.47 |  |  |  |  |  |
| VTM | Johnny QP37 | 0.24 | 0.57 |  | EE1-1.2.1 | DaylightRoad2 QP39 | 0.71 | 0.31 |  |  |  |  |  |
| VTM | Johnny QP42 | -0.33 | 0.56 |  | EE1-1.2.1 | DaylightRoad2 QP42 | 1.00 | 0.54 |  |  |  |  |  |
| VTM | Kimono1 QP37 | 0.14 | 0.43 |  | EE1-1.2.1 | FourPeople QP34 | -0.33 | 0.68 |  |  |  |  |  |
| VTM | Kimono1 QP42 | 0.33 | 0.49 |  | EE1-1.2.1 | FourPeople QP39 | -0.60 | 0.54 |  |  |  |  |  |
| VTM | KristenAndSara QP37 | -0.30 | 0.43 |  | EE1-1.2.1 | Jets QP34 | 0.10 | 0.45 |  |  |  |  |  |
| VTM | KristenAndSara QP42 | -0.05 | 0.51 |  | EE1-1.2.1 | Jets QP39 | 0.20 | 0.66 |  |  |  |  |  |
| VTM | redkayak QP37 | 0.05 | 0.44 |  | EE1-1.2.1 | Johnny QP34 | 0.33 | 0.41 |  |  |  |  |  |
| VTM | redkayak QP42 | -0.24 | 0.50 |  | EE1-1.2.1 | Johnny QP39 | 0.40 | 0.58 |  |  |  |  |  |
| VTM | Spincalendar QP37 | 0.09 | 0.62 |  | EE1-1.2.1 | Kimono1 QP34 | -0.90 | 0.53 |  |  |  |  |  |
| VTM | Spincalendar QP42 | 0.00 | 0.45 |  | EE1-1.2.1 | Kimono1 QP39 | -0.62 | 0.75 |  |  |  |  |  |
|  |  |  |  |  | EE1-1.2.1 | KristenAndSara QP34 | 0.00 | 0.60 |  |  |  |  |  |
|  |  |  |  |  | EE1-1.2.1 | KristenAndSara QP39 | 0.52 | 0.76 |  |  |  |  |  |
|  |  |  |  |  | EE1-1.2.1 | redkayak QP34 | 0.33 | 0.41 |  |  |  |  |  |
|  |  |  |  |  | EE1-1.2.1 | redkayak QP39 | 0.62 | 0.51 |  |  |  |  |  |
|  |  |  |  |  | EE1-1.2.1 | Spincalendar QP34 | 0.40 | 0.50 |  |  |  |  |  |
|  |  |  |  |  | EE1-1.2.1 | Spincalendar QP39 | 0.52 | 0.66 |  |  |  |  |  |
|  |  |  |  |  | EE1-1.2.1 | Tango QP39 | 0.62 | 0.58 |  |  |  |  |  |
|  |  |  |  |  | EE1-1.2.1 | Tango QP42 | 0.81 | 0.66 |  |  |  |  |  |

Figure 2: MOS plots

In the above figure, the results for the different test categories are aggregated. The different experiments are marked in the figure. It is noted that in the super-resolution category, EE1-2.1 refers to the VTM RPR-based approach while EE1-2.3.1 referes to NN-based super-resolution.

As noted earlier, document JVET-Y0085 includes results for an internal remote experts viewing (REV) conducted by the proponents [4]. The results of the two evaluations are compared in the table below. If the results show the same tendency and have overlapping confidence intervals, the test is marked as “Consistent” (14 cases). If the tendency is (potentially only slightly) diverging but the confidence intervals overlap, this is marked with “Overlap” (11 cases). Finally, if the confidence intervals do not overlap, this is marked by “Diverging” (3 cases). Given that the tests were done in diverse, uncontrolled environments, it is concluded that both experiments show a significant congruency (25 out of 28 test cases, 89%).

Table 5: MOS results for the A/B comparisons under test

| Name | VTM | ECM |
| --- | --- | --- |
| ArenaOfValor QP37 | Overlap | Diverging |
| ArenaOfValor QP42 | Consistent | Consistent |
| BQTerrace QP37 | Overlap | Overlap |
| BQTerrace QP42 | Consistent | Overlap |
| FourPeople QP37 | Consistent | Consistent |
| FourPeople QP42 | Overlap | Consistent |
| Johnny QP37 | Overlap | Consistent |
| Johnny QP42 | Consistent | Diverging |
| Kimono1 QP37 | Diverging | Consistent |
| Kimono1 QP42 | Consistent | Overlap |
| KristenAndSara QP37 | Overlap | Overlap |
| KristenAndSara QP42 | Overlap | Consistent |
| redkayak QP37 | Consistent | Consistent |
| redkayak QP42 | Overlap | Consistent |

# Conclusions

Based on the reported MOS values, the following conclusions are drawn:

In the deblocking filter experiment the figures indicate that the visual performance impact of the proposal is negligible for both, the VTM and the ECM test cases. In the ECM case, two visually significant differences are reported on the ArenaOfValour sequence, with the indication of a significant benefit on the one QP point and the indication of the significant disadvantage on the other QP point.

For the EE1-1.1 experiment, a significant visual benefit is reported for 7 out of 18 test cases. In one additional case (KristenAndSara QP34), the confidence interval is only very slightly touching the zero line. In all other cases, the zero line is included in the CI and therefore, comparable visual performance is suggested.

In the EE1-1.2 experiment, significant visual benefit is reported for 10 out of 26 test cases. The benefits seem to be more prominent on the UHD sequences. In two cases (Kimono1 QP37 and FourPeople QP42), a significant disadvantage is observed.

In the super-resolution category (EE1-2.x), the largest visual benefits are observed compared to the base line VTM. The gains appear to be higher by tendency for the NN-based method.

# References

- J. Jung, M. Wien, V. Baroncini, “Draft SC29/AG5 Guidelines for remote experts viewing sessions,” Doc. JVET-W0185, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 23rd meeting, online, July 2021.

- E. Alshina, M. Wien, A. Segall, J.Sauer, “AhG11/EE1 viewing preparation report,“ Doc. JVET-Y0045, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 25th meeting, online, Jan. 2022.

- M. Wien (RWTH), H. Zhang, X. Li (Tencent), “AhG10: Report of Teleconference on Viewing Session Preparation for Deblocking,” Doc. JVET-Y0083, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 25th meeting, online, Jan. 2022.

- H. Zhang, J. Jung, X. Li, S. Liu, “AHG10: Report of Deblocking filter setting for VTM,” Doc. JVET-Y0085, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 25th meeting, online, Jan. 2022.