| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0216-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0153: EE2-3.11: Non-adjacent spatial neighbors for affine merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yan Zhang | Tel: Email: | yzh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for EE2-3.11 test: experiment of the non-adjacent spatial neighbor for affine merge mode.

# Results

## Test 3.11: Non-adjacent spatial neighbor for affine merge mode

Non-adjacent inherited affine candidates are added to affine AMVP mode; non-adjacent affine inherited candidates, non-adjacent affine constructed candidates, and a new type of affine merge candidate where the translational affine parameters and non-translational affine parameters are derived from different CUs are introduced to affine merge mode. Full RA and partial LDB crosscheck result are matching with the EE2 3.11 test result.

Table 1. Crosscheck result of Test 3.11

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.35% | -0.26% | -0.35% | 101% | 100% |  |  |  |  |  |
| Class A2 | -0.57% | -0.25% | -0.40% | 100% | 100% |  |  |  |  |  |
| Class B | -0.33% | -0.08% | -0.13% | 101% | 101% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.22% | 0.02% | -0.10% | 101% | 101% | -0.49% | -0.39% | -0.41% | 101% | 102% |
| Class E |  |  |  |  |  | -0.23% | 0.45% | -0.45% | 101% | 99% |
| Overall | -0.35% | -0.12% | -0.22% | 101% | 101% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.25% | -0.18% | -0.02% | 101% | 102% | -0.74% | -0.22% | -0.97% | 101% | 102% |
| Class F | -0.23% | -0.27% | -0.28% | 100% | 100% | -0.55% | -0.53% | 0.21% | 100% | 101% |

## Anchor change test:

To demonstrate that the gain of EE2 3.11 test comes from introducing the non-adjacent affine candidates instead of increasing the sub-block merge list size and affine ARMC group size, in a supplement test, the ECM3.1 anchor is modified to have the same sub-block merge list size as well as affine ARMC group size, partial crosscheck result match the result provided by tester (run time is not reliable for this specific test).

Table 2. Crosscheck of Test 3.11 anchor change result

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.02% | -0.03% | 0.14% | 91% | 86% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

Implementation and simulation results of JVET-Y0153 have been confirmed.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).