| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0218-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0089 “Non-EE2: DMVR with BCW enabled” |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Z. Zhang | Tel: Email: | zhizhang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports the results of crosschecking JVET-Y0089 proposed by InterDigital. JVET-Y0089 proposes to enable BCW with DMVR. The BCW weights is inherited from merge candidate. The DMVR process applies mean removal (MR) to cost function when the BCW weight is not equal to BCW_DEFAULT.

The update of the code had been reviewed and confirmed consistent with the proposal description.

The verification simulation was run for RA configuration. The verification simulation results matched the results in JVET-Y0089 in term of BD-rate. The overall encoding / decoding times are {100%, 99%} compared to anchor.

# Verification

InterDigital had provided the software and results of their proposed methods in JVET-Y0089[1]. The code was updated according to the description in JVET-Y0089.

The simulation was conducted under the common test conditions as defined in [2]. The BD-rate results match the results reported in JVET-Y0089. The simulation results for random access configuration are as follows:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.10% | -0.16% | 99% | 97% |
| Class A2 | -0.05% | -0.01% | -0.03% | 98% | 98% |
| Class B | -0.02% | 0.05% | -0.01% | 101% | 100% |
| Class C | 0.01% | -0.02% | 0.02% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.01% | -0.04% | 100% | 99% |
| Class D | 0.01% | -0.02% | 0.17% | 102% | 101% |
| Class F (optional) | 0.05% | 0.03% | 0.01% | 102% | 101% |

# Reference

[1] P. Bordes, A. Robert, Y. Chen, F. Galpin, “Non-EE2: DMVR with BCW enabled”, JVET-Y0089, Jan. 2022.

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-V2017, Apr. 2021.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).