| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0219-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0125 (AHG12: Enhanced bi-directional motion compensation) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Han Huang, Y-J. Chang, C.-C. Chen, Marta Karczewicz, Vadim Seregin, Yan Zhang, Zhi, Zhang 5775 Morehouse Dr, San Diego, CA 92121 USA | Email: | hanhuang@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution provides the cross-check report for JVET-Y0125 (AHG12: Enhanced bi-directional motion compensation).

# Introduction

In JVET-Y0125, the uni-directional MC predictor is regarded as out-of-boundary (OOB) when the location of the reference interpolation sample is located outside the reference picture beyond half sample. For each predictor in a bi-directional MC block, when one uni-directional MC predictor is OOB and the other one is non-OOB, this bi-directional predictor is changed into the non-OOB uni-directional MC predictor.

In this report, two results are provided. In the first test, an independent implementation of JVET-Y0125 is integrated on top of ECM-3.1, the result shown in Table 1. In the second test, the result is generated by using the provided JVET-Y0125 software and is shown in Table 2. The current available result reported in Test 2 matches those reported in JVET-Y0125. It should be noted that runtime provided in this report is not accurate.

The main difference between the independent implementation and JVET-Y0125 software is as follows:

The OOB check is performed before BDOF process in the independent implementation. The predictor in OOB direction is replaced by the predictor in the non-OOB direction, then regular weighted averaging in bi-prediction or BDOF process is performed. In the JVET-Y0125 implementation, masking is applied after BDOF process to decide whether the final predictor is set equal to the non-OOB predictor.

Table 1: Test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.34% | -0.28% | -0.19% | 91% | 95% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.91% | -0.62% | -0.37% | 94% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.14% | 0.07% | -0.53% | 82% | 83% |

Table 2: Test 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.31% | -0.24% | -0.20% | 97% | 94% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.81% | -0.41% | -0.38% | 99% | 95% |
| Class F | -0.02% | -0.06% | -0.04% | 87% | 89% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.15% | -0.19% | -0.59% | 82% | 89% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).