| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0221-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0092 (Non-EE2: On chroma intra prediction mode) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ramin G. Youvalari | Email: | ramin.ghaznavi-youvalari@nokia.com |
| Source: | Nokia Technologies |  |  |

_____________________________

# Abstract

This contribution is the cross-check of contribution JVET-Y0092.

Partial results are obtained, and all match the coding results reported in JVET-Y0092.

# Introduction

Contribution JVET-Y0092 proposes methods to improve the coding efficiency of chroma intra prediction, including a decoder-side derived chroma intra prediction mode (DIMD chroma) and a fusion of a non-LM mode and a LM mode.

Two tests have been conducted:

Test 1: The fusion of MMLM_LT mode and DIMD or non-LM modes are tested.

Test 2: Is the same as test 1 but for non-I Slices the fusion of MMLM_LT mode and DIMD chroma mode are applied.

# Experimental results

The proposed methods are implemented on top of ECM-3.1 [1]. The simulation results are tested under ECM test condition [2] with all intra and random access configurations as shown in below tables.

The decoding runtimes in this crosscheck document are not reliable.

Test 1:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11 % | -1.74 % | -1.27 % | 106 % | 107 % |
| Class A2 | -0.07 % | -1.43 % | -1.32 % | 110 % | 107 % |
| Class B | -0.10 % | -1.29 % | -1.40 % | 104 % | 106 % |
| Class C | -0.11 % | -1.35 % | -1.28 % | 104 % | 107 % |
| Class E | -0.14 % | -1.82 % | -1.70 % | 103 % | 107 % |
| Overall | -0.11 % | -1.49 % | -1.39 % | 105 % | 107 % |
| Class D | -0.10 % | -0.87 % | -1.05 % | 105 % | 109 % |
| Class F | -0.11 % | -0.89 % | -0.81 % | 103 % | 106 % |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02 % | -0.61 % | -0.45 % | 101 % | 95 % |
| Class A2 | -0.04 % | -0.61 % | -0.47 % | 101 % | 97 % |
| Class B | 0.02 % | -0.79 % | -0.88 % | 102 % | 96 % |
| Class C | 0.03 % | -0.76 % | -0.51 % | 100 % | 99 % |
| Class E |  |  |  |  |  |
| Overall | 0.00 % | -0.71 % | -0.62 % | 101 % | 97 % |
| Class D | 0.07 % | -0.61 % | -0.98 % | 102 % | 98 % |
| Class F | -0.04 % | -0.35 % | -0.43 % | 103 % | 98 % |

Test 2:

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05 % | -0.43 % | -0.51 % | 100 % | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.02 % | -0.65 % | -0.72 % | 101 % | #NUM! |
| Class C | -0.03 % | -0.56 % | -0.17 % | 99 % | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02 % | -0.27 % | -0.79 % | 101 % | #NUM! |
| Class F | -0.06 % | -0.24 % | -0.26 % | 102 % | #NUM! |

# References

- ECM-3.1, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-X2017, Oct. 2021.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).