| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0228-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0068: EE1-2.1-related: RPR encoder with multiple scale factors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This document verifies reported PSNR and bitrate numbers in JVET-Y0068. It can also be noted that JVET-Y0068 is on top of VTM-13.0 and not on top of the NNVC anchor. The reported results in the cross-check are compared to the NNVC anchor and show very similar BDR as shown in JVET-Y0068.

# Introduction

In some case it can be beneficial to encode a sequence in lower resolution and then upscale it before display instead of coding it in full resolution. VVC provide RPR which can enable such functionality. In EE1-2.1 RPR usage is determined by multi-pass coding the whole sequence and RPR 2x (coding in half resolution) is selected if it has compression benefits compared to coding in original resolution.

In JVET-Y0068 [1] they try down-scaling the first picture in the GOP and then upscale it and determine the PSNR compared to no re-scaling. Then heuristics based on the PSNR and the QP is used to select downscaling all pictures in the GOP by RPR 2x, RPR 1.5x or no scaling. When RPR 2x is selected the QP is reduced by 6, when RPR 1.5x is selected the QP is reduced by 4 and otherwise no QP change.

# Results

We confirm the reported bitrate and PSNR. We report comparison with the NNVC anchor (instead of compared to VTM-13.0 as in JVET-Y0068) and thus have some slight differences in BDR compared to the BDR reported by the proponents. Encoding and decoding times are not accurate.

Table 1: Comparison vs NNVC anchor (the test JVET-Y0068 is based on VTM-13.0)

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -3,47% | 2,23% | 3,72% | -6,89% | 0,93% | 1,00% | 85% | 84% | 4% |
| Class A2 | -1,55% | 7,82% | 6,70% | -4,59% | 8,41% | 6,81% | 95% | 94% | 2% |
| Class B | 0,08% | 1,83% | 1,26% | -1,07% | 1,79% | 1,19% | 104% | 105% | 0% |
| Class C | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 105% | 107% | 0% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0,98% | 2,62% | 2,51% | -2,65% | 2,46% | 1,96% | 98% | 99% | 1% |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -1,55% | 2,38% | 2,60% | -4,30% | 3,39% | 3,99% | 124% | 92% | 7% |
| Class A2 | -2,81% | 12,66% | 9,69% | -6,97% | 9,96% | 8,09% | 102% | 88% | 4% |
| Class B | 0,52% | 2,13% | 2,27% | -1,37% | 3,13% | 3,98% | 110% | 101% | 0% |
| Class C | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 0,00% | 108% | 99% | 0% |
| Class E | 0,09% | 0,25% | 0,27% | -0,38% | 0,53% | 0,58% | 114% | 107% | 0% |
| Overall | -0,57% | 3,14% | 2,73% | -2,32% | 3,18% | 3,22% | 111% | #DIV/0! | 2% |

# References

[1] J. Nam, S. Yoo, J. Lim, “EE1-2.1-related: RPR encoder with multiple scale factors”, JVET-Y0068, on-line meeting, January 2022

[2] E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang , “Exploration Experiments on Neural Network-based Video Coding (EE1)”, JVET-X2023, on-line meeting, October 2022

[3] S. Liu, A. Segall, E. Alshina, R.-L. Liao , “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, on-line meeting, October 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).