| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0235 |
| --- | --- |

| Title: | Crosscheck of JVET-Y0161 (EE2-3.12-related: Extensions of history-parameter-based affine model inheritance) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70 3508 5829 yo-kidani@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution is a cross-check report of JVET-Y0161. The proposal introduces the extensions of the history-parameter-based affine model inheritance, which is being investigated as EE2-3.12 in this meeting.

Summary

In JVET-Y0161 [1], a total of the following four extensions is proposed to further improve the coding performance.

- Aspect 1:

The size of sub-block-based merge candidate list is increased to 15, and all the affine merge candidates are involved in the adaptive reordering merge candidates (ARMC) process.

- Aspect 2:

The number of neighboring blocks, which are used to generate affine merge candidates jointly with affine models stored in tables, are increased from five to seven. The two additional neighboring blocks are located at the right and bottom of the B2, which is the top-left neighboring block for current block.

- Aspect 3:

A second history-parameter table with base MV information is appended. The history-parameter tables stored in the CTU above/above-right to the current CTU can be used by blocks in the current CTU.

- Aspect 4:

Pair-wised affine merge candidates are generated by two affine merge candidates which are history-derived or not history-derived. A pair-wised affine merge candidate is generated by averaging the CPMVs of existing affine merge candidates in the list.

The source code provided by the proponents was carefully analysed and it is confirmed that the implementation matches the description in JVET-Y0161.

Simulations for Test 1 and Test 2 of JVET-Y0161 were conducted in the RA and LB configurations of the CTC for enhanced compression tool testing [2]. Test 1 consists only aspect 1, whereas Test 2 includes all the aspects proposed by JVET-Y0161. The cross-check results are provided in the accompanying excel documents. The cross checker could exactly reproduce the results regarding BD-rate but not EncT/DecT provided by the proponent. According to the proponent, the difference of EncT/DecT might be derived from unstable clusters the proponent utilized. It is noted that the provided software was compiled on 64-bit Ubuntu environment. The experiments were performed on the cluster PCs.

Conclusion

We had verified results reported in JVET-Y0161. The results of the cross-check matched with proponent’s report.

References

- K. Zhang, L. Zhang, Z. Deng, N. Zhang, and Y. Wang, “EE2-3.12-related: Extensions of history-parameter-based affine model inheritance,” JVET-Y0161, by teleconference, 12–21 January 2022.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-X2017, by teleconference, 6-15 October. 2021.

Patent rights declaration(s)

KDDI Corporation does not have any current or pending patent rights relating to the technology described in this contribution.