| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12–21 January 2022 | Document: JVET-Y0247-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Y0128: Non-EE2: fixing issues for RPR enabling and non-CTC configuration in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This document verifies reported PSNR2 and bitrate numbers for config 1 (each frame scaled) of simulation 4.3 and 4.5 of JVET-Y0128. For config 2 in simulation 4.4 we can verify that simulations that are completed match the proponents.

# Introduction

JVET-Y0128 suggest some additional fixes for RPR in relation to:

- LIC gated by define RPRFIX_INTER_LIC,

- Adaptive DMVR gated by JVET_X0049_ADAPT_DMVR_FIX,

- BM AMVP merge mode gated by JVET_X0083_FIX_BM_AMVP_MERGE_MODE.

- TM AMVP gated by define RPRFIX_INTER_TMAMVP,

Redundant signalling:

#define RPRFIX_INTER_LIC                  ((INTER_LIC) && 1) // Fixes for RPR+LIC

The purpose of the fix related to LIC is mainly to avoid signalling the CU LIC flag when rescaling is used.

#define JVET_X0049_ADAPT_DMVR_FIX

#define JVET_X0083_FIX_BM_AMVP_MERGE_MODE

The purpose of the fixes related to adaptive DMVR and BM AMVP merge mode is to remove redundant signalling of those. The fixes also relate to merge list construction.

Avoid TM when reference picture is scaled:

#define RPRFIX_INTER_TMAMVP       ((TM_AMVP) && 1) // Fixes for RPR+TMAMVP

The purpose of the fix for TM AMVP, is to introduce a condition check such that TM can not be enabled when reference picture is scaled, instead conventional AMVP will be used.

Avoid encoder decoder mismatch:

There is also some change that is below RPR_ENABLE that replace static const declaration with const declaration related to affine mmvd in getAfMmvdMvf.

# Results

We confirm the reported bitrate and PSNR2 when RPR is used according to config 1 (each frame re-scaled). For config 2 (resolution change every 16:th frame) we can confirm simulations that have been completed.

Re-generation of simulation 4.3 in JVET-Y0128 using config 1 (anchor to ECM-3.1):

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | 0,32% | 25,09% | 20,87% | 25% | 31% |
| Class A2 | 18,88% | 81,32% | 48,85% | 23% | 29% |

Re-generation of simulation 4.5 in JVET-Y0128 and using config 1 (anchor ECM-3.1):

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -1,66% | 3,42% | 5,11% | 69% | 67% |
| Class A2 | -0,37% | 15,95% | 10,95% | 78% | 76% |

# References

[1] Z. Zhang, H. Huang, C-C. Chen, V. Seregin, M. Karczewicz, P. Bordes, “Non-EE2: fixing issues for RPR enabling and non-CTC configuration in ECM”, JVET-Y0128, on-line meeting, January 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).