| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0006-v1 |
| --- | --- |

| Title: | JVET AHG report: ECM software development (AHG6) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Fabrice Le Leannec Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com fabricel@xiaomi.com zhangkai.video@bytedance.com |
| Source: | AHG6 on ECM software development |  |  |

_____________________________

# Abstract

This report summarizes the activities of the AHG6 on ECM software development that has taken place between the 25th and 26th JVET meetings.

# Introduction

The mandates for the AHG are as follows.

- Coordinate development of the ECM software and associated configuration files.

- Produce documentation of software usage for distribution with the software.

- Prepare and deliver ECM-4.0 software version and the reference configuration encodings according to JVET-Y2017 common test conditions.

- Investigate encoder speedup and other encoder software optimization.

- Coordinate with ECM algorithm description editors to identify any mismatches between software and text, make further updates and cleanups to the software as appropriate.

# Software development

ECM software repository is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM.

ECM software is based on VTM-10.0 with enabled MCTF including the update from JVET-V0056, and GOP32, which is very close to VTM-11.0.

The following adopted aspects were integrated into ECM-4.0:

JVET-Y0116 (version 2.1a): Extension of MRL list to 5 lines 1,3,5,7,12

JVET-Y0065 (test 3.1c, not in LP CTC): GPM intra-inter mode

JVET-Y0134 (test 3.6a): MV candidate reordering for TMVP and NAMVP types, and reference picture selection for TMVP

JVET-Y0067 (test 3.9d): TM based reordering for MMVD and affine MMVD and MVD sign prediction

JVET-Y0058 (test 3.13): Modifications of IBC merge/AMVP list construction

JVET-Y0142 (test 4.4a): Adaptive intra MTS with fixed threshold

JVET-Y0106: CCSAO edge offset

JVET-Y0141 (test 3): Sign prediction improvement

JVET-Y0159 (method 1): Inter MTS uses fixed 4 candidates

JVET-Y0089: DMVR with BCW

JVET-Y0128: Fixing issues for RPR enabling and non-CTC configuration in ECM

JVET-Y0129: MVD signalling

JVET-Y0156: Fix for histogram of gradients derivation in DIMD mode

JVET-Y0060: Add AffineAMVP cfg option

JVET-Y0152 (SW/CTC): Fast skip method of TT partition search

JVET-Y0155 (SW/CTC): Fixes and clean up for temporal prefilter

JVET-Y0240 (SW): Block importance mapping

ECM software improvements:

MS-SSIM

log output alignment with VTM

ECM-4.0 was tagged on February 15, 2022.

## CTC Performance

In this section, ECM-4.0 test results following ECM CTC configuration descried in JVET-X2017 are summarized.

The below tables show ECM-4.0 performance over ECM-3.1 anchor.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.94% | -0.86% | 104% | 110% |
| Class A2 | -0.51% | -0.98% | -3.54% | 103% | 110% |
| Class B | -0.49% | -1.35% | 0.08% | 107% | 109% |
| Class C | -0.32% | 0.08% | 0.12% | 111% | 111% |
| Class E | -0.55% | -0.85% | -0.69% | 106% | 110% |
| Overall | -0.46% | -0.82% | -0.80% | 107% | 110% |
| Class D | -0.29% | -0.18% | -0.01% | 114% | 115% |
| Class F | -0.40% | -0.70% | -0.35% | 99% | 100% |
| Class TGM | -1.01% | -1.66% | -1.57% | 96% | 95% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.39% | -2.32% | -2.31% | 94% | 104% |
| Class A2 | -1.53% | -2.88% | -4.81% | 96% | 108% |
| Class B | -1.51% | -2.91% | -1.73% | 100% | 113% |
| Class C | -1.12% | -1.25% | -1.47% | 102% | 124% |
| Class E |  |  |  |  |  |
| Overall | -1.39% | -2.34% | -2.39% | 99% | 113% |
| Class D | -0.92% | -0.86% | -1.23% | 106% | 127% |
| Class F | -0.86% | -0.61% | -0.69% | 97% | 102% |
| Class TGM | -1.26% | -1.33% | -1.29% | 98% | 98% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.66% | -3.57% | -2.32% | 107% | 122% |
| Class C | -1.80% | -2.60% | -2.03% | 109% | 128% |
| Class E | -0.94% | -2.03% | -2.20% | 110% | 100% |
| Overall | -1.53% | -2.86% | -2.19% | 108% | 118% |
| Class D | -1.59% | -3.25% | -2.91% | 111% | 135% |
| Class F | -1.30% | -2.99% | -1.30% | 105% | 99% |
| Class TGM | -1.47% | -1.59% | -1.44% | 102% | 99% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over ECM-3.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.24% | -3.20% | -1.95% | 111% | 127% |
| Class C | -1.20% | -1.27% | -1.02% | 115% | 141% |
| Class E | -0.65% | -2.35% | -0.63% | 116% | 112% |
| Overall | -1.08% | -2.35% | -1.31% | 114% | 127% |
| Class D | -1.10% | -2.23% | -0.98% | 116% | 146% |
| Class F | -0.94% | -0.81% | -0.89% | 111% | 112% |
| Class TGM | -1.39% | -1.47% | -1.30% | 105% | 105% |

Next tables show ECM-4.0 performance over VTM-11.0ecm anchor, the software is located at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/VTM11_ANC.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0ecm |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.60% | -12.43% | -17.68% | 357% | 262% |
| Class A2 | -6.24% | -13.37% | -14.77% | 342% | 254% |
| Class B | -5.70% | -14.30% | -13.65% | 382% | 248% |
| Class C | -6.89% | -9.30% | -9.80% | 373% | 242% |
| Class E | -7.67% | -12.17% | -13.14% | 363% | 264% |
| Overall | -6.53% | -12.37% | -13.57% | 366% | 253% |
| Class D | -5.73% | -8.03% | -7.65% | 371% | 241% |
| Class F | -10.93% | -15.45% | -14.70% | 253% | 255% |
| Class TGM | -16.70% | -18.91% | -18.31% | 245% | 276% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -16.20% | -17.76% | -23.60% | 389% | 471% |
| Class A2 | -16.97% | -22.37% | -24.24% | 375% | 546% |
| Class B | -14.46% | -21.68% | -19.89% | 403% | 505% |
| Class C | -16.40% | -17.23% | -17.19% | 426% | 504% |
| Class E |  |  |  |  |  |
| Overall | -15.83% | -19.85% | -20.78% | 400% | 506% |
| Class D | -16.99% | -17.54% | -17.59% | 411% | 523% |
| Class F | -14.23% | -17.29% | -17.00% | 349% | 363% |
| Class TGM | -16.11% | -19.04% | -19.39% | 387% | 299% |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -12.29% | -26.50% | -24.33% | 351% | 397% |
| Class C | -13.71% | -20.74% | -19.80% | 365% | 378% |
| Class E | -11.18% | -18.16% | -19.41% | 344% | 299% |
| Overall | -12.48% | -22.49% | -21.59% | 353% | 364% |
| Class D | -15.49% | -22.79% | -21.74% | 348% | 387% |
| Class F | -12.98% | -20.76% | -18.58% | 325% | 296% |
| Class TGM | -15.26% | -22.73% | -22.40% | 370% | 292% |
|  | Low delay P Main 10 |  |  |  |  |
|  | Over VTM-11.0ecm |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -13.10% | -27.56% | -25.62% | 324% | 377% |
| Class C | -14.70% | -21.22% | -20.76% | 308% | 399% |
| Class E | -12.17% | -18.72% | -20.33% | 321% | 298% |
| Overall | -13.40% | -23.24% | -22.68% | 318% | 362% |
| Class D | -16.61% | -23.06% | -22.80% | 290% | 398% |
| Class F | -13.44% | -20.52% | -19.30% | 311% | 296% |
| Class TGM | -14.92% | -22.44% | -22.03% | 373% | 291% |

The Excel files with the complete ECM results are attached to this report.

# Recommendations

The AHG recommends to:

- Continue to develop ECM software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/issues.

- Encourage people to submit merge requests fixing identified bugs.