| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z007-v1 |
| --- | --- |

| Title: | JVET AHG report: Low latency and constrained complexity (AHG7) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | Tangi Poirier Alberto Duenas Shan Liu Limin Wang Jizheng Xu | Tel: Email: | tangi.poirier@interdigital.com alberto.duenas@outlook.com shanl@tencent.com limin.2.wang@nokia.com xujizheng@bytedance.com |
| Source: | AHG |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG7: Low latency and constrained complexity, between the the 25th meeting (teleconference, 12-21 January 2022) and the 26th meeting (teleconference, 20-29 April 2022).

# Mandates

At the 25th meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on Low latency and controlled complexity was established with the following mandates:

- Identify additional application scenarios and their requirements for low latency and constrained complexity, taking into account aspects of real-time encoding and decoding.

- Discuss requirements of already identified scenario such as cloud gaming, game casting, video conferencing, video surveillance and remote control of systems.

- Evaluate and refine new CTC for low latency and constrained complexity application scenarios, and investigate a set of tools that provide a reasonable tradeoff regarding complexity vs. compression, as well as latency constraints.

- Conduct tests with ECM and VTM to determine the impact of discussed configurations on coding efficiency and run time, and conduct visual tests in coordination with AHG4.

- Review current test sequences and if necessary collect new test materials that are suitable for the intended application domains, and establish an applicable dataset in coordination with AHG4.

- Coordinate with AHG3 and AHG12 to discuss and recommend configuration(s) applicable to ECM and VTM, taking into account complementarity with existing CTCs.

# Related contributions

## Low delay configuration

### JVET-Z0110: On low delay configuration

In this contribution, the coding efficiency and coding speed by low delay B (LDB) and low delay P configurations are compared under different conditions.

### JVET-Z0114: AHG7: Low delay configuration for cloud gamin

This contribution proposes a new low delay configuration (named LDB-1Ref-asymmetric) where the number of reference frames is restricted to 1 frame per list, and reference frame list is modified such that reference list 0 refers to the previous picture in the GOP and reference list 1 refers to the picture at the start of the GOP.

### JVET-Z0116: AHG7: Refined low latency and controlled complexity configuration for cloud gaming

This contribution proposes a refined low latency and controlled complexity (LLCC) configuration for cloud gaming (JVET-Y0043) by disabling MTS for low latency and controlled complexity video applications such as cloud gaming.

## GDR implementation

### JVET-Z0118: AHG7: GDR Implementation for ECM 4.0

This contribution presents a GDR implementation for ECM 4.0 [1]. With this GDR implementation, there is no need to impose constraints on coding tools, which helps to reduce the code size for ECM significantly as compared to VTM-like GDR [3]. In addition, this new GDR implementation provides great flexibility in future tool development for ECM. Specifically, adding of additional new coding tools in ECM code will not affect part of GDR code.

Simulations were conducted under low-delay B configuration specified by LLCC AhG. The results demonstrate that the overall loss of the new GDR over anchor is only x.xx%.

### JVET-Z0141: AHG7: GDR in ECM-4.0

This contribution presents results of GDR implementation for ECM4.0 [1]. GDR has been implemented in VTM [2] and then ported in ECM-3.0 [3] with new coding tools turned off. This GDR implementation has been tested with all new tools of ECM enabled.

## Sequences

### JVET-Z0138: AHG7: Update on gaming sequences

This contribution proposes an update on new gaming sequences proposed in JVET-Y-0041 at the last meeting. It also proposes to create a new class of content for gaming sequences.

# Recommendation

The AHG recommends reviewing input contributions and:

- to coordinate with AhG4 so that new sequences corresponding to low delay and to low latency and controlled complexity scenarios are added to the JVET CTCs.

- to define configurations for low latency and controlled complexity, taking in account some of the suggestions made on the reflector.

- and to integrate a GDR implementation in the latest version of ECM.