| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0008-v1 |
| --- | --- |

| Title: | JVET AHG Report: High bit depth, high bit rate, and high frame rate coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | Adrian Browne Tomohiro Ikai Dmytro Rusanovskyy Xiaoyu Xiu Yue Yu | Email: | adrian.browne@sony.com ikai.tomohiro@sharp.co.jp dmytror@qti.qualcomm.com xiaoyuxiu@kwai.com yue.yu@oppo.com |
| Source: | AHG8 |  |  |

_____________________________

# Abstract

This document summarizes the activity of AHG8: High bit depth, high bit rate, and high frame rate coding between the 25th meeting by teleconference (12-21 January 2022) and the 26th meeting by teleconference (20-29 April 2022).

# Mandates

The AHG was established with the following mandates:

- Study the benefits and characteristics of VVC coding tools for high bit depth, high bit rate, and high frame rate coding.

- Study lossless coding characteristics of VVC.

- Identify technologies for future extension of VVC to support such application usage.

- Study the JVET-U2018 testing conditions for high bit depth, high bit rate, and high frame rate coding, and suggest improvements as applicable.

- Contribute to the development of software and conformance testing for operation range extensions in coordination with AHG3 and AHG5.

- Identify suitable test material for testing of high bit depth, high bit rate, and high frame rate coding in coordination with AHG4.

- Study VVC entropy decoding throughput and latency in the cases of high bit depth, high bit rate, and high frame rate coding.

# Activities

Previously, the AHG has used the main JVET reflector, jvet@lists.rwth-aachen.de, with [AHG8] in message headers, however no correspondence marked as AHG8 was sent between the 25th and 26th meetings.

The major area of work related to the AHG in this meeting cycle was the generation and cross-checking of conformance bitstreams for VVC version 2 in coordination with AHG5. A revision of JVET-Y2026 “Conformance testing for VVC operation range extensions (Draft 3)” was uploaded on 18th March 2022.

No contributions related to the study of high bit depth, high bit rate or high frame rate coding have been registered for the 26th meeting.

# Benchmarks

The results in this section compare the performance of VTM16.0 against an anchor of VTM15.0. Results comparing VTM16.0 against VTM 14.0 are also included in this contribution as the previous AHG report did not provide benchmarks for VTM 15.0.

## Standard QP range

The standard QP range covers QPs 22 to 37 for 12 bit coding.

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM15.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.77% | -1.48% | -1.05% | -1.91% | -2.09% | -0.76% | -1.35% | -1.47% | 93% | 100% |
| Class H2 |  |  |  |  |  | -0.71% | -1.54% | -1.32% | 92% | 100% |
| Overall | -0.77% | -1.48% | -1.05% | -1.91% | -2.09% | -0.73% | -1.44% | -1.40% | 92% | 100% |
|  | All Intra |  |  |  |  |  |  |  |  |  |
|  | Over VTM15.0 |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  | PSNR |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | Y | U | V | EncT | DecT |
| Class H1 | -0.65% | -1.24% | -1.19% | -2.40% | -3.24% | -0.94% | -1.55% | -1.64% | 92% | 100% |
| Class H2 |  |  |  |  |  | -0.80% | -1.02% | -0.61% | 92% | 100% |
| Overall | -0.65% | -1.24% | -1.19% | -2.40% | -3.24% | -0.87% | -1.29% | -1.13% | 92% | 100% |

The majority of the BD-rate gains reported in these results can be attributed to the adoption of the updated deblocking configuration from ECM in JVET-Y0085 “AHG10: Report of Deblocking filter setting for VTM”.

The encoder time speed up can be attributed to the adoption of the software changes in JVET-Y0152 “AHG10: Fast skip of TT split partitioning on top of ECM reference software” and JVET-Y0126 “AHG10: VTM encoder configurations for tests targeting improved coding performance”.

## Low QP Range

The low QP range covers QPs -13 to 12 for 12 bit coding, and -33 to -8 for 16 bit coding.

ss

| HDR PQ |  |  | AI |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM15.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | 0.01% | 0.00% | 0.00% | 0.01% | 0.01% | 96% | 100% |
| PQ422 | 0.01% | 0.00% | 0.01% | 0.01% | 0.00% | 0.00% | 97% | 100% |
| Overall | 0.01% | 0.00% | 0.01% | 0.00% | 0.01% | 0.00% | 97% | 100% |
|  |  |  | LDB |  |  |  |  |  |
|  |  |  | Over VTM15.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.01% | 0.00% | 0.01% | 0.00% | -0.03% | 0.00% | 97% | 99% |
| PQ422 | 0.01% | -0.03% | 0.00% | 0.01% | -0.04% | 0.00% | 98% | 99% |
| Overall | 0.01% | -0.02% | 0.01% | 0.01% | -0.03% | 0.00% | 98% | 99% |
|  |  |  | RA |  |  |  |  |  |
|  |  |  | Over VTM15.0 |  |  |  |  |  |
|  | wPsnrY | wPsnrU | wPsnrV | psnrY | psnrU | psnrV | EncT | DecT |
| PQ444 | 0.00% | -0.02% | -0.01% | -0.01% | -0.02% | -0.02% | 97% | 99% |
| PQ422 | 0.00% | 0.00% | -0.02% | 0.00% | 0.00% | -0.02% | 97% | 99% |
| Overall | 0.00% | -0.01% | -0.02% | -0.01% | -0.01% | -0.02% | 97% | 99% |
| Overall PQ | 0.00% | -0.01% | 0.00% | 0.00% | -0.01% | 0.00% | 97% | 99% |

| HDR HLG |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM15.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | 0.00% | 0.00% | 0.00% | 99% | 99% |
| HLG422 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM15.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | -0.01% | 0.03% | 98% | 98% |
| HLG422 | -0.01% | 0.00% | 0.02% | 99% | 100% |
| Overall | -0.01% | 0.00% | 0.03% | 98% | 99% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM15.0 |  |  |
|  | psnrY | psnrU | psnrV | EncT | DecT |
| HLG444 | -0.02% | 0.00% | 0.00% | 97% | 99% |
| HLG422 | -0.01% | -0.01% | 0.01% | 99% | 100% |
| Overall | -0.02% | -0.01% | 0.00% | 98% | 100% |
| Overall HLG | -0.01% | 0.00% | 0.01% | 99% | 100% |

| SVT RGB |  |  | AI |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM15.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| SVT12 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  |  |  | LDB |  |  |
|  |  |  | Over VTM15.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| SVT12 | 0.00% | -0.12% | -0.12% | 101% | 101% |
| Overall | 0.00% | -0.06% | -0.06% | 100% | 101% |
|  |  |  | RA |  |  |
|  |  |  | Over VTM15.0 |  |  |
|  | psnrG | psnrB | psnrR | EncT | DecT |
| SVT16 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| SVT12 | 0.00% | -0.06% | -0.06% | 99% | 100% |
| Overall | 0.00% | -0.03% | -0.03% | 99% | 100% |
| Overall RGB | 0.00% | -0.03% | -0.03% | 100% | 100% |

- Recommendations

The AHG recommends the following:

- To continue high bit depth, high bit rate, and high frame rate studies to support conformance activities for the next meeting cycle.