| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0012-v1 |
| --- | --- |

| Title: | JVET AHG report: Enhanced compression beyond VVC capability (AHG12) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | AHG Report |  |  |
| Author(s) or Contact(s): | M. Karczewicz Y. Ye L. Zhang B. Bross X. Li K. Naser H. Yang | Author(s) or Contact(s): | martak@qti.qualcomm.com yan.ye@alibaba-inc.com lizhang.idm@bytedance.com benjamin.bross@hhi.fraunhofer.de xlxiangli@tencent.com karam.naser@interdigital.com haitao.yang@huawei.com |
| Source: | AHG 12 |  |  |

_____________________________

# Abstract

This document reports the work of the JVET ad hoc group on enhanced compression beyond VVC capability (AHG12) between the 25th meeting by teleconference, 12-21 January 2022, and the 26th meeting by teleconference, 20-29 April 2022.

# Introduction

The mandates given to the AHG are:

- Solicit and study non-neural-network video coding tools with enhanced compression capabilities beyond VVC.

- Discuss and propose refinements to the ECM4 algorithm description JVET-Y2025.

- Study the performance and complexity tradeoff of these video coding tools.

- Coordinate with AHG6 on ECM software development.

- Refine test conditions in JVET-Y2017, generate anchors, identify new test sequences to be added in coordination with AHG4 and AHG7.

- Analyse the results of exploration experiments described in JVET-Y2024 in coordination with the EE coordinators.

- Coordinate with AHG11 to study the interaction with neural network-based coding tools.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A few emails were exchanged regarding EE2.

# Activities

The Common Test Conditions were updated (JVET-Y2017). The test conditions for HDR were added and the checksums for some of the test sequences were corrected. The primary activity of the AHG was the “Exploration experiment on enhanced compression beyond VVC capability” (JVET-Y2024). The combined improvements of the ECM-4.0 over VTM-11.0ecm anchor for AI, RA and LB configurations are:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -6.60% | -12.43% | -17.68% | 357% | 262% |
| Class A2 | -6.24% | -13.37% | -14.77% | 342% | 254% |
| Class B | -5.70% | -14.30% | -13.65% | 382% | 248% |
| Class C | -6.89% | -9.30% | -9.80% | 373% | 242% |
| Class E | -7.67% | -12.17% | -13.14% | 363% | 264% |
| Overall | -6.53% | -12.37% | -13.57% | 366% | 253% |
| Class D | -5.73% | -8.03% | -7.65% | 371% | 241% |
| Class F | -10.93% | -15.45% | -14.70% | 253% | 255% |
| Class TGM | -16.70% | -18.91% | -18.31% | 245% | 276% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -16.20% | -17.76% | -23.60% | 389% | 471% |
| Class A2 | -16.97% | -22.37% | -24.24% | 375% | 546% |
| Class B | -14.46% | -21.68% | -19.89% | 403% | 505% |
| Class C | -16.40% | -17.23% | -17.19% | 426% | 504% |
| Class E |  |  |  |  |  |
| Overall | -15.83% | -19.85% | -20.78% | 400% | 506% |
| Class D | -16.99% | -17.54% | -17.59% | 411% | 523% |
| Class F | -14.23% | -17.29% | -17.00% | 349% | 363% |
| Class TGM | -16.11% | -19.04% | -19.39% | 387% | 299% |

|  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -12.29% | -26.50% | -24.33% | 351% | 397% |
| Class C | -13.71% | -20.74% | -19.80% | 365% | 378% |
| Class E | -11.18% | -18.16% | -19.41% | 344% | 299% |
| Overall | -12.48% | -22.49% | -21.59% | 353% | 364% |
| Class D | -15.49% | -22.79% | -21.74% | 348% | 387% |
| Class F | -12.98% | -20.76% | -18.58% | 325% | 296% |
| Class TGM | -15.26% | -22.73% | -22.40% | 370% | 292% |

The rate reduction over VTM for RA configuration for {Y, U, V} increased from {-14.77%,	-18.61%, -19.42%} to {-15.83%, -19.85%, -20.78%}.

# Contributions

In addition to 21 EE2 contributions, 38 (comparing to 41 last meeting) related contributions were received which can be subdivided as follows:

## In Loop Filters (3)

JVET-Z0105, "AHG12: Virtual boundary processing for the new In-Loop filter tools in ECM", A. M. Kotra, N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-Z0146, "AHG12: Using samples before deblocking filter for adaptive loop filter",	N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

JVET-Z0149, "Non-EE2: Adaptive Filter Shape Switch for ALF", W. Yin, K. Zhang, L. Zhang (Bytedance)

## Intra (5)

JVET-Z0064, "AHG12: Convolutional cross-component model (CCCM) for intra prediction", P. Astola, J. Lainema, R. G. Youvalari, A. Aminlou, K. Panusopone (Nokia)

JVET-Z0100, "Non-EE2: Weighted Chroma Prediction (WCP)",  J.-Y. Huo, H.-Q. Du, Z.-Y. Zhang, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), M. Li, Y. Liu (OPPO)

JVET-Z0124, "Non-EE2: Spatial GPM",	 F. Wang, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-Z0140, "AHG12: Enhanced CCLM",	C.-W. Kuo, X. Xiu, N. Yan, H.-J. Jhu, W. Chen, H. Gao, X. Wang (Kwai)

JVET-Z0143, "Non-EE2: Chroma intra modes derived from collocated luma blocks and neighboring chroma blocks",	 Y.-J. Chang, K. Cao, B. Ray, V. Seregin, M. Karczewicz (Qualcomm) Rufitskiy, E. Dinan (Ofinno)

## Inter (20)

JVET-Z0059, "Non-EE2: Adaptive width for GPM blending area", Y. Kidani, H. Kato, K. Unno, K. Kawamura (KDDI)

JVET-Z0063, "AHG12: Fix for parsing of MHP information in ECM", R. Yu (Ericsson)

JVET-Z0066, "EE2-related: Template matching using extended MVP candidate list", K. Kim, D. Kim, J.-H. Son, J.-S. Kwak (WILUS)

JVET-Z0068, "EE2-2.6 related: Longer chroma interpolation filters", K. Andersson, R. Yu (Ericsson)

JVET-Z0069, "AHG12: Longer chroma filters for RPR in ECM", K. Andersson, R. Yu (Ericsson)

JVET-Z0078, "Non-EE2: Support MMVD and Affine MMVD without template matching process", H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

JVET-Z0083, "Non-EE2: Fix on MHP parsing condition", N. Park, J. Nam, J. Lim, S. Kim (LGE)

JVET-Z0085, "Non-EE2: AmvpMerge without decoder side mv refinement process", H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

JVET-Z0102, "Non-EE2: Correction of the ARMC re-ordering step regarding the zero candidates", G. Laroche, P. Onno, R. Bellessort (Canon)

JVET-Z0103, "Non-EE2: On ARMC improvements", G. Laroche, P. Onno (Canon)

JVET-Z0123, "EE2-related: On chroma interpolation filters for motion compensation", J. Gan, Y. Yu, H. Yu (OPPO)

JVET-Z0125, "EE2-related: Regression based affine candidate derivation", Y. Zhang, H. Huang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)

JVET-Z0126, "Non-EE2: Improvement on Local Illumination Compensation", Z. Xie, Y. Yu, H. Yu, D. Wang (OPPO)

JVET-Z0127, "Non-EE2: On the maximum number of MHP merge candidates",	H. Huang, V. Seregin, M. Karczewicz (Qualcomm)

JVET-Z0130, "EE2-related: Motion compensation boundary padding", Z. Zhang, H. Huang, C.-C. Chen, Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm), F. Le Léannec, P. Andrivon, M. Radosavljevć, E.Thomas (Xiaomi)

JVET-Z0137, "Non-EE2: Adaptive Blending for GPM",	H. Gao, X. Xiu, W. Chen, H.-J. Jhu, C.-W. Kuo, N. Yan, X. Wang (Kwai)

JVET-Z0142, "EE2-related: On MMVD and Affine MMVD Extension",	M. Salehifar, Y. He, K. Zhang, N. Zhang, L. Zhang (Bytedance)

JVET-Z0145, "EE2-2.5 related: More test results for ARMC with refined motion", Y. Wang, K. Zhang, N. Zhang, Z. Deng, L. Zhang (Bytedance)

JVET-Z0148, "EE2-2.5 related: AMVP with MVP refinement and ARMC with refined motion",	C. Zhou, Z. Lv, J. Zhang (vivo), Y. Wang, K. Zhang, N. Zhang, Z. Deng, L. Zhang (Bytedance)

JVET-Z0184, "EE2-3.5-related: Additional results of zero-vector candidates replacement in the IBC Merge/AMVP list", D. Ruiz Coll, A. Filippov, V. Rufitskiy (Ofinno)

## RPR (3)

JVET-Z0062, "AHG12: RPR luma filter modifications and RPR enabling fixes in ECM",	 R. Yu, K. Andersson (Ericsson)

JVET-Z0067, "Non-EE2: RPR bug fixes and new results of RPR with luma-only re-scaling", P. Bordes, F. Galpin, H. Guermoud, E. Francois (InterDigital)

JVET-Z0079, "Non-EE2: Support RPR on ECM-4.0 and handling method for template matching based coding tools", H. Jang, J. Nam, N. Park, J. Lim, S. Kim (LGE)

## Transform Coding (2)

JVET-Z0048, "EE2-related: Modification of LFNST for MIP coded block", J.-Y. Huo, W.-H. Qiao, X. Hao, Y.-Z. Ma, F.-Z. Yang (Xidian Univ.), M. Li, Y. Liu (OPPO)

JVET-Z0147, "Non-EE2: Improvement of Inter-MTS in ECM",	B. Ray, V. Seregin, M. Karczewicz (Qualcomm)

## Screen Content Coding (3)

JVET-Z0131, "EE2-related: Block Vector Difference Binarization", K. Cao, V. Seregin, M. Karczewicz (Qualcomm)

JVET-Z0152, "EE2-related: IBC Merge Mode with Block Vector Differences", N. Zhang, J. Xu, K. Zhang, M. Salehifar, L. Zhang (Bytedance)

JVET-Z0159, "Non-EE2: Reconstruction-Reordered IBC for screen content coding", Z. Deng, K. Zhang, L. Zhang (Bytedance)

## ECM Encoder and Test Conditions (2)

JVET-Z0072, "AHG10/AHG12: Enhanced reference picture structures for ECM and VTM", K. Andersson, R. Sjöberg, R. Yu, L. Litwic (Ericsson)

JVET-Z0150, "Memory usage report on VTM / ECM", T. Hashimoto, Y. Yasugi, T. Ikai (Sharp)

# Recommendations

The AHG recommends to:

- To review all the related contributions.