| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0023 |
| --- | --- |

| Title: | EE1: Summary of Exploration Experiments on Neural Network-based Video Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang, and L.Wang | Tel: Email: | elena.alshina@huawei.com chenwei06@kwai.com Franck.Galpin@InterDigital.com yue.li@bytedance.com mazhan@nju.edu.cn hongtaow@qti.qualcomm.com liqiangwang@tencent.com |
| Source: | EE coordinators |  |  |

# _____________________________

# Abstract

This document summarizes Exploration Experiment 1 (EE1) tests performed between the JVET-Y and JVET-Z meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, and analyze their complexity aspects. In NN-based filter category BD-rate gain over AhG11 anchor is 8.2...11.5% (depending on complexity) is demonstrated. In Super Resolution category 1.1 % BD-rate average gain over AhG11 anchor can be achieved (among them 3% for 4K content).

# Introduction

In total 7 tests studying NN-based filter technologies and 4 tests for Super Resolution technologies have been conducted. Several improvements as well as simplifications on top of technologies studied in EE1 have be proposed in EE1-related contributions. An effect of NN parameters quantizing to fixed point 16 and even 8 bits precision was studied.

Viewing was prepared and conducted together with SC 29 AG 5 for the proposals with highest BD-rate gain over AhG11 anchor.

Anchor and test conditions in EE1 remains un-changed for the long time. Ahg11 conducts testing for 5 QPs (22, 27, 32, 37, 42) [1]. Common reference is VTM11.0 with improved GOP-based temporal filter [2]. SALD [3] implementation for two major NN-based filters architectures have been tested in EE1.

# NN-based filters category

Three “clusters” NN-filters providing BD-rate saving relatively to AhG11 anchor can be identified. They are marked by red, green and blue circles on Fig. 1 and 2.

Fig. 1. BD-rate relatively to anchor in RA test for some NN-based filter depending on kMAC/pxl.

Fig. 2. BD-rate relatively to anchor in RA test for some NN-based filter depending on total memory size for all NN parameters.

Test EE1-1.2 with NN parameters quantized to Int8 likely the least complex among variants depicted on Fig. 1-2. But still computation complexity in kMAC/pxl is higher than 400 (while 64 is reference number for RTX 3080, 4K@60).

Three major NN-based filters architectures have been used in EE1 test.

# 2.1. NN-based filter architecture originated by Bytedance and Qualcomm.

The most popular among them was proposed by Bytedance [4] in JVET-X0065 (Fig. 3) enforced by residual scaling and some other elements by  Qualcomm [5] JVET-X0140, later improved by Ericsson [7] and finally InterDigital provided SALD [3] implementation for inference.

In this filter design Y is processed separately, U and V components re processed jointly.

Fig. 3. Basic aarchitecture of Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling. M denotes the number of feature maps. N stands for the number of samples in one dimension. (b) Construction of Attention Residual Block in (a)

The calculation process in the attention module can be written as:

F_out = F_in × f (Rec, Pred, BS, QP) + F_in

where F_in and F_out denote the input and the output of the attention module, respectively. Rec, Pred, BS, and QP stand for the reconstruction, the prediction, the boundary strengths, and the sequence-level input quantization parameter respectively.

NN parameters precision for all tests in this sub-category is Float 32. Test results in format recommended by AhG11 are summarized in Table 1. Encoding time (RA cfg) is 5 (SADL) / 1.3 (libtorch) of AhG11 anchor, decoder run time is 2000 (SADL) / 280 (libtorch) of AhG11 anchor (both ran on CPU). Training time for default version with 8residual Blocks is ~60 hour per model.

Shift of some Chroma gain to Luma by Chroma QP offset adjustment if safer in all-intra configuration: 10% Chroma BD-rate degradation result is 0.7% Luma BD-rate improvement. In RA cfg 10% Chroma BD-rate degradation provides only 0.4% extra gain for Luma. But increment of Chroma QP by 1 makes NN-based filter gain more balanced (thanks to Xidian Uni. and ZTE for studying this aspect).

SADL implementation for inference provides 0.4% better gain (Test EE1-1.6 vs EE1-1.7) than PyTorch inference.

Table 1. Performance results for NN-filters based on architectures from Bytedance and Qualcomm.

|  |  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  | Inference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contribution | Source | Total Number of Parameters (Millions) | Worst Case Complexity (kMAC /pixel) | Y | Cb | Cr | Y | Cb | Cr |  |
| Split ALF in combination with JVET-X0065 |  |  |  |  |  |  |  |  |  |  |
| EE1-1.4.1 | JVET-Z0086 | 36.7 | 630.0 | -10.0% | -23.9% | -23.5% | -7.4% | -20.7% | -20.9% | PyTorch v1.6 |
| EE1-1.4.2 | JVET-Z0086 | 36.7 | 630.0 | -10.4% | -13.7% | -13.3% | -8.7% | -10.9% | -11.3% | PyTorch v1.6 |
| Solution for blocking artifacts ( JVET-Y0143) |  |  |  |  |  |  |  |  |  |  |
| EE1-1.3 | JVET-Z0070 | 6.3 | 649.0 | -10.0% | -21.6% | -21.4% | -7.3% | -19.8% | -20.1% | PyTorch v1.6 |
| EE1-1.6-8RB | JVET-Z0112 | 6.2 | 649.0 | -9.8% | -21.2% | -21.1% | -7.4% | -19.3% | -19.4% | SADL |
| EE1-1.6-16RB | JVET-Z0112 | 11.6 | 1151.0 | -11.0% | -23.0% | -24.5% | -8.4% | -21.5% | -22.5% | SADL |
| SADL implementation |  |  |  |  |  |  |  |  |  |  |
| EE1-1.7-8RB | JVET-Z0113 | 6.2 | 649.0 | -10.2% | -22.2% | -22.1% | -7.2% | -19.7% | -20.0% | SADL |
| EE1-1.7-16RB | JVET-Z0113 | 11.6 | 1151.0 | -11.4% | -23.9% | -25.4% | -8.2% | -22.2% | -23.3% | SADL |
| Advanced RDO for filter selection |  |  |  |  |  |  |  |  |  |  |
| EE1-1.6-RDO | JVET-Z0154 | 11.6 | 1151.0 | -11.5% | -23.1% | -24.6% | -8.9% | -21.6% | -22.7% | SADL |
| EE1-1.7-RDO | JVET-Z0155 | 11.6 | 1151.0 |  |  |  | -8.5% | -22.4% | -23.5% | SADL |
| Ablations study |  |  |  |  |  |  |  |  |  |  |
| EE1-1.6-8RB-retrain | JVET-Z0106 | 6.2 | 649.0 |  |  |  | -7.4% | -18.6% | -19.1% | PyTorch v1.9 |
| EE1-1.6-8RB-retrain-long | JVET-Z0106 | 6.2 | 649.0 |  |  |  | -7.6% | -18.5% | -19.1% | PyTorch v1.9 |
| EE1-1.6-8RB-w/o part | JVET-Z0106 | 6.2 | 638.0 |  |  |  | -7.6% | -18.4% | -19.3% | PyTorch v1.9 |
| EE1-1.6-8RB-w/o BS | JVET-Z0106 | 6.2 | 638.0 |  |  |  | -7.6% | -18.4% | -19.0% | PyTorch v1.9 |
| EE1-1.6-8RB-w/o part &BS | JVET-Z0106 | 6.2 | 628.0 |  |  |  | -7.4% | -18.6% | -19.2% | PyTorch v1.9 |
| EE1-1.6-8RB-w/o pred | JVET-Z0106 | 6.2 | 638.0 |  |  |  | -7.3% | -18.5% | -19.1% | PyTorch v1.9 |
| EE1-1.6-4RB | JVET-Z0106 | 5.6 | 481.0 |  |  |  | -6.8% | -19.3% | -19.6% | PyTorch v1.9 |
| EE1-1.6-16RB | JVET-Z0106 | 7.6 | 985.0 |  |  |  | -8.3% | -17.7% | -18.5% | PyTorch v1.9 |
| EE1-1.6-8RB att.last | JVET-Z0106 | 6.2 | 646.0 |  |  |  | -7.6% | -18.7% | -19.6% | PyTorch v1.9 |
| EE1-1.6-8RB w/o att. | JVET-Z0106 | 6.2 | 646.0 |  |  |  | -7.6% | -18.6% | -19.2% | PyTorch v1.9 |

In EE1 related contribution ablation study for this NN-based filter architecture as performed. Ericsson was able to re-train model from the scratch, and so verify training procedure. As expected after re-training performance is slightly different, but difference is 0.0% BD-rate Y and up to 0.7% BD-rate Chroma.

Another useful finding from Ericson is longer training (378 epochs instead 155) improves performance by 0.2% BD-rate (in all intra cfg).

Results of ablation study by Ericsson (JVET-Z0106) are visualized on Fig. 4. All test results (except just retrain) corresponds long training.

The change for number of residual blocks (default is 8, 4 and 16 have been tested) has major effect on performance and complexity. Red line of Fig. 4 connects RB4, RB8 and RB16 variants of NN filter. The curvature of this curve shows that number of residual blocks 8 is optimal performance/complexity trade-off.

Removal from design prediction information provides the biggest performance drop. Removal of BS or partitioning information, as well as disabling attention mechanism provides almost not performance / complexity change. Even removal both partitioning and BS from design is still close to the optimal, but reduces tensors size in the memory.

Fig. 4. Ablation study by Ericsson (JVET-Z0106).

It should be noticed that ablation test have been provide in all-intra cfg only. In Inter configuration an effect of various elements of NN-filter design might be very different. Such kind of study can be recommended.

# 2.2. NN-based filter architecture originated by Tencent.

In this filter design Y, U and V components are processed jointly. Two variants tested in EE1 are sown on Fig. 5. They were proposed by Tencent in

Fig. 5. Basic NN-filter architecture by Tencent: with two models EE1-1.1 (left) and with single model models EE1-1.2 (right).

Performance results for both variants is very close. The number of parameters in single model variant is 1.5 smaller. Smaller number of parameters is compensated by extra input (Slice type). It is reported that the training time ~150 hours for both variants.

Test results in format recommended by AhG11 are summarized in Table 2. Implementation change from Torch to SALD w/o change for NN parameters precision (32 bits) is nearly lossless. Encoding and decoding run-time surprisingly higher SADL variant: Enc. 1.2 of anchor (libtorch)  2.0 of anchor (SADL), Dec. 300 of anchor (libtorch)  1700 of anchor (SADL).

Table 2. Performance results for NN-filters based on architectures from Tencent.

|  |  |  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tets | Bits | Total Number of Parameters (Milli) |  | Worst Case Complexity (kMAC /pixel) | Y | Cb | Cr | Y | Cb | Cr |  |
| EE1-1.1.1 | F32 | 3.1 | 401.0 |  | -8.8% | -19.9% | -19.4% | -7.4% | -16.8% | -17.3% | Torch |
| EE1-1.1.2 | F32 | 3.1 | 401.0 |  | -8.8% | -19.9% | -19.4% | -7.4% | -16.8% | -17.3% | SADL |
| EE1-1.1.3-int16 | Int16 | 3.1 | 401.0 |  | -8.8% | -19.9% | -19.4% | -7.4% | -16.8% | -17.3% | SADL |
| EE1-1.1-int8 | Int8 | 3.1 | 401.0 |  | -8.2% | -19.0% | -18.1% | -6.3% | -15.0% | -16.1% | SADL |
| Related EE1-1.1.1 | F 32 | 1.5 | 401.0 |  | -9.0% | -20.0% | -19.5% |  |  |  |  |
| EE1-1.2.1 | F 32 | 1.9 | 485.0 |  | -8.7% | -18.6% | -19.0% | -6.5% | -14.9% | -16.0% | Torch |
| EE1-1.2.2 | F 32 | 1.9 | 485.0 |  | -8.7% | -18.6% | -19.0% | -6.5% | -14.9% | -16.0% | SADL |
| EE1-1.2.3-int16 | Int16 | 1.9 | 485.0 |  | -8.7% | -18.7% | -19.0% | -6.5% | -14.9% | -16.0% | SADL |
| EE1-1.2-int8 | Int 8 | 1.9 | 485.0 |  | -8.6% | -18.1% | -18.4% | -6.3% | -14.4% | -15.4% | SADL |

Quantizing NN parameters to 16 bits comes almost w/o performance change. With quantized to 8 bits precision parameters NN filter performance slightly degrades (0.1~0.2% BD-rate) for variant with single model (EE1-1.2). For the variant with 2 models degradation due to NN parameter quantization to 8 bits is higher (~0.6% BD-rate).

It is expected that additional speed up is possible if not only NN weights but also tensor values after activation will be quantized to limited (16, or even better 8 bits) precision.

# 2.3. CNN post-processing filter based on depthwise separable convolution and attention mechanism.

Filter design in this test used WCDANN architecture (Fig. 6). This design was proposed in [1] by OPPO and Xidian Uni.

(a)

(b)

(c)

Fig. 6. (a) Architecture of WCDANN. WCDANN consists of M WCDABs. “3ⅹ3 Conv” denotes 3ⅹ3 common convolution layer. ReLU denotes Rectified Linear Unit for activation. Each Conv has 64 channels. WCDABM denotes the M-th weakly connect dense attention block.(b) Architecture of WCDAB. WCDAB consists of M WCDABs. “1ⅹ1 Conv” denotes 1ⅹ1 common convolution layer. RABi denotes i-th residual attention block (i=1, …,4). CSAB denotes channel-spatial joint attention block. (c) Architecture of RAB.

This NN architecture if implemented with conventional convolutions is very “heavy” (kMAC/pxl is over 3000, number of parameters 4.7 Millions). If implemented with Depth-Wise Separable convolutions then both complexity characteristics (kMAC/pxl and total number of parameters) reduce by ×4, while performance is almost unchanged. Unfortunately only Luma BD-rate was reported in EE test, hopefully JVET will see Chroma BD-rate as well in a future.

# Super Resolution category

In this round of EE eventually apple-to-apple comparison for super-resolution with RPR filer up-sampling and NN-based super-resolution becomes possible. Tables 3 and 4 summarized performance test results and complexity.

It tests EE1-2.1.1 LGE combines existing in VVC standard RPR re-sampling filter with adaptive selection of coded picture size [11].

It tests EE1-2.1.2 additionally Chroma QP is reduced in order to balance Luma / Chroma performance. Still some average gain (0.4%) relatively to AhG11 anchor is preserved in RA configuration. In all intra after Chroma QP adjustment there is no gain in average.

Table 3. Performance results for test in Super Resolution category. Average BD-rate (all classes) is reported.

|  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contribution | Total Number of Parameters (Millions) | Worst Case Complexity (kMAC /pixel) | Y | Cb | Cr | Y | Cb | Cr |  |
| EE1-2.1.1 | 0.0 | 0.0 | -1.0% | 2.6% | 2.5% | -0.6% | 3.1% | 2.7% | RPR filter & adaptive size |
| EE1-2.1.2 | 0.0 | 0.0 | -0.4% | -1.4% | -1.4% | 0.0% | 0.5% | 0.0% | RPR filter & adaptive size |
| EE1-2.2 | 2.4 | 244.0 | 52.7% | 11.1% | 11.6% | 14.8% | 1.5% | -4.7% | NN-filter1 w/o adaptive size |
| EE1-2.3 | 7.0 | 854.0 | 51.8% | -7.6% | -10.6% | 12.4% | -12.5% | -20.2% | NN-filter1 w/o adaptive size |
| EE1-2.4 | 7.0 | 854.0 | -1.3% | -0.6% | -0.8% | -1.1% | -0.4% | -0.7% | NN-filter2 & adaptive size |

Two variants of NN-based filets tested in this EE1 round. Both are initiated by Bytedance [12], [13]. Luma is up-sampled separately. Chorma up-sampling NN receives Luma as extra input.

(a)

(b)

Fig. 7 (a) Architecture of Luma (a) and Chroma (b) up-sampling NN.

In test EE1-2.3 U and V use dedicated network, while in test EE1-2.2 same network is used for U and V. This explains difference in complexity.

If NN-based up-sampling filter from EE1-2.3 replaces RPR filter in test EE1-2.1 (adaptive coded picture size). Then extra 0.3% gain in RA cfg amf 0.5% in all-intra cfg can be obtained. It shall be noticed that adaptive coded picture size selection is more beneficial for high resolution content. Table 4 shows results for only 4K classes (A1 and A2).

Table 4. Performance results for test in Super Resolution category. Average BD-rate (only 4K).

|  |  |  | Random Access (CTC) |  |  | All Intra (CTC) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Contribution | Total Number of Parameters (Millions) | Worst Case Complexity (kMAC /pixel) | Y | Cb | Cr | Y | Cb | Cr |  |
| EE1-2.1.1 | 0.0 | 0.0 | -2.5% | 5.0% | 5.2% | -2.2% | 7.5% | 6.1% | RPR filter & adaptive size |
| EE1-2.1.2 | 0.0 | 0.0 | -1.3% | -2.9% | -2.3% | -0.9% | 2.0% | 0.8% | RPR filter & adaptive size |
| EE1-2.2 | 2.4 | 244.0 | -5.9% | 18.5% | 7.8% | -8.1% | 33.2% | 0.8% | NN-filter1 w/o adaptive size |
| EE1-2.3 | 7.0 | 854.0 | -6.5% | 0.3% | -12.7% | -9.2% | 13.8% | -13.8% | NN-filter1 w/o adaptive size |
| EE1-2.4 | 7.0 | 854.0 | -3.0% | -1.4% | -1.7% | -2.7% | -0.8% | -1.5% | NN-filter2 & adaptive size |

Replacement of RPR filter by NN-based one gives more or less same performance improvement. Average gain achieved for 4K content is almost 3%.

# Visual test

The best performing NN-based filters (tests EE1-1.2 and EE1-1.7) were selected for viewing. Also super resolution with RPR filter (EE1-2.1) and NN based filters (EE1.2.4) went to the viewing.  Viewing results are discussed in JVET-Z0053. It appears that blocking artifacts problem of EE1.7 still not resolved.

# Cross-checks

During preparing EE1 summary some complexity numbers were corrected by cross-checkers. NN-based filters performance was verified by OPPO (cross-checked tests EE1-1.1, 1.2, 1.6 and 1.7) and Bytedance (cross-checked test EE1-1.4).

# Recommendations

Continue EE study, considering ablation study. Pay more attention during cross-check not only for verification performance results but also complexity numbers. Study inference implementation for limited precision for NN parameters and for activations to provide more possibilities for optimization. Revise summary template [14] used for NN-based proposals: 1) add encoder/decoder run-time information, 2) remove only 4K results. Conduct BoG for summary template revision and double-check performance/complexity numbers for non-EE1 proposals.

# References

[1] S. Liu, A. Segall, E. Alshina, R.-L. Liao Common Test Conditions and evaluation procedures for neural network-based video coding technology JVET-W2016.

[2] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/tree/master/Software%20Patches

[3] F. Galpin, F. Mom, T. Dumas, P. Bordes, P. Nikitin, E. François (InterDigital)  AHG11: Small Ad-hoc Deep-Learning Library  JVET-Y0110 AHG11: Small Ad-hoc Deep-Learning Library (SADL) update.

[4] Y. Li, K. Zhang, L. Zhang  EE1-1.2: Test on Deep In-Loop Filter with Adaptive Model Selection and External Attention, JVET-X0065

[5] H. Wang, J. Chen, K. Reuze, A.M. Kotra, M. Karczewicz (Qualcomm) EE1-1.4: Tests on Neural Network-based In-Loop Filter with Constrained Computational Complexity, JVET-X0140

[6] Y. Li, K. Zhang, L. Zhang (Bytedance), H. Wang, K. Reuze, A.M. Kotra, M. Karczewicz (Qualcomm) EE1-1.2: Test on Deep In-Loop Filter with Adaptive Parameter Selection and Residual Scaling JVET-Y0143

[7] K.Andersson, J. Ström, D. Liu, R. Sjöberg (Ericsson) EE1-related: Combination of VVC deblocking and NN loop filtering JVET-Y0098

[8] L. Wang, X. Xu, S. Liu (Tencent) EE1-1.1: neural network based in-loop filter with constrained storage and low complexity JVET-Y0078

[9] L. Wang, X. Xu, S. Liu (Tencent) EE1-1.1-related: alternative filter designs JVET-Y0078

[10] H. Zhang, C. Jung (Xidian University), D. Zou, M. Li (OPPO) AHG11: CNN post-processing filter based on depthwise separable convolution and attention mechanism JVET-Y0052

[11] J. Nam, S. Yoo, J. Lim, S. Kim (LGE) EE1-2.1-related: RPR encoder with multiple scale factors JVET-Y0068

[12] C. Lin, Y. Li, K. Zhang, L. Zhang (Bytedance) EE1-2.3: CNN-based Super Resolution for Video Coding Using Decoded Information JVET-Y0069

[13] C. Lin, Y. Li, K. Zhang, L. Zhang (Bytedance) EE1-2.4: CNN-based Super Resolution for Video Coding Using Separate Networks for Chroma Components JVET-Y0070

[14] A. Segall BoG Report: EE1 Viewing Preparation and Neural Networks Video Coding Results Analysis JVET-X0188