| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0024-v3 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Ling Li Karam Naser Jacob Ström Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com aurali@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as intra prediction, inter prediction, screen content coding, and entropy coding.

The software basis for this EE is ECM-4.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0. ECM-4.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-y-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-y-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Intra prediction |  |  |  |
| 1.1 | Slope adjustment for CCLM | Nokia J. Lainema JVET-Z0049 | OPPO Z. Xie JVET-Z0174 |
| 1.2a | DIMD chroma mode | Alibaba X. Li JVET-Z0051 | InterDigital K. Naser JVET-Z0185 |
| 1.2b | Fusion of chroma intra prediction modes | Alibaba X. Li JVET-Z0051 | InterDigital K. Naser JVET-Z0185 |
| 1.2c | Test 1.2a + Test 1.2b | Alibaba X. Li JVET-Z0051 | InterDigital K. Naser JVET-Z0185 |
| 1.2d | Test 1.2a with reduced processing + Test 1.2b | Alibaba X. Li JVET-Z0051 | InterDigital K. Naser JVET-Z0185 |
| 1.3a | Combination of Test 1.1 and Test 1.2c | Nokia J. Lainema Alibaba X. Li JVET-Z0050 | Qualcomm Y.-J. Chang JVET-Z0167 |
| 1.3b | Combination of Test 1.1 and Test 1.2d | Nokia J. Lainema Alibaba X. Li JVET-Z0050 | Qualcomm Y.-J. Chang JVET-Z0167 |
| 2 Inter prediction |  |  |  |
| 2.1a | Extended active reference pictures | Qualcomm H. Huang JVET-Z0054 | Alibaba R.-L. Liao |
| 2.1b | Block-level reference picture reordering | Qualcomm H. Huang JVET-Z0054 | Alibaba R.-L. Liao |
| 2.1c | Test 2.1a + Test 2.1b | Qualcomm H. Huang JVET-Z0054 | Alibaba R.-L. Liao |
| 2.2a | Enhanced bi-directional motion compensation | Kwai Y.-W. Chen JVET-Z0136 | Tencent G. Li InterDigital A. Robert JVET-Z0208 |
| 2.2b | Test 2.2a with BDOF modifications | Qualcomm Z. Zhang Kwai Y.-W. Chen JVET-Z0136 | Tencent G. Li |
| 2.2c | Test 2.2b with discarded BDOF offsets | Kwai Y.-W. Chen Qualcomm Z. Zhang | withdrawn |
| 2.3 | Template matching based OBMC | vivo Z. Lv JVET-Z0061 | Qualcomm Y.-J. Chang JVET-Z0166 |
| 2.4 | Template matching based reordering for GPM split modes | Qualcomm C.-C. Chen JVET-Z0056 | KDDI Y. Kidani |
| 2.5a | ARMC with refined motion | Bytedance Y. Wang JVET-Z0058 | Wuhan Univ. G. Ren JVET-Z0163 |
| 2.5b | Test 2.5a with reduced complexity | Bytedance Y. Wang JVET-Z0058 | Wuhan Univ. G. Ren JVET-Z0163 |
| 2.6a | 12-tap interpolation filter for chroma | Bytedance X. Xie | withdrawn |
| 2.6b | 8-tap interpolation filter for chroma | Bytedance X. Xie | withdrawn |
| 2.6c | 6-tap interpolation filter for chroma | Bytedance X. Xie JVET-Z0117 | Kwai X. Xiu |
| 2.7a | History based affine inheritance without tables stored in the line buffer | Bytedance K. Zhang JVET-Z0139 | Xiaomi P. Andrivon |
| 2.7b | History based affine inheritance with tables stored in the line buffer | Bytedance K. Zhang JVET-Z0139 | Xiaomi P. Andrivon |
| 2.7c | Test 2.7a + affine candidates derived from non-adjacent blocks | Bytedance K. Zhang JVET-Z0139 | Tencent L.-F. Chen |
| 2.7d | Test 2.7b + affine candidates derived from non-adjacent blocks | Bytedance K. Zhang | withdrawn |
| 2.7e | Test 2.7 with template matching disabled | Bytedance K. Zhang | withdrawn |
| 2.8a | Non-adjacent affine model derivation without constrained memory usage | Kwai W. Chen JVET-Z0139 | Qualcomm Y. Zhang |
| 2.8b | Non-adjacent affine model derivation with constrained memory usage | Kwai W. Chen JVET-Z0139 | Qualcomm Y. Zhang |
| 2.8c | Test 2.8 with template matching disabled | Kwai W. Chen | withdrawn |
| 2.9a | Test 2.8a + Test 2.7 | Kwai W. Chen | withdrawn |
| 2.9b | Test 2.8b + Test 2.7 | Kwai W. Chen JVET-Z0139 | Tencent L.-F. Chen |
| 4 Screen content coding |  |  |  |
| 3.1 | Cross-component palette coding | Bytedance B. Vishwanath JVET-Z0055 | Nokia J. Lainema |
| 3.2 | IBC with extended reference area | Bytedance J. Xu JVET-Z0153 | Ofinno D. Ruiz Coll |
| 3.3 | Enlarged HMVP table for IBC | Bytedance N. Zhang JVET-Z0075 | Ofinno D. Ruiz Coll |
| 3.4 | IBC with template matching | InterDigital A. Robert JVET-Z0084 | Ofinno D. Ruiz Coll |
| 3.5a | Invalid BVP candidate adjustment based on IBC reference region | Ofinno D. Ruiz Coll JVET-Z0160 | Qualcomm K. Cao |
| 3.5b | Replacement of zero-vector candidates in IBC Merge/AMVP list | Ofinno D. Ruiz Coll JVET-Z0160 | Qualcomm K. Cao |
| 3.5c | Test 3.5a + Test 3.5b | Ofinno D. Ruiz Coll JVET-Z0160 | Qualcomm K. Cao |
| 3.6a | Test 3.2 + Test3.3 | Bytedance J. Xu, N. Zhang JVET-Z0165 | Ofinno D. Ruiz Coll |
| 3.6b | Test 3.2 + Test 3.4 | Bytedance J. Xu InterDigital A. Robert JVET-Z0095 JVET-Z0165 | Ofinno D. Ruiz Coll |
| 3.6c | Test 3.2 + Test 3.5c | Bytedance J. Xu Ofinno D. Ruiz Coll JVET-Z0165 |  |
| 3.6e | Test 3.3 + Test 3.4 | Bytedance N. Zhang InterDigital A. Robert JVET-Z0095 JVET-Z0165 | Ofinno D. Ruiz Coll |
| 3.6f | Test 3.3 + Test 3.5c | Bytedance N. Zhang Ofinno D. Ruiz Coll JVET-Z0165 | Alibaba R.-L. Liao |
| 3.6g | Test 3.4 + Test 3.5c | InterDigital A.Robert Ofinno D. Ruiz Coll JVET-Z0095 JVET-Z0165 |  |
| 3.6h | Test 3.2 + Test 3.3 + Test 3.4 | Bytedance J. Xu, N. Zhang InterDigital A. Robert JVET-Z0095 JVET-Z0165 | Ofinno D. Ruiz Coll |
| 3.6i | Test 3.2 + Test 3.3 + Test 3.5c | Bytedance N. Zhang Ofinno D. Ruiz Coll JVET-Z0165 | InterDigital A. Robert JVET-Z0207 |
| 3.6j | Test 3.2 + Test 3.4 + Test3.5c | Bytedance J. Xu, N. Zhang InterDigital A.Robert Ofinno D. Ruiz Coll JVET-Z0165 |  |
| 3.6k | Test 3.3 + Test 3.4 + Test 3.5c | Bytedance J. Xu, N. Zhang InterDigital A.Robert Ofinno D. Ruiz Coll JVET-Z0165 |  |
| 3.6l | Test 3.2 + Test 3.3 + Test 3.4 + Test 3.5c | Bytedance J. Xu, N. Zhang InterDigital A.Robert Ofinno D. Ruiz Coll JVET-Z0165 | Alibaba R.-L. Liao |
| 4 Entropy coding |  |  |  |
| 4.1a | Probability estimation with adaptive weights | Kwai X. Xiu JVET-Z0134 | Qualcomm V. Seregin Alibaba J. Chen |
| 4.1b | Test 4.1a + inherited context initialization | Kwai X. Xiu JVET-Z0134 | Qualcomm V. Seregin Alibaba J. Chen |
| 4.2a | Temporal CABAC initialization | Qualcomm V. Seregin JVET-Z0133 | Kwai X. Xiu |
| 4.2b | Adaptive window size adjustment | Qualcomm V. Seregin JVET-Z0133 | Kwai X. Xiu |
| 4.2c | Test 4.2a + Test 4.2b | Qualcomm V. Seregin JVET-Z0133 | Kwai X. Xiu OPPO Kazushi Sato JVET-Z0173 |
| 4.3a | Test 4.1a + Test 4.2b | Qualcomm V. Seregin Kwai X. Xiu JVET-Z0135 | Xiaomi P. Andrivon LGE J. Zhao Alibaba J. Chen |
| 4.3b | Test 4.1a + Test 4.2b + Test 4.2a | Qualcomm V. Seregin Kwai X. Xiu JVET-Z0135 | Xiaomi P. Andrivon LGE J. Zhao Alibaba J. Chen |

# Description of tests

## Intra prediction

### Test 1.1: Slope adjustment for CCLM

CCLM uses a model with 2 parameters to map luma values to chroma values. The slope parameter “a” and the bias parameter “b” define the mapping as follows:

chromaVal = a * lumaVal + b

It is proposed signal an adjustment “u” to the slope parameter to update the model to the following form:

chromaVal = a’ * lumaVal + b’,

where

a’ = a + u

b’ = b – u * yr

As a result, the mapping function is rotated around the point with luminance value yr chosen to be the average of the reference luma samples as shown in Figure 1.

Slope adjustment integer parameter u ranges between -4 and 4, inclusive, and signaled in the bitstream, the unit of the parameter is 1/8th of a chroma sample value per one luma sample value (for 10-bit content).

Figure 1. Current (left) and proposed CCLM (right)

### Test 1.2: DIMD chroma mode and fusion of intra modes

In Test 1.2a, DIMD chroma mode is tested, chroma prediction of the current block is based on the collocated reconstructed luma samples where horizontal and vertical gradients are calculated for each collocated reconstructed luma sample of the current chroma block to build a HoG as shown in Figure 2.

Figure 2. Collocated reconstructed luma samples for DIMD chroma mode

When the intra prediction mode derived from the DIMD chroma mode is the same as the intra prediction mode derived from the DM mode, the intra prediction mode with the second largest histogram amplitude value is used as the DIMD chroma mode. A CU level flag is signaled to indicate whether the proposed DIMD chroma mode is applied.

In Test 1.2b, DM mode and the four default modes can be fused with the MMLM_LT mode as follows:

pred=w0*pred0+w1*pred1+(1≪(shift-1))≫shift

where pred0 is the predictor obtained by applying the non-LM mode, pred1 is the predictor obtained by applying the MMLM_LT mode and pred is the final predictor of the current chroma block. The two weights, w0 and w1 are determined by the intra prediction mode of adjacent chroma blocks as when the above and left adjacent blocks are both coded with LM modes, {w0, w1}={1, 3}; when the above and left adjacent blocks are both coded with non-LM modes, {w0, w1}={3, 1}; otherwise, {w0, w1}={2, 2} , and shift is set equal to 2.

For non-LM mode, a flag is signaled to indicate whether the fusion is applied, the fusion is only applied in I slices.

In Test 1.2c the DIMD chroma mode and the fusion of chroma intra prediction modes are combined where DIMD chroma mode is applied, and for I slices, the DM mode, the four default modes and the DIMD chroma mode can be fused with the MMLM_LT mode, while for non-I slices, only the DIMD chroma mode can be fused with the MMLM_LT mode using equal weights.

In Test 1.2d, the same combination as in Test 1.2c is tested while DIMD chroma mode is derived based on the neighboring reconstructed Y, Cb and Cr samples in the second neighboring row and column as shown in Figure 3, instead of using collocated reconstructed luma samples.

Figure 3. Reconstructed neighbor sample used in DIMD chroma mode

### Test 1.3: Combination of intra tests

Test 1.3a is a combination of Test 1.1 and Test 1.2c.

Test 1.3b is a combination of Test 1.1 and Test 1.2d.

## Inter prediction

### Test 2.1: Block-level reference picture reordering

In Test 2.1a, the number of active reference pictures in random access configurations is increased to the number of available reference pictures at encoder, and encoder fast algorithm is applied:

- The reference picture with the index larger than 1 in the affine motion estimation process is skipped if it is not selected in the regular AMVP mode.

- The reference picture with the index larger than 1 is skipped if the ME cost for the reference index 1 is larger than 1.25 times the cost of the reference index 0.

In Test 2.1b, block level reference pictures reordering based on template matching is tested. For the uni-prediction AMVP mode, the reference pictures in List 0 and List 1 are interweaved to generate a joint list. For each hypothesis of the reference picture in the joint list template matching is performed to calculate the cost. The joint list is reordered based on ascending order of the template matching cost. The index of the selected reference picture in the reordered joint list is signaled in the bitstream. For the bi-prediction AMVP mode, a list of pairs of reference pictures from List 0 and List 1 is generated and similarly reordered based on the template matching cost. The index of the selected pair is signaled in the bitstream.

In Test 2.1c, the block level reference picture reordering method is tested with the increased number of active reference pictures (Test 2.1a + Test 2.1b).

### Test 2.2: Enhanced bi-directional motion compensation

In Test 2.2a, one uni-directional MC sample is regarded as out-of-boundary (OOB) when one of its reference samples is located outside the reference picture beyond half sample. For each prediction sample in a bi-directional block, when one of its uni-directional prediction samples is OOB and the other one is non-OOB, the corresponding bi-directional sample is set equal to the non-OOB sample instead of weighted average of the two uni-directional prediction samples.

BDOF gradients are calculated using the original uni-prediction before OOB processing, and BDOF offset is added to unmodified by OOB handling samples, other BDOF offsets are discarded.

In Test 2.2b, the OOB checking is conducted prior to the BDOF process, where the OOB sample in one uni-directional prediction block is replaced by its collocated prediction sample from the other direction if the collocated sample is non-OOB. In this test, BDOF process is not modified.

### Test 2.3: Template matching based OBMC

Template matching based OBMC scheme is tested. In the method, sample blending operation is processed for each 4x4 top and left blocks at CU boundaries, and three template matching costs Cost1, Cost2, Cost3 are computed as SAD between the reconstructed samples of a template shown in Figure 4 and its corresponding reference samples derived by MC process according to the following three types of motion information described for block A as an example:

Cost1 is calculated according to A motion information.

Cost2 is calculated using the above neighbor A motion information.

Cost3 is calculated as a weighted prediction using A and the above neighbor A motion information with the weighting factors equal to 3/4 and 1/4, respectively.

Figure 4. OBMC template

The final predictor is done based on the Cost1, Cost2, and Cost3 comparison.

The original MC result using current block’s motion information is denoted as s1, and the MC result using neighboring block’s motion information is denoted as s2. The final prediction result is denoted as s.

- If Cost1 is the smallest, then s(i,j)= s1(i,j).

- If (Cost2 + (Cost2 >> 2) + (Cost2 >> 3)) <= Cost1, then blending mode 1 is used.

For luma blocks, the number of blending sample rows is 4.

s(i,0) = (26×s1(i,0) + 6×s2(i,0) + 16)≫5

s(i,1) = (7×s1(i,1) + s2(i,1) + 4)≫3

s(i,2) = (15×s1(i,2) + s2(i,2) + 8)≫4

s(i,3) = (31×s1(i,3) + s2(i,3) + 16)≫5

For chroma blocks, the number of blending sample rows is 1.

s(i,0) = (26×s1(i,0) + 6×s2(i,0) + 16)≫5

- If Cost1 <= Cost2, then blending mode 2 is used.

For luma blocks, the number of blending sample rows is 2.

s(i,0) = (15×s1(i,0) + s2(i,0) + 8)≫4

s(i,1) = (31×s1(i,1) + s2(i,1) + 16)≫5

For chroma blocks, the number of blending sample rows/columns is 1.

s(i,0) = (15×s1(i,0) + s2(i,0) + 8)≫4

- Otherwise, blending mode 3 is used.

For luma blocks, the number of blending sample rows is 4.

s(i,1) = (7×s1(i,1) + s2(i,1) + 4)≫3

s(i,2) = (15×s1(i,2) + s2(i,2) + 8)≫4

s(i,3) = (31×s1(i,3) + s2(i,3) + 16)≫5

For chroma blocks, the number of blending sample rows is 1.

s(i,0) = (7×s1(i,0) + s2(i,0) + 4)≫3

### Test 2.4: Template matching based reordering for GPM split modes

The reordering method for GPM split modes is a two-step process performed after the respective reference templates of the two GPM partitions in a coding unit are generated, as follows:

- GPM partition edge is extended into the reference templates of the two GPM partitions, resulting in 64 reference templates, and computing the TM cost for each reference template

- reordering GPM split modes based on their TM cost values in ascending order and marking the best 32 as available split modes.

The edge on the template is extended from the CU, as Figure 5 illustrates, but GPM blending is not used in the templates across the edge.

Figure 5. GPM templates and partition edge

After ascending reordering using TM cost, an index is signaled using Golomb-Rice code to indicate the use of GPM split mode.

### Test 2.5: ARMC with refined motion

In the method, each merge candidate in the merge candidate list is refined using TM/multi-pass DMVR and the refined motion is used in ARMC to reorder the merge candidate list as shown on Figure 6. ARMC with refined motion is applied to template matching merge (TM merge) mode, adaptive DMVR mode, and template matching for advanced motion vector prediction (TM-AMVP) mode. When multi-pass DMVR is used to derive the refined motion, only the first pass (i.e., PU level) of multi-pass DMVR is applied in reordering.

Figure 6. AMC with refined motion

In Test 2.5b, when template matching is used to derive the refined motion, the template size is set equal to 1 and only the first 8 merge candidates are reordered with the refined motion in TM merge mode.

### Test 2.6: Longer interpolation filters for chroma

6-tap chroma interpolation filters are tested in addition to 4-tap filters used for motion compensation. A flag is signaled in SPS to indicate whether 6-tap or 4-tap chroma interpolation filter is used. The 4-tap chroma interpolation filter is applied for sequences in class A1/A2 while the 6-tap interpolation filter is applied for sequences in other classes.

### Test 2.7: History based affine inheritance

In the test, affine model can be inherited from a previous affine-coded blocks which may not be neighboring to the current block. The tested method consists of the following aspects:

- Two history tables for affine inheritance to derive affine merge candidates

- Pair-wise affine candidates derived from history- and non-history-based affine candidates

- Increased sub-block merge list size from 5 to 15

- Regular merge candidates derived from the first affine history table

A history-parameter table is established, each entry stores a set of affine parameters: a, b, c and d, each of which is represented by a 16-bit signed integer. Entries are categorized by reference list and reference index, and 5 reference indices are supported for each reference list in the history table.

For each category, at most 7 entries can be stored, resulting in 70 entries totally in the history table. At the beginning of each CTU row, the number of entries for each category is initialized as 0.

A history-affine-parameter-based candidate is derived from a neighbouring 4×4 block denoted as A0, A1, A2, B0, B1, B2 or B3 in Figure 8 and a set of affine parameters stored in a corresponding entry in the history table. The MV of a neighbouring 4×4 block served as the base MV. The MV of the current block at position (x, y) is calculated as:

,

where (mvhbase, mvvbase) represents the MV of the neighbouring 4×4 block, (xbase, ybase) represents the center position of the neighbouring 4×4 block. (x, y) can be the top-left, top-right and bottom-left corner of the current block to obtain the corner-position MVs (CPMVs) for the current block, or it can be the center of the current block to obtain a regular MV for the current block.

A second history-parameter table with base MV information is appended. There are 9 entries in the second table, wherein an entry comprises a base MV, a reference index, 4 affine parameters for each reference list, and a base position. An additional merge candidate can be generated from the second table with the base MV information and the corresponding affine models stored as an entry. The difference between the first and the second tables is illustrated in Figure 7.

Figure 7. Affine candidate derived from the history tables

Pair-wise affine merge candidates are generated by two affine merge candidates which are history-derived or not history-derived. A pair-wise affine merge candidates is generated by averaging the CPMVs of existing affine merge candidates in the list.

Such candidates are added into the sub-block-based merge candidate list and affine AMVP candidate. The size of sub-block-based merge candidate list is increased from 5 to 15.

Besides, such candidates are used to derive MVs located at the center of the current block, as regular merge candidates.

In Test 2.7b, affine history tables stored in the above and above right CTUs can be used by the blocks of the current CTU.

The requested test with disabled template matching was not provided.

### Test 2.8: Non-adjacent affine model derivation

Non-adjacent spatial neighbor positions used in the regular merge mode are added for affine merge and affine AMVP modes. The tested method consists of the following aspects:

- Non-adjacent constructed and non-adjacent inherited affine candidates

- Affine candidates derived from neighboring and non-adjacent blocks

- Increased sub-block merge list size from 5 to 15

- ARMC subgroup size of affine merge is increased from 3 to 15

The motion information of the non-adjacent spatial neighbors is utilized to generate additional inherited and constructed affine merge candidates.

For inherited candidates, the same derivation process of the inherited affine merge candidates in the VVC is kept unchanged except that the CPMVs are inherited from non-adjacent spatial neighbors. The non-adjacent spatial neighbors are checked based on their distances to the current block, i.e., from near to far. At a specific distance, only the first available neighbor (that is coded with the affine mode) from each side (e.g., the left and above) of the current block is included for inherited candidate derivation. As indicated by the red dash arrows in Figure 8 (a), the checking orders of the neighbors on the left and above sides are bottom-to-up and right-to-left, respectively.

Figure 8. Spatial neighbors for deriving affine merge candidates: (a) for deriving inherited affine merge candidates (b) for deriving constructed affine merge candidates

For constructed candidates, as shown in the Figure 8 (b), the positions of one left and above non-adjacent spatial neighbors are firstly determined independently. After that, the location of the top-left neighbor can be determined accordingly which can enclose a rectangular virtual block together with the left and above non-adjacent neighbors. Then, as shown in the Figure 7, the motion information of the three non-adjacent neighbors is used to form the CPMVs at the top-left (A), top-right (B) and bottom-left (C) of the virtual block, which is finally projected to the current CU to generate the corresponding constructed candidates.

Figure 9. From non-adjacent neighbors to constructed affine merge candidates

One second type of constructed affine candidates that are derived from non-adjacent neighbors are introduced for affine merge mode. Those new constructed affine candidates are derived based on the same affine candidate construction scheme as in Test 2.7a. However, instead of using history-based look-up table, the non-translational affine parameters are inherited from the non-adjacent spatial neighbors. Specifically, the second type of affine constructed candidates are generated from the combination of 1) the translational affine parameters of adjacent neighboring 4x4 blocks; and 2) the non-translational affine parameters {a, b, c, d} inherited from the non-adjacent spatial neighbors as defined in Figure 8 (a).

Due to the inclusion of the additional non-adjacent candidates, the size of the affine merge candidate list is increased from 5 to 15. The subgroup size of the affine merge mode for the adaptive reordering method is increased from 3 to 15.

In Test 2.8b, below changes are applied to constrain the memory usage:

- The area from where the non-adjacent neighbors come is restricted to be within the current CTU (i.e., no additional storage requirements for line buffer).

- The storage granularity for affine motion information, including CPMVs and reference indexes, is reduced from 8x8 to 16x16 (i.e., only the affine motion from the top-left 8x8 block is saved). Additionally, the saved CPMVs are projected to each 16x16 block before storage, such that the position and size information are not needed.

- Only the top-left and top-right CPMVs are stored (i.e., always using 4-parameter affine model).

The requested test with disabled template matching was not provided.

### Test 2.9: Combined history based and non-adjacent affine candidates

Test 2.7c: Test 2.7a + the first type of constructed non-adjacent affine candidates from Test 2.8a.

The test contains the following aspects from Test 2.7 and Test 2.8:

- Two history tables for affine inheritance to derive affine merge candidates

- Pair-wise affine candidates derived from history- and non-history-based affine candidates

- Non-adjacent constructed affine candidates

- Increased sub-block merge list size from 5 to 15

- Regular merge candidates derived from the first affine history table

Test 2.9b: Test 2.7a + the first type of constructed non-adjacent affine candidates from Test 2.8a + the inherited non-adjacent affine candidates with memory constraints from Test 2.8b.

The test contains the following aspects from Test 2.7 and Test 2.8:

- Two history tables for affine inheritance to derive affine merge candidates

- Pair-wise affine candidates derived from history- and non-history-based affine candidates

- Non-adjacent constructed and non-adjacent inherited affine candidates with memory constraint

- Increased sub-block merge list size from 5 to 15

- ARMC reordering size is increased from 15 to 45. The best 15 candidates are inserted into the affine merge candidate list, i.e., the size of the affine merge candidate list is kept unchanged.

- Regular merge candidates derived from the first affine history table

Memory requirements for 4K resolution sequences.

| Test | Memory |
| --- | --- |
| Test 2.7a | 0.8 KB |
| Test 2.7b | 12.5 KB |
| Test 2.8a | 1, 868 KB |
| Test 2.8b | 4.2 KB |
| Test 2.7c | 0.8 KB |
| Test 2.9b | 5 KB |

## Screen content coding

### Test 3.1: Cross-component palette coding

In cross-component palette coding, a cross-component look-up table is derived based on reconstructed neighboring samples by storing a luma sample quantized to 8 bits and a corresponding chroma sample value. The look-up table is derived from the top, left, top-left and bottom-right blocks in the current CTU and their extensions by 8 chroma reference lines in the neighboring CTUs.

For each chroma sample, the co-located luma sample value, denoted as Yc is fetched as a key to retrieve a mapping chroma sample value from the table. If Yc cannot be found, Yc ± 1, Yc ± 2 and Yc ± 3 are checked in order as keys to find the mapping chroma sample value. If none of the candidate keys are found, the mapping chroma sample value is set to be the average of the chroma values in the table.

In addition to LM down-sampling filter in ECM, five candidate down-sampling filters corresponding to down-sampling locations Y1, Y2…, Y5 as shown in Figure 10 are introduced to generate the best co-located luma sample value for the YCbCr 4:2:0 color format. The encoder tries all candidate down-sampling filters and signals an index per block of the best filter to the decoder.

Figure 10. Luma down-sampling positions

### Test 3.2: IBC with extended reference area

In this test, the reference area for IBC is extended to two CTU rows above. Figure 11 illustrates the reference area for coding CTU (m, n).

Figure 11. Reference area (green) for IBC when CTU (m, n) is coded

### Test 3.3: Enlarged HMVP table for IBC

In the test, the HMVP table size for IBC is increased to 25. After up to 20 IBC merge candidates are derived with full pruning, they are reordered together. After reordering, the first 6 candidates with the lowest template matching costs are selected as the final candidates in the IBC merge list.

### Test 3.4: IBC with template matching

In the test, the IBC-TM merge list has been modified compared to the one used by regular IBC merge mode such that the candidates are selected according to a pruning method with a motion distance between the candidates as in the regular TM merge mode. The zero motion candidates have been replaced by (-W, 0), (0, -H), (-W, -H) MVs.

In the IBC-TM merge mode, the selected candidates are refined with the template matching method, TM-merge flag is signaled to indicate the mode.

In the IBC-TM AMVP mode, up to 3 candidates are selected from the IBC-TM merge list. Each of those 3 selected candidates are refined using the template matching method and sorted according to their resulting TM cost.

TM refinement is performed at integer position, and in IBC-TM AMVP mode, it is performed either at integer or 4-pel precision depending on the AMVR value. The refinement is done within the existed IBC reference area.

### Test 3.5: BVP candidate adjustment based on IBC reference region

In Test 3.5a, clipping to the nearest IBC buffer boundaries is applied to one or both components BVP candidates pointing outside the IBC reference region before being used in the IBC Merge/AMVP list derivation.

In Test 3.5b, zero MV candidates in IBC merge and AMVP modes are replaced by 3 candidates are located on the nearest corners of the reference region, and 3 additional candidates are determined in the middle of the three sub-regions (A, B, and C), whose coordinates are determined by the width, and height of the current block and the ΔX and ΔY parameters, as is depicted in Figure 12.

Figure 12. Zero MV candidates replacement

Test 3.5c is a combination of Test 3.5a and 3.5b.

### Test 3.6: Combination of IBC tests

The following combinations were tested.

Test 3.6a: extended IBC area (Test 3.2) + enlarged IBC HMVP (Test 3.3)

Test 3.6b: extended IBC area (Test 3.2) + IBC TM (Test 3.4)

Test 3.6c: extended IBC area (Test 3.2) + BV clipping & zero BV replacement (Test 3.5c)

Test 3.6e: enlarged IBC HMVP (Test 3.3) + IBC TM (Test 3.4)

Test 3.6f: enlarged IBC HMVP (Test 3.3) + BV clipping & zero BV replacement (Test 3.5c)

Test 3.6g: BV clipping & zero BV replacement (Test 3.5c) + IBC TM (Test 3.4)

Test 3.6h: extended IBC area (Test 3.2) + enlarged IBC HMVP (Test 3.3) + IBC TM (Test 3.4)

Test 3.6i: extended IBC area (Test 3.2) + IBC TM (Test 3.4) + BV clipping & zero BV replacement (Test 3.5c)

Test 3.6j: extended IBC area (Test 3.2) + IBC TM (Test 3.4) + BV clipping & zero BV replacement (Test 3.5c)

Test 3.6k: enlarged IBC HMVP (Test 3.3) + IBC TM (Test 3.4) + BV clipping & zero BV replacement (Test 3.5c)

Test 3.6l: extended IBC area (Test 3.2) + enlarged IBC HMVP (Test 3.3) + IBC TM (Test 3.4) + BV clipping & zero BV replacement (Test 3.5c)

## Entropy coding

### Test 4.1: Improved probability estimation for CABAC

In test 4.1a, instead of using semi average of the two states p0 and p1, a weighted average is performed

p = (ω0 ∙ p0 + ω1 ∙ p1 )≫s,

where ω0 and ω1 are the weights selected from a pre-defined set {10,12,16,20,22}; s is the bitwise right-shift value, which is equal to 5 when (ω0 + ω1 ) ≤ 32 and 6 otherwise.

Three different sets of weights are pre-determined for each context model at I-, B- and P-slice types. It takes 6 bits to store the indices of two weights (3bits for each) for each context. There are 508 contexts in ECM-4.0 and three different sets of weights needed for I-, B- and P-slice types. Therefore, the total storage of maintaining the weights are 6bits × 3 × 508 = 1,143 bytes.

In test 4.1b, context initialization stored at previously coded picture after coding the last CTU can be used to initialize an inter slice having the same slice type and QP. For each inter slice, one control flag is signaled to select the context initialization scheme used for the slice. When the flag is equal to zero, it indicates that the context models of the slice are initialized using one of the existing context initialization tables (as indicated by the flag sh_cabac_init_flag). Otherwise, the context models of the slice are initialized by inheriting the ones from the previously coded picture.

### Test 4.2: CABAC initialization from previous inter slice and windows adjustment

In test 4.2a, context initialization stored at previously coded picture after coding the last CTU can be used to initialize an inter slice having the same slice type, QP, and temporal ID. The buffer size for storing previous initializations is set equal to 5 for each slice type, when the buffer is full the entry with the smallest QP and temporal ID is removed first before storing the initialization.

In Test 4.2b, short and long window sizes used in CABAC update are adjusted by two delta parameters stored in a look-up table per context and retrieved by a previous coded bin used as an index. The actual window sizes used to code the current bin after adjustment is (shift0+delta0) and (shift1+delta1), where shift0 and shift1 are existed predefined windows sizes stored in the context initialization tables. The window adjustment values take 4 bits each, so both values are stored in 1 byte and the total size of the LUT for 508 contexts is 1,016 bytes.

### Test 4.3: Combination of entropy coding tests

Test 4.3a is a combination of Test 4.1a (adaptive weights) and Test 4.2b (windows adjustment)

Test 4.3b is a combination of Test 4.1a (adaptive weights), Test 4.2b (windows adjustment), and Test 4.2a (temporal initialization)

# Test results

See the accompanied Excel table.

# EE related documents

| JVET-Z0048 | EE2-related: Modification of LFNST for MIP coded block |
| --- | --- |
| JVET-Z0059 | Non-EE2: Adaptive width for GPM blending area |
| JVET-Z0066 | EE2-related: Template matching using extended MVP candidate list |
| JVET-Z0067 | Non-EE2: RPR bug fixes and new results of RPR with luma-only re-scaling |
| JVET-Z0078 | Non-EE2: Support MMVD and Affine MMVD without template matching process |
| JVET-Z0079 | Non-EE2: Support RPR on ECM-4.0 and handling method for template matching based coding tools |
| JVET-Z0083 | Non-EE2: Fix on MHP parsing condition |
| JVET-Z0085 | Non-EE2: AmvpMerge without decoder side mv refinement process |
| JVET-Z0100 | Non-EE2: Weighted Chroma Prediction (WCP) |
| JVET-Z0102 | Non-EE2: Correction of the ARMC re-ordering step regarding the zero candidates |
| JVET-Z0103 | Non-EE2: On ARMC improvements |
| JVET-Z0123 | EE2-related: On chroma interpolation filters for motion compensation |
| JVET-Z0124 | Non-EE2: Spatial GPM |
| JVET-Z0125 | EE2-related: Regression based affine candidate derivation |
| JVET-Z0126 | Non-EE2: Improvement on Local Illumination Compensation |
| JVET-Z0127 | Non-EE2: On the maximum number of MHP merge candidates |
| JVET-Z0130 | EE2-related: Motion compensation boundary padding |
| JVET-Z0131 | EE2-related: Block Vector Difference Binarization |
| JVET-Z0137 | Non-EE2: Adaptive Blending for GPM |
| JVET-Z0142 | EE2-related: On MMVD and Affine MMVD Extension |
| JVET-Z0143 | Non-EE2: Chroma intra modes derived from collocated luma blocks and neighboring chroma blocks |
| JVET-Z0145 | EE2-2.5 related: More test results for ARMC with refined motion |
| JVET-Z0147 | Non-EE2: Improvement of Inter-MTS in ECM |
| JVET-Z0148 | EE2-2.5 related: AMVP with MVP refinement and ARMC with refined motion |
| JVET-Z0149 | Non-EE2: Adaptive Filter Shape Switch for ALF |
| JVET-Z0152 | EE2-related: IBC Merge Mode with Block Vector Differences |
| JVET-Z0157 | EE2-3.2-related: IBC Adaptation for Camera-Captured Contents |
| JVET-Z0159 | Non-EE2: Reconstruction-Reordered IBC for screen content coding |
| JVET-Z0184 | EE2-3.5-related: Additional results of zero-vector candidates replacement in the IBC Merge/AMVP list |

# AHG12 related documents

| JVET-Z0062 | AHG12: RPR luma filter modifications and RPR enabling fixes in ECM |
| --- | --- |
| JVET-Z0063 | AHG12: Fix for parsing of MHP information in ECM |
| JVET-Z0064 | AHG12: Convolutional cross-component model (CCCM) for intra prediction |
| JVET-Z0069 | AHG12: Longer chroma filters for RPR in ECM |
| JVET-Z0072 | AHG10/AHG12: Enhanced reference picture structures for ECM and VTM |
| JVET-Z0105 | AHG12: Virtual boundary processing for the new In-Loop filter tools in ECM |
| JVET-Z0140 | AHG12: Enhanced CCLM |
| JVET-Z0146 | AHG12: Using samples before deblocking filter for adaptive loop filter |