| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0047-v2 |
| --- | --- |

| Title: | AHG13: Improvements of film grain analysis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Edouard François, Zoubida Ameur (InterDigital), Miloš Radosavljević (Xiaomi) | Tel: Email: | {philippe.delagrange, edouard.francois,zoubida.ameur}@interdigital.com milosr@xiaomi.com |
| Source: | InterDigital, Xiaomi |  |  |

_____________________________

# Abstract

This contribution proposes improvements for film grain analysis part. It introduces improved film grain scaling factor estimation and introduces two new options in the VTM code. Those new options are related with the ability to use externally calculated masks of flat regions and externally calculated filtered frames. Current version only supports the use of MCTF and Canny edge detector in conjunction with morphological operations that are coded within VTM. Film grain synthesis part at the decoder side is not affected by this contribution.

# Proposed improvements

This contribution proposes an enhanced VTM software implementation focusing on the film grain analysis and parameter estimation module at the encoder side. It further enhances the implementation presented in JVET-W0072. More precisely, this contribution describes a method to automatically estimate intensity intervals and associated scaling factors used in the film grain synthesis process with higher accuracy.

Film grain was studied in JVET within JVET-V0093, further improved in JVET-W0072 and JVET-W0095, and later-on studied within core experiment on film grain synthesis (JVET-W2022) during JVET-W and JVET-X meeting and within AHG13. Since VTM-15.1, film grain analysis and synthesis modules are part of the reference software.

# Code changes

Two new options are added in the configuration parameters, to be used in VTM within film grain analysis part. They are:

--SEIFGCExternalDenoised=path/denoised_sequence_name.yuv

--SEIFGCExternalMask= path/mask_sequence_name.yuv

These new options provide a means to use any denoising algorithm and any mask (for flat blocks detection) calculation algorithm. If those parameters are provided, VTM will use external sources for film grain analysis part. If the parameter is not provided (empty string by default) VTM will use MCTF and Canny edge detector in conjunction with morphological operations (MCTF and Canny for film grain analysis are already part of VTM). We think those two additional options could be useful for further film grain analysis development and experimentation.

One bug-fix relates to the number of intensity intervals. In analysis part (encoder side) number of estimated intensity intervals was limited to 10. However, such limitation should not exist for number of intensity intervals. It only exists for the number of cut-off frequency pairs.

The use of morphological operations in the mask derivation process was also adjusted to get more rigid mask derivation for flat blocks which leads to less biased data points. By doing so, we extended the influence of edges to the wider image region, and we avoided the influence of edges to be included in film grain estimation process.

# Film Grain Analysis

Since film grain is dependent on the local characteristics of an image, different scalings of the film grain can be applied for different intensities or intensity intervals of an input image in order to get appropriate intensity of the film grain before finally blending it to the input image. After creating a film grain pattern in accordance with frequency filtering model, scaling to the proper intensity based on the scaling factor (SF) is performed. It determines the level at which the film grain will be perceived at the final image, and by doing that we ensure that the film grain is simulated at the correct scale.

Currently, film grain SEI message as defined in VSEI and implemented in VTM comprises the following parameters that define film grain scaling function which is applied at the decoder/synthesis side:

- fg_intensity_interval_upper_bound[ c ][ i ] less than fg_intensity_interval_lower_bound[ c ][ i + 1 ]

- fg_comp_model_value[ c ][ i ][ 0 ],

where fg_comp_model_value[ c ][ i ][ 0 ] represents the scaling factor, i is index of the interval, and c is color component. Given representation leads to the piecewise constant scaling function (stepwise scaling function). One example of such scaling function is given in Fig. 1.

Figure 1: An example of film grain scaling function for 10-bit video

Note that each step of a given stepwise function represents one intensity interval, defined with its bounds (lower and upper bound) and scaling factor value. In JVET-W0072 we proposed a method to automatically estimate intervals and associated scaling factor given the input image, filtered image, and mask that indicates flat region of an image. This contribution uses the same approach as previous one, with some further improvements mainly within curve fitting and data-point regularization and filtering.

# Proposed improved solution to estimate intensity intervals and scaling parameters

Almost identical approach as in previous version to get data-points for scaling factor estimation is used. It iterates through the image on 8x8, 16x16 or 32x32 block size granularity to collect data points. The block size for scaling factor estimation is aligned with synthesis process that uses the block size (during film grain blending) depending on input sequence resolution.

The rest of the algorithm is better adjusted for a higher performance. The different main steps are described below.

Step1 – Data points derivation

Initial Data points derivation

First, filtered frame and mask are created in a pre-processing step. The film grain image is then created by finding the difference between original and filtered frame.

Once the film grain image is received, along with the mask and filtered frame, both film grain image and filtered image are scanned block by block at 8x8 or 16x16 or 32x32 granularity (depending on frame size). Only the blocks at the flat parts of the image are processed further. If the mask indicates a flat region, the observed blocks are going to the next step – film grain parameter estimation.

Thereafter, to estimate intensity intervals and associated scaling factors, first step would be to calculate indicative features for each block that enters the process of film grain parameter estimation. The first feature is the average value (avg) calculated on a block within the filtered frame. The second feature is the variance of the film grain block (var), and it is calculated on a block from the film grain image (recall film grain image is the difference between the original and the filtered frame). Those two features are the main features used in estimating intensity intervals and scaling factors. They define data points (avg, var) that are used in further analysis (see Fig. 2).

Data points regularization

Next step is data points regularization, used to regularize high variations and to control the excessively fluctuating points. We propose to use the following regularization function:

double k=3.0;

double m=.5;

double tmp = k * pow((double)(var), m) + .5;

var = (int)tmp;

Coefficient m defines regularization of dispersity/heteroscedasticity of data points.

Coefficient k defines the level of film grain to be added back, it should be aligned to the level of the film grain being removed from the source (during the filtering and quantization).

Discarding potential outliers

After regularization is applied, the data points higher than 16 << (bitDepth - 8) are discarded from the calculation to limit data points to meaningful values. If var is higher than the maximum defined value, most likely variance is biased by edges and other low frequency components removed during the filtering (edges or fine details).

Step2 – Curve fitting

Sub-range averaging

Further process is illustrated on Fig. 2 (on Fig. 2 data points are already regularized and limited to max value). Complete intensity dynamic range (horizontal axis) is divided (uniformly quantized) into non-overlapping intervals of size 16.

For each intensity sub-range we calculate the average value of the variance, but only if minimum number of data points within the sub-interval is Nmin> 8.

Discarding potential outliers (1st pass)

If there is single point without any neighbouring points, it is considered as an estimation error and it is removed. Single points are filtered out (single non-zero interval).

Extreme points extrapolation

A step of extension of points to left and to the right towards the zero-variance is then applied to smooth transition from intensities with film grain to the intensities without film grain. The process finds the most left and most right point and extends the data-point range. At most 4 new points are added to the left and 4 new points to the right. It is particularly visible on left part of Fig. 2 (note red or blue points added even on the intensities that are not covered by the initial data points. Initial data points are plotted with black circles).

Also remove points if intensity (horizontal axis) is less then 40 or larger than 950 (for 10-bit signal; these values are shifted for bitdepths other than 10). Indeed, film grain is usually not added to the very dark and very bright regions.

Curve fitting (1st pass)

Next step is to fit the parametrized curve by using 4rd order polynomial fitting (red curve on Fig. 2).

Discarding potential outliers (2nd pass)

+0.6*std and -1.2*std around the first fitted curve is used to filter out the points outside the given range. This step filters out remaining outliers and biased variance points.

Curve fitting (2nd pass)

The remaining points are entering second pass of curve fitting. We repeat previous process and again fit the polynomial curve, but this time using reduced set of data-points (blue curve on Fig. 2).

Step3 – Final scaling function approximation

Final step approximates the scaling function represented by the 4rd order polynomial curve by a simplified stepwise scaling function. The stepwise function is derived by using Lloyd-max non-uniform quantization with 4 quantization levels. Quantizer is adapted (trained) for each new set of points on-fly. See blue quantized scaling function on Fig. 2 which represents the final scaling function.

Thus, the data points are used to fit the function/curve, and intensity interval and scaling factor estimation is made of 2 steps:

- derive a scaling function by curve fitting

- approximate the scaling function by a simplified scaling function, in our case stepwise function

It leads to several intensity intervals and scaling factors as it is illustrated in Fig. 2 (blue curve and quantized blue curve are final solution).

Figure 2: Data-points and 2-pass curve fitting and quantization (black circles: input data points, red: non-filtered fitting curve, blue: filtered fitting curve).

By analyzing quantized curve, we can easily obtain number of intensity intervals, bounds of intervals (fg_intensity_interval_lower_bound[ c ][ i ] and fg_intensity_interval_upper_bound[ c ][ i ]) and scaling factor (fg_comp_model_value[ c ][ i ][ 0 ]) associated with each estimated interval. Note that in the given example, even if there are 4 quantization levels, quantization of the curve leads to more intensity intervals. Those intervals and scaling factors are embedded in the SEI message and transmitted to the decoder.

Note, the proposed changes only affect encoder side.

# Software usage

The software is available within the proposal package (file source_FG_analysis_improvts.zip). Proposed changes are implemented on top of the VTM-15.2.

Several improvements are incorporated in the new version of software. However, software usage mainly remains unchanged given the previous implementation.

To enable the proposed film grain analysis improvements, macro FG_IMPROVEMENT is set to 1. Thereafter, we suggest using film_grain_characteristics_test_analysis.cfg file to provide the encoder with additional parameters.

# Additional observations

Currently, MCTF and Canny edge detector give good results when used in film grain scaling factor estimation. Coefficients k and m in regularization function are adjusted for this scenario. We also tested software with other external masks and denoisers.

We used flat block finder, same as in av1 (A. Kokaram, D. Kelly, H. Denman and A. Crawford, "Measuring noise correlation for improved video denoising," 2012 19th, ICIP). It gives similar performance as Canny+morphology.

We tested different denoisers, ffmpeg hqdn3d=6:6:6:6, reconstructed VTM frame with QP=22 and Qp=26. We observed, compared to MCTF, that those denoisers estimate higher amplitude of scaling factors, however with similar intensity intervals. Obviously, as denoising is stronger in those cases, more noise is removed, and consequently more noise is estimated (leading to higher scaling factor amplitudes). Hence, estimated scaling factor depends on how much noise we removed at the first place. The fact that different denoisers might be used (including reconstructed frames), coefficients in regularization function can be adopted based on the strength of filtering. It is left for further study.

# Conclusion

Presented contribution improves the film grain scaling factor estimation, and greatly improves visual quality of the synthesized film grain by using automatically estimated model parameters. We propose to update current software (VTM and HM reference software) with proposed modifications (film grain analysis and synthesis are already part of the reference codes).

# Acknowledgment

# This work has been partially achieved in the context of the 3EMS project funded by the “Region Bretagne”. We thank to Dolby for cross-check.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xiaomi does not have any current or pending patent rights relating to the technology described in this contribution.