| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0049-v1 |
| --- | --- |

| Title: | EE2-1.1: Slope adjustment for CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Alireza Aminlou Pekka Astola Ramin G. Youvalari | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results of EE2 Test 1.1 on adding a slope adjustment to cross-component linear model (CCLM) prediction. The adjustment is tilting or rotating the linear function which maps luma values to chroma values with respect to a center point determined by the average luma value of the reference samples as proposed originally in JVET-Y0055. The impact in ECM 4.0 is reportedly {for Y, U, V, EncT, DecT}:

AI { -0.04%, -1.16%, -1.07%, 101%, 100%}, RA {-0.03%, -0.51%, -0.44%, 100%, 100%},

LDB {-0.03%, -0.28%, -0.04%, 100%, 100%}, LDP {-0.07%, -0.15%, -0.25%, 100%, 99%}

The impact is reportedly higher for the TGM categories, with -0.19%, -0.84%, -0.67% impact in AI-TGM and -0.15%, -0.29% and -0.23% impact in RA-TGM (for Y, U and V, respectively).

# Studied approach

Basic principle

CCLM uses a model with 2 parameters to map luma values to chroma values. The slope parameter “a” and the bias parameter “b” define the mapping as follows:

chromaVal = a * lumaVal + b

It is proposed signal an adjustment “u” to the slope parameter to update the model to the following form:

chromaVal = a’ * lumaVal + b’

where

a’ = a + u

b’ = b - u * yr

With this selection the mapping function is tilted or rotated around the point with luminance value yr. It is proposed to use the average of the reference luma samples used in the model creation as yr in order to provide a meaningful modification to the model. Picture below illustrates the process.

Figure 1: Illustration of the effect of the slope adjustment parameter “u”. Left: model created with the current CCLM. Right: model updated as proposed.

Implementation

Slope adjustment parameter is provided as an integer between -4 and 4, inclusive, and signaled in the bitstream. The unit of the slope adjustment parameter is 1/8th of a chroma sample value per one luma sample value (for 10-bit content).

Adjustment is available for the CCLM models that are using reference samples both above and left of the block (“LM_CHROMA_IDX” and “MMLM_CHROMA_IDX”), but not for the “single side” modes. This selection is based on coding efficiency vs. complexity trade-off considerations.

When slope adjustment is applied for a multimode CCLM model, both models can be adjusted and thus up to two slope updates are signaled for a single chroma block.

Encoder approach

The proposed encoder approach performs an SATD based search for the best value of the slope update for Cr and a similar SATD based search for Cb. If either one results as a non-zero slope adjustment parameter, the combined slope adjustment pair (SATD based update for Cr, SATD based update for Cb) is included in the list of RD checks for the TU.

# Experimental results

Coding efficiency impact is reported in the tables below following the JVET common test conditions.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03 % | -1.74 % | -1.73 % | 100 % | 100 % |
| Class A2 | -0.05 % | -1.25 % | -0.61 % | 101 % | 100 % |
| Class B | -0.04 % | -1.47 % | -1.60 % | 101 % | 100 % |
| Class C | -0.05 % | -0.96 % | -0.89 % | 101 % | 100 % |
| Class E | -0.03 % | -0.23 % | -0.21 % | 101 % | 100 % |
| Overall | -0.04 % | -1.16 % | -1.07 % | 101 % | 100 % |
| Class D | -0.04 % | -0.68 % | -0.71 % | 101 % | 99 % |
| Class F | -0.08 % | -0.69 % | -1.06 % | 99 % | 100 % |
| ClassTGM | -0.19 % | -0.84 % | -0.67 % | 98 % | 100 % |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02 % | -0.47 % | -0.73 % | 99 % | 100 % |
| Class A2 | -0.02 % | -0.63 % | -0.28 % | 98 % | 100 % |
| Class B | -0.03 % | -0.72 % | -0.61 % | 101 % | 101 % |
| Class C | -0.04 % | -0.20 % | -0.14 % | 102 % | 100 % |
| Class E |  |  |  |  |  |
| Overall | -0.03 % | -0.51 % | -0.44 % | 100 % | 100 % |
| Class D | 0.02 % | -0.42 % | -0.55 % | 101 % | 101 % |
| Class F | 0.01 % | -0.54 % | -0.58 % | 102 % | 101 % |
| ClassTGM | -0.15 % | -0.29 % | -0.23 % | 98 % | 99 % |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01 % | -0.47 % | -0.43 % | 99 % | 100 % |
| Class C | 0.02 % | -0.15 % | -0.26 % | 100 % | 100 % |
| Class E | -0.18 % | -0.16 % | 0.89 % | 101 % | 99 % |
| Overall | -0.03 % | -0.28 % | -0.04 % | 100 % | 100 % |
| Class D | -0.07 % | 0.34 % | 0.71 % | 101 % | 99 % |
| Class F | -0.20 % | 0.18 % | -0.57 % | 100 % | 99 % |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03 % | -0.01 % | -0.21 % | 100 % | 99 % |
| Class C | 0.00 % | -0.38 % | -0.34 % | 100 % | 99 % |
| Class E | -0.21 % | -0.10 % | -0.22 % | 100 % | 99 % |
| Overall | -0.07 % | -0.15 % | -0.25 % | 100 % | 99 % |
| Class D | -0.06 % | -0.31 % | 0.52 % | 101 % | 99 % |
| Class F | -0.10 % | -0.04 % | -0.46 % | 100 % | 100 % |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).