| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0050-v1 |
| --- | --- |

| Title: | EE2-1.3: Combined intra prediction tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Alireza Aminlou Pekka Astola Ramin G. Youvalari Xinwei Li Ru-Ling Liao Jie Chen Yan Ye | Email: | firstname.lastname@nokia.com {sid.lxw, ruling.lrl, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Nokia, Alibaba group |  |  |

_____________________________

# Abstract

This contribution reports results of the two combined intra prediction Tests 1.3a and 1.3b in EE2. Test 1.3a is a combination of Test 1.1 (slope adjustment for CCLM) and Test 1.2c (combination of Test 1.2a on DIMD chroma mode and Test 1.2b on fusion of chroma intra prediction modes). Test 1.3b represents a combination of Test 1.1 and Test 1.2d. Test 1.2d is asserted to be a lower complexity version of Test 1.2c, as the DIMD chroma mode is derived in Test 1.2d using only neighboring reconstructed samples. The impacts of the two combined tests are reported to be {for Y, U, V, EncT, DecT}:

- Test 1.3a (combination of Tests 1.1 and 1.2c):

AI {-0.13%, -2.28%, -2.12%, 103%, 101%}, RA {-0.05%, -1.07%, -0.96%, 102%, 100%}

- Test 1.3b (combination of Tests 1.1 and 1.2d with reduced processing requirements):

AI {-0.12 %, -2.13%, -2.03%, 103%, 100%}, RA {-0.05%, -1.02%, -0.89%, 101%, 100%}

# Studied approaches

Details of the studied technologies can be found in the reports provided for Test 1.1 (JVET-Z0049) and Test 1.2 (JVET-Z0051).

# Experimental results

Coding efficiency impact for the two combination tests implemented on top of ECM 4.0 [1] is reported in the tables below following the JVET common test conditions [2].

## Test 1.3a

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12 % | -2.58 % | -2.50 % | 103 % | 102 % |
| Class A2 | -0.13 % | -2.38 % | -1.74 % | 106 % | 102 % |
| Class B | -0.13 % | -2.45 % | -2.62 % | 102 % | 101 % |
| Class C | -0.15 % | -1.81 % | -1.75 % | 103 % | 100 % |
| Class E | -0.13 % | -2.24 % | -1.77 % | 102 % | 101 % |
| Overall | -0.13 % | -2.28 % | -2.12 % | 103 % | 101 % |
| Class D | -0.11 % | -1.47 % | -1.39 % | 103 % | 100 % |
| Class F | -0.29 % | -1.47 % | -1.67 % | 103 % | 101 % |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05 % | -0.98 % | -0.83 % | 103 % | 100 % |
| Class A2 | -0.06 % | -1.07 % | -0.68 % | 102 % | 100 % |
| Class B | -0.04 % | -1.36 % | -1.47 % | 101 % | 100 % |
| Class C | -0.05 % | -0.78 % | -0.64 % | 100 % | 100 % |
| Class E |  |  |  |  |  |
| Overall | -0.05 % | -1.07 % | -0.96 % | 102 % | 100 % |
| Class D | 0.01 % | -0.88 % | -0.84 % | 100 % | 100 % |
| Class F | -0.09 % | -0.81 % | -0.82 % | 101 % | 100 % |

## Test 1.3b

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10 % | -2.49 % | -2.33 % | 103 % | 101 % |
| Class A2 | -0.10 % | -2.21 % | -1.54 % | 106 % | 101 % |
| Class B | -0.13 % | -2.29 % | -2.52 % | 102 % | 100 % |
| Class C | -0.13 % | -1.78 % | -1.81 % | 103 % | 99 % |
| Class E | -0.11 % | -1.91 % | -1.73 % | 102 % | 99 % |
| Overall | -0.12 % | -2.13 % | -2.03 % | 103 % | 100 % |
| Class D | -0.11 % | -1.20 % | -1.32 % | 103 % | 99 % |
| Class F | -0.22 % | -1.27 % | -1.61 % | 103 % | 101 % |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.98% | -0.83% | 102% | 100% |
| Class A2 | -0.05% | -1.02% | -0.64% | 102% | 100% |
| Class B | -0.03% | -1.37% | -1.36% | 101% | 100% |
| Class C | -0.08% | -0.62% | -0.54% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.05% | -1.02% | -0.89% | 101% | 100% |
| Class D | -0.03% | -0.93% | -0.89% | 100% | 100% |
| Class F | -0.05% | -0.97% | -0.76% | 101% | 100% |

# References

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0

- M. Karczewicz, et. al., “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, February 2022.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).