| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0053-v1 |
| --- | --- |

| Title: | 26th Meeting, by teleconference, 20–29 April 2022 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Wien, RWTH Aachen University | Tel: Email: | +49-241-80-27867 wien@lfb.rwth-aachen.de |
| Source: | AhG |  |  |

_____________________________

# Abstract

Remote expert viewing tests were conducted before the 26th JVET meeting for the exploration activity on EE1/DNN-based coding tools and for studying new gaming-type test material proposed in JVET-Y0041. The tests were performed using mp4 files provided by the proponents with a total of 68 A-B-comparisons for EE1 and 14 comparisons for the gaming-type content. Sequences of 720p, 1080p, and 2160p resolution were evaluated for EE1. For the gaming-type content, 1080p and 2160p were evaluated. Calls for participation in the subjective viewing were issued on the JVET reflector. Overall, 18 experts volunteered in participating in the tests. The testing procedure followed the draft SC29/AG5 guidelines for remote experts viewing, providing an A/B comparison of sequences under test.

The number of participants was only 18 which was indicated as the lowest number to be accepted for running the tests. Due to the uncontrolled and variable viewing conditions the resulting data may be of limited reliability and should be treated with care.

Based on the reported MOS values, the following observations are made: For the EE1-2 experiment, a significant visual benefit is reported for 14 out of 24 test cases. For the remaining cases, MOS = 0 is included in the confidence interval such that comparable quality is indicated. In the EE1-1.7 experiment, 9 test cases out of indicate a significant visual benefit while one case showed a significant loss. The remaining 14 cases indicate comparable quality. In the super-resolution category (EE1-2.x), visual benefit is observed for three cases for the NN-based method (2.4) and one for the RPR-based approach. The other cases indicate no benefit by the confidence interval with two cases for the RPR-based approach which show a tendency towards loss.

It is recommended to determine a sufficiently large group of volunteers well before the planned test date in order to avoid difficulties in acquisition at test time. This may be done by the end of the current meeting or (latest) by the AhG meeting in preparation of the viewing tests. The test preparation would be completed if the requested number of participants is reached.

# Introduction

Remote expert viewing tests were conducted before the 26th JVET meeting for the exploration activity on EE1/DNN-based coding tools [2] and for evaluation of new gaming-type content [3]. The tests were performed using mp4 files provided by the proponents with a total of 68 A-B-comparisons for EE1 and 14 comparisons for the gaming-type content. Sequences of 720p, 1080p, and 2160p resolution were evaluated for EE1. For the gaming-type content, 1080p and 2160p were evaluated. Calls for participation in the subjective viewing were issued on the JVET reflector. Overall, 18 experts volunteered in participating in the tests. The testing procedure followed the draft SC29/AG5 guidelines for remote experts viewing, providing an A/B comparison of sequences under test [1]. Tests were conducted on Thu, 2022-03-31 (4 participants) and Fri, 2022-04-01 (14 participants).

# Test arrangement

Sequences of 5s duration were used for evaluation. For all but the Campfire sequence, the first 5s were assessed. For the Tango sequence, the duration is slightly below 5s.

The following EE1 tests were suggested to be included into viewing:

Table 1: EE1 tests suggested for the viewing

| EE1 Test | Proposal | Viewing categories |
| --- | --- | --- |
| 1.2 | JVET-Y0080 | UHD, 1080p, 720p |
| 1.7 | JVET-Y0143 JVET-Y0098 JVET-Y0110 | UHD, 1080p, 720p |
| 2.1 | JVET-Y0068 | UHD |
| 2.4 | JVET-Y0068 JVET-Y0070 | UHD |

The sequence compared in the EE1 category are listed below:

Table 2: EE1 Pairs of mp4 files assessed in the visual test

| anchor/ArenaOfValor_QP34.mp4 | EE1-1.2/ArenaOfValor_QP34.mp4 |
| --- | --- |
| anchor/ArenaOfValor_QP39.mp4 | EE1-1.2/ArenaOfValor_QP39.mp4 |
| anchor/BQTerrace_QP34.mp4 | EE1-1.2/BQTerrace_QP34.mp4 |
| anchor/BQTerrace_QP39.mp4 | EE1-1.2/BQTerrace_QP39.mp4 |
| anchor/Campfire_QP34.mp4 | EE1-1.2/Campfire_QP34.mp4 |
| anchor/Campfire_QP37.mp4 | EE1-1.2/Campfire_QP37.mp4 |
| anchor/CatRobot_QP39.mp4 | EE1-1.2/CatRobot_QP39.mp4 |
| anchor/CatRobot_QP42.mp4 | EE1-1.2/CatRobot_QP42.mp4 |
| anchor/DaylightRoad2_QP39.mp4 | EE1-1.2/DaylightRoad2_QP39.mp4 |
| anchor/DaylightRoad2_QP42.mp4 | EE1-1.2/DaylightRoad2_QP42.mp4 |
| anchor/FourPeople_QP34.mp4 | EE1-1.2/FourPeople_QP34.mp4 |
| anchor/FourPeople_QP39.mp4 | EE1-1.2/FourPeople_QP39.mp4 |
| anchor/Johnny_QP34.mp4 | EE1-1.2/Johnny_QP34.mp4 |
| anchor/Johnny_QP39.mp4 | EE1-1.2/Johnny_QP39.mp4 |
| anchor/Kimono1_QP34.mp4 | EE1-1.2/Kimono1_QP34.mp4 |
| anchor/Kimono1_QP39.mp4 | EE1-1.2/Kimono1_QP39.mp4 |
| anchor/KristenAndSara_QP34.mp4 | EE1-1.2/KristenAndSara_QP34.mp4 |
| anchor/KristenAndSara_QP39.mp4 | EE1-1.2/KristenAndSara_QP39.mp4 |
| anchor/redkayak_QP34.mp4 | EE1-1.2/redkayak_QP34.mp4 |
| anchor/redkayak_QP39.mp4 | EE1-1.2/redkayak_QP39.mp4 |
| anchor/Spincalendar_QP34.mp4 | EE1-1.2/Spincalendar_QP34.mp4 |
| anchor/Spincalendar_QP39.mp4 | EE1-1.2/Spincalendar_QP39.mp4 |
| anchor/Tango2_QP39.mp4 | EE1-1.2/Tango2_QP39.mp4 |
| anchor/Tango2_QP42.mp4 | EE1-1.2/Tango2_QP42.mp4 |
| anchor/ArenaOfValor_QP34.mp4 | EE1-1.7/ArenaOfValor_QP34.mp4 |
| anchor/ArenaOfValor_QP39.mp4 | EE1-1.7/ArenaOfValor_QP39.mp4 |
| anchor/BQTerrace_QP34.mp4 | EE1-1.7/BQTerrace_QP34.mp4 |
| anchor/BQTerrace_QP39.mp4 | EE1-1.7/BQTerrace_QP39.mp4 |
| anchor/Campfire_QP34.mp4 | EE1-1.7/Campfire_QP34.mp4 |
| anchor/Campfire_QP37.mp4 | EE1-1.7/Campfire_QP37.mp4 |
| anchor/CatRobot_QP39.mp4 | EE1-1.7/CatRobot_QP39.mp4 |
| anchor/CatRobot_QP42.mp4 | EE1-1.7/CatRobot_QP42.mp4 |
| anchor/DaylightRoad2_QP39.mp4 | EE1-1.7/DaylightRoad2_QP39.mp4 |
| anchor/DaylightRoad2_QP42.mp4 | EE1-1.7/DaylightRoad2_QP42.mp4 |
| anchor/FourPeople_QP34.mp4 | EE1-1.7/FourPeople_QP34.mp4 |
| anchor/FourPeople_QP39.mp4 | EE1-1.7/FourPeople_QP39.mp4 |
| anchor/Johnny_QP34.mp4 | EE1-1.7/Johnny_QP34.mp4 |
| anchor/Johnny_QP39.mp4 | EE1-1.7/Johnny_QP39.mp4 |
| anchor/Kimono1_QP34.mp4 | EE1-1.7/Kimono1_QP34.mp4 |
| anchor/Kimono1_QP39.mp4 | EE1-1.7/Kimono1_QP39.mp4 |
| anchor/KristenAndSara_QP34.mp4 | EE1-1.7/KristenAndSara_QP34.mp4 |
| anchor/KristenAndSara_QP39.mp4 | EE1-1.7/KristenAndSara_QP39.mp4 |
| anchor/redkayak_QP34.mp4 | EE1-1.7/redkayak_QP34.mp4 |
| anchor/redkayak_QP39.mp4 | EE1-1.7/redkayak_QP39.mp4 |
| anchor/Spincalendar_QP34.mp4 | EE1-1.7/Spincalendar_QP34.mp4 |
| anchor/Spincalendar_QP39.mp4 | EE1-1.7/Spincalendar_QP39.mp4 |
| anchor/Tango2_QP39.mp4 | EE1-1.7/Tango2_QP39.mp4 |
| anchor/Tango2_QP42.mp4 | EE1-1.7/Tango2_QP42.mp4 |
| anchor/Campfire_QP34.mp4 | EE1-2.1/Campfire_QP34.mp4 |
| anchor/Campfire_QP37.mp4 | EE1-2.1/Campfire_QP37.mp4 |
| anchor/CatRobot_QP39.mp4 | EE1-2.1/CatRobot_QP39.mp4 |
| anchor/CatRobot_QP42.mp4 | EE1-2.1/CatRobot_QP42.mp4 |
| anchor/DaylightRoad2_QP39.mp4 | EE1-2.1/DaylightRoad2_QP39.mp4 |
| anchor/DaylightRoad2_QP42.mp4 | EE1-2.1/DaylightRoad2_QP42.mp4 |
| anchor/Tango2_QP39.mp4 | EE1-2.1/Tango2_QP39.mp4 |
| anchor/Tango2_QP42.mp4 | EE1-2.1/Tango2_QP42.mp4 |
| anchor/Campfire_QP34.mp4 | EE1-2.4/Campfire_QP34.mp4 |
| anchor/Campfire_QP37.mp4 | EE1-2.4/Campfire_QP37.mp4 |
| anchor/CatRobot_QP39.mp4 | EE1-2.4/CatRobot_QP39.mp4 |
| anchor/CatRobot_QP42.mp4 | EE1-2.4/CatRobot_QP42.mp4 |
| anchor/DaylightRoad2_QP39.mp4 | EE1-2.4/DaylightRoad2_QP39.mp4 |
| anchor/DaylightRoad2_QP42.mp4 | EE1-2.4/DaylightRoad2_QP42.mp4 |
| anchor/Tango2_QP39.mp4 | EE1-2.4/Tango2_QP39.mp4 |
| anchor/Tango2_QP42.mp4 | EE1-2.4/Tango2_QP42.mp4 |

For the new gaming-type sequences of JVET-Y0041 [3] the compared mp4 files are listed below. For theses sequences, the sequences were coded using comparable configurations for HM and VTM and QPs providing comparable bitrates.

Table 3: Pairs of mp4 files for JVET-Y0041assessed in the visual test

| HM/Darktree_1920x1080_60fps_10bit_420_QP33.mp4 | VTM/Darktree_1920x1080_60fps_10bit_420_QP32.mp4 |
| --- | --- |
| HM/Darktree_1920x1080_60fps_10bit_420_QP38.mp4 | VTM/Darktree_1920x1080_60fps_10bit_420_QP37.mp4 |
| HM/Darktree_1920x1080_60fps_10bit_420_QP43.mp4 | VTM/Darktree_1920x1080_60fps_10bit_420_QP42.mp4 |
| HM/FontainebleauCinematic_3840x2160_60fps_10bit_420_QP33 | VTM/FontainebleauCinematic_3840x2160_60fps_10bit_420_QP32 |
| HM/FontainebleauCinematic_3840x2160_60fps_10bit_420_QP39 | VTM/FontainebleauCinematic_3840x2160_60fps_10bit_420_QP37 |
| HM/FontainebleauCinematic_3840x2160_60fps_10bit_420_QP44 | VTM/FontainebleauCinematic_3840x2160_60fps_10bit_420_QP42 |
| HM/FontainebleauCinematicS_1920x1080_60fps_10bit_420_QP33 | VTM/FontainebleauCinematicS_1920x1080_60fps_10bit_420_QP32 |
| HM/FontainebleauCinematicS_1920x1080_60fps_10bit_420_QP38 | VTM/FontainebleauCinematicS_1920x1080_60fps_10bit_420_QP37 |
| HM/FontainebleauCinematicS_1920x1080_60fps_10bit_420_QP44 | VTM/FontainebleauCinematicS_1920x1080_60fps_10bit_420_QP42 |
| HM/FontainebleauFPV_1920x1080_60fps_10bit_420_QP33.mp4 | VTM/FontainebleauFPV_1920x1080_60fps_10bit_420_QP32.mp4 |
| HM/FontainebleauFPV_1920x1080_60fps_10bit_420_QP38.mp4 | VTM/FontainebleauFPV_1920x1080_60fps_10bit_420_QP37.mp4 |
| HM/FontainebleauFPV_1920x1080_60fps_10bit_420_QP43.mp4 | VTM/FontainebleauFPV_1920x1080_60fps_10bit_420_QP42.mp4 |
| HM/Racing_3840x2160_50fps_420_10bit_QP40.mp4 | VTM/Racing_3840x2160_50fps_420_10bit_QP37.mp4 |
| HM/Racing_3840x2160_50fps_420_10bit_QP45.mp4 | VTM/Racing_3840x2160_50fps_420_10bit_QP42.mp4 |

# Test procedure

## Test session setup

The viewing compared the reconstruction quality of the proposals under test to the reconstruction quality of the VTM anchor. It was held as an online session via Zoom with the participants having downloaded the (anonymized) mp4 files beforehand in a password protected zip file. The password was provided to the participants before the start of the viewing.

In the following, video A and video B denote either the anchor or the proposal from the pairs of mp4 files listed in Table 2 and Table 3. The assignment of anchor and proposal to A or B was randomized and not revealed to the test subjects. The set of tests was split into six viewing sessions (21 votes each for 2× UHD, 1080p, and 720p for EE1, and for JVET-Y0041 13 votes for UHD and 9 votes for 1080p).

Each basic test cell (BTC) was constructed as follows:

- “Original” (on a mid-gray background, 1 s presentation)

- Original Video

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “A” (on a mid-gray background, 1 s presentation)

- Video A

- “B” (on a mid-gray background, 1 s presentation)

- Video B

- “Vote N” (on a mid-gray background, 5 s presentation)

The test subjects were asked to rate the video sequences presented in each test cell as follows:

- “A was much better than B”

- “A was better than B”

- “B was better than A”

- “B was much better than A”

The used voting sheet is presented in the figure below. The participants were supposed to fill the voting sheet and enter their results into an online form after completion of the session. The participants were also asked to enter the used screen size and any specific observations in a comment field of the online form.

The voting sheets were used in order to simplify the workload for the participants during the session.

Figure 1: Voting sheet

Each viewing session contained a stabilization phase of BTCs to be discarded and an additional test point comparing the original sequence to a coded version.

The viewers were advised to view the content at 1.5×height of the UHD screen and configure VLC not to scale the presented content, such that the distance matched the content resolution.

Before the start of the test sessions, the viewers were provided a demo playlist in order to familiarize themselves with the procedure of the test. The viewers were verbally instructed about the procedure and the rating scale in the introduction of the session. They were asked not to stop the video during the test sessions but to use the assigned voting time to record their judgement.

The test sessions were run in the order of decreasing length, i.e. EE1 (order 720p, 1080p, 2160p) first and then JVET-Y0041 (order 1080p, 2160p).

## Voting scores

For evaluation, the votes of the test subjects were mapped to MOS numbers as follows:

- +3	: “Proposal much better than anchor”

- +1	: “Proposal better than anchor”

- -1	: “Anchor better than proposal”

- -3	: “Anchor much better than proposal”

From these values, the MOS and the 95% confidence intervals were calculated. A resulting MOS value of zero indicates that anchor and proposal were rated to have the same visual quality. The scores of the four participants with the lowest Pearson correlation were removed from the evaluation.

## Participants comments

Some participants raised the fact that sequence pairs might look very similar. The overall duration of the experiment was mentioned. The following specific comments were recorded.

DNN:

- (720p) FourPeople and Spin Calendar quality is too low, better to use QP-1 next time.

- (720p) NN looks a bit blurry and anchor is more sharp. NN sometimes give artificial edges.

- (1080p) BQTerrace quality too high.

- (1080p) NN can give artificial edges and colors.

- (2160p) Tango quality too high.

JVET-Y0041:

- (1080p) Motion too fast for viewing (esp. Darktree)

# Test results

The MOS values and the corresponding confidence intervals (CI) resulting from the evaluation are reported in Table 4. According to the assignment of voting scores, a positive number indicates a visual benefit for the proposal over the anchor while a negative number indicates the proposal to be inferior to the anchor.

The tests are marked in the table. The ‘check’ test represents a test cell where the assessment of the original against a coded version or the assessment of high and low QP for the same source have been evaluated. The existence or location of these cells were not revealed to the participants before the experiments.

The scores of the 18 participants have not been further processed with the following exemption: The votes of participants voting the original or the lower QP inferior to the higher QP in the ‘check’ test were not regarded in the evaluation of the corresponding session.

Table 4: MOS results for the A/B comparisons under test

| Test | Name | MOS | CI |  | Test | Name | MOS | CI |  | Test | Name | MOS | CI |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EE1-1.2 | ArenaOfValor QP34 | 0.00 | 0.57 |  | EE1-1.7 | ArenaOfValor QP34 | -0.11 | 0.65 |  | Y0041 | Darktree QP33 | 0.11 | 0.47 |
| EE1-1.2 | ArenaOfValor QP39 | 0.78 | 0.62 |  | EE1-1.7 | ArenaOfValor QP39 | 0.22 | 0.56 |  | Y0041 | Darktree QP38 | 0.89 | 0.38 |
| EE1-1.2 | BQTerrace QP34 | 0.11 | 0.57 |  | EE1-1.7 | BQTerrace QP34 | -0.22 | 0.56 |  | Y0041 | Darktree QP43 | 1.78 | 0.64 |
| EE1-1.2 | BQTerrace QP39 | 0.56 | 0.51 |  | EE1-1.7 | BQTerrace QP39 | 0.67 | 0.65 |  | Y0041 | FontainebleauCinematic QP33 | 0.44 | 0.53 |
| EE1-1.2 | Campfire QP34 | 0.60 | 0.68 |  | EE1-1.7 | Campfire QP34 | 0.60 | 0.42 |  | Y0041 | FontainebleauCinematic QP39 | 0.78 | 0.54 |
| EE1-1.2 | Campfire QP37 | 1.40 | 0.68 |  | EE1-1.7 | Campfire QP37 | 0.87 | 0.71 |  | Y0041 | FontainebleauCinematic QP44 | 1.78 | 0.56 |
| EE1-1.2 | CatRobot QP39 | 0.33 | 0.62 |  | EE1-1.7 | CatRobot QP39 | 0.60 | 0.57 |  | Y0041 | FontainebleauCinematicS QP33 | -0.11 | 0.57 |
| EE1-1.2 | CatRobot QP42 | 0.73 | 0.65 |  | EE1-1.7 | CatRobot QP42 | 1.00 | 0.66 |  | Y0041 | FontainebleauCinematicS QP38 | 0.78 | 0.44 |
| EE1-1.2 | DaylightRoad2 QP39 | 0.87 | 0.46 |  | EE1-1.7 | DaylightRoad2 QP39 | 0.73 | 0.52 |  | Y0041 | FontainebleauCinematicS QP44 | 2.00 | 0.48 |
| EE1-1.2 | DaylightRoad2 QP42 | 0.87 | 0.60 |  | EE1-1.7 | DaylightRoad2 QP42 | 0.60 | 0.78 |  | Y0041 | FontainebleauFPV QP33 | 0.33 | 0.55 |
| EE1-1.2 | FourPeople QP34 | 0.06 | 0.59 |  | EE1-1.7 | FourPeople QP34 | -0.65 | 0.60 |  | Y0041 | FontainebleauFPV QP38 | 0.89 | 0.38 |
| EE1-1.2 | FourPeople QP39 | 1.12 | 0.41 |  | EE1-1.7 | FourPeople QP39 | 0.06 | 0.68 |  | Y0041 | FontainebleauFPV QP43 | 1.67 | 0.45 |
| EE1-1.2 | Johnny QP34 | -0.06 | 0.49 |  | EE1-1.7 | Johnny QP34 | 0.29 | 0.67 |  | Y0041 | Racing QP40 | 1.33 | 0.48 |
| EE1-1.2 | Johnny QP39 | 0.88 | 0.53 |  | EE1-1.7 | Johnny QP39 | 1.12 | 0.53 |  | Y0041 | Racing QP45 | 2.56 | 0.51 |
| EE1-1.2 | Kimono1 QP34 | 0.11 | 0.47 |  | EE1-1.7 | Kimono1 QP34 | -0.33 | 0.78 |  |  |  |  |  |
| EE1-1.2 | Kimono1 QP39 | 0.78 | 0.62 |  | EE1-1.7 | Kimono1 QP39 | -0.67 | 0.85 |  | Check | CatRobot QP39 | 1.13 | 0.26 |
| EE1-1.2 | KristenAndSara QP34 | 1.00 | 0.34 |  | EE1-1.7 | KristenAndSara QP34 | -0.06 | 0.59 |  | Check | FontainebleauFPV QP32 | 1.33 | 0.35 |
| EE1-1.2 | KristenAndSara QP39 | 1.12 | 0.63 |  | EE1-1.7 | KristenAndSara QP39 | 0.53 | 0.63 |  | Check | Kimono1 QP39 | 2.56 | 0.40 |
| EE1-1.2 | redkayak QP34 | 0.78 | 0.54 |  | EE1-1.7 | redkayak QP34 | 0.44 | 0.53 |  | Check | KristenAndSara QP34 | 3.00 | 0.00 |
| EE1-1.2 | redkayak QP39 | 1.22 | 0.44 |  | EE1-1.7 | redkayak QP39 | 1.00 | 0.45 |  | Check | Racing QP37 | 1.56 | 0.43 |
| EE1-1.2 | Spincalendar QP34 | -0.18 | 0.48 |  | EE1-1.7 | Spincalendar QP34 | 0.18 | 0.59 |  | Check | Tango2 QP42 | 1.40 | 0.42 |
| EE1-1.2 | Spincalendar QP39 | 0.18 | 0.59 |  | EE1-1.7 | Spincalendar QP39 | 0.53 | 0.79 |  |  |  |  |  |
| EE1-1.2 | Tango2 QP39 | 0.20 | 0.64 |  | EE1-1.7 | Tango2 QP39 | 0.20 | 0.51 |  |  |  |  |  |
| EE1-1.2 | Tango2 QP42 | 0.47 | 0.71 |  | EE1-1.7 | Tango2 QP42 | 0.60 | 0.78 |  |  |  |  |  |

Figure 2: MOS plots

In the above figure, the results for the different test categories are aggregated. The different experiments are marked in the figure. It is noted that in the super-resolution category, EE1-2.1 refers to the VTM RPR-based approach while EE1-2.4 refers to NN-based super-resolution.

# Conclusions

The number of participants was only 18 which was indicated as the lowest number to be accepted for running the tests. Due to the uncontrolled and variable viewing conditions the resulting data may be of limited reliability and should be treated with care.

Based on the reported MOS values, the following observations are made:

For the EE1-2 experiment, a significant visual benefit is reported for 14 out of 24 test cases. For the remaining cases, MOS=0 is included in the confidence interval such that comparable quality is indicated.

In the EE1-1.7 experiment, 9 test cases out of indicate a significant visual benefit while one case showed a significant loss. The remaining 14 cases indicate comparable quality.

In the super-resolution category (EE1-2.x), visual benefit is observed for three cases for the NN-based method (2.4) and one for the RPR-based approach. The other cases indicate no benefit by the confidence interval with two cases for the RPR-based approach which show a tendency towards loss.

It is recommended to determine a sufficiently large group of volunteers well before the planned test date in order to avoid difficulties in acquisition at test time. This may be done by the end of the current meeting or (latest) by the AhG meeting in preparation of the viewing tests. The test preparation would be completed if the requested number of participants is reached.

# References

- J. Jung, M. Wien, V. Baroncini, “Draft SC29/AG5 Guidelines for remote experts viewing sessions,” Doc. JVET-W0185, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 23rd meeting, online, July 2021.

- E. Alshina, M. Wien, A. Segall, J.Sauer, “AhG11/AhG4/EE1 viewing preparation report,“ Doc. JVET-Z0045, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 26th meeting, online, Apr. 2022.

- G. Martin-Cocher, M. Badawi, T. Poirier, S. Puri, K. Naser, “AhG-7 Proposed new class of gaming sequences with depth and optical flow information”, Doc. JVET-Y0041, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 25th meeting, online, Jan. 2022.