| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0055 |
| --- | --- |

| Title: | EE2-3.1: Cross-component palette coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bharath Vishwanath, Kai Zhang, Li Zhang 8910 University Center Lane, San Diego, CA, 92122, USA | Tel: Email: | bharath.vishwanath@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports results for EE2-3.1 on cross-component palette coding (CC-PLT). CC-PLT employs a lookup table to record the corresponding chroma sample value, given a luma sample value. The lookup table is built based on neighboring reconstructed samples from multiple reference lines. For non 4:4:4 sequences, a multi-filter approach is employed to derive co-located luma values.

On top of ECM-4.0, simulation results are reported as:

Class TGM: AI: -2.61%, -3.51%, -3.92%, 101%, 101%; RA: -1.25%, -1.61%, -1.83%,100%,100%

Class F: AI: -0.20%, -0.34%, -0.42%, 101%, 101%; RA: -0.36%, -1.14%, -0.99%, 101%, 100%

# Introduction

In VVC and ECM, separate palette coding for luma and chroma components is used for the dual-tree structure, ignoring cross-component correlations. In JVET-Y0062 [1], CC-PLT was proposed to address this and improve the coding efficiency.

# Proposed method

A new prediction mode known as Cross-component palette coding (CC-PLT) is proposed to explore cross-component correlations for chroma coding, specifically for the dual-tree structure.

With the CC-PLT mode, chroma samples in the current chroma block are reconstructed as follows:

Step #1: Based on the reconstructed neighboring samples, CC-PLT creates a cross-component look-up table (CC-LUT).  Given a luma sample value, a corresponding chroma sample value is recorded in CC-LUT. CC-LUT could be derived from multiple reference lines.

Step #2: For each chroma sample, the co-located luma sample value, denoted as Yc is fetched as a key to retrieve a mapping chroma sample value in CC-LUT. If Yc cannot be found in CC-LUT, Yc ± 1, Yc ± 2 and Yc ± 3 are checked in order as keys to find the mapping chroma sample value. If none of the candidate keys are in CC-LUT, the mapping chroma sample value is set to be the average of the chroma values in CC-PLT.

Step #3: For each chroma sample, the mapping chroma sample value is output as the prediction value.

Furthermore, in addition to the down-sampling filter in ECM, five candidate down-sampling filters corresponding with down-sampling locations Y1, Y2…, Y5 as shown in Fig. 1 are introduced to generate the best co-located luma sample value for the YCbCr 4:2:0 color format. The encoder tries the candidate down-sampling filters and signals an index of the best filter to the decoder.

Fig. 1 Luma down-sampling positions

# Experimental results

The tests were implemented on top of ECM-4.0 [1]. The simulations were performed following the common test conditions [2]. Table 1 shows the overall results for AI and RA tests compared to ECM-4.0. For these results, we derived the look-up table based on the reconstructed samples from the top, left, top-left and bottom-right blocks in the current CTU and their extensions by eight chroma reference lines in the neighboring CTUs.

Table 1: Performance of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.20% | -0.34% | -0.42% | 101% | 101% |
| Class TGM | -2.61% | -3.51% | -3.92% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.36% | -1.14% | -0.99% | 101% | 100% |
| Class TGM | -1.25% | -1.61% | -1.83% | 100% | 100% |

# Conclusions

In this contribution, CC-PLT is proposed for screen content coding, which brings significant coding gains with a negligible complexity increase. It is recommended to adopt CC-PLT into ECM.

# References

[1] B. Vishwanath, K. Zhang, L. Zhang, J. Xu, “Non-EE2: Cross-Component Palette Coding”,

JVET-Y0062, Jan. 2022.

[2] ECM-4.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and Evaluation Procedures for Enhanced Compression Tool Testing”, JVET-X2017, Oct. 2021.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).