| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0060-v1 |
| --- | --- |

| Title: | On new levels for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Masaru Ikeda, Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, JAPAN | Tel: Email: | +81-70-7598-7392 teruhiko.s@sony.com |
| Source: | Sony Group Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to correct Max bit rate for level 7.1 and 7.2 of HEVC. Max CPB size and Max bit rate for level 6.3 and higher were added in JVET-Y1005. Max CPB size is increased according to increase of max luma sample rate. However, Max bit rate for level 7.1 is not increased. This contribution proposes to increase the value of Max bit rate for level 7.1 and 7.2 for high tier according to the increase of max luma sample rate.

Introduction

The Table A.8 and A.9 are specified as follows in JVET-Y1005.

Table A.8 – General tier and level limits

| Level | Max luma picture size MaxLumaPs (samples) | Max CPB size MaxCPB (CpbVclFactor or CpbNalFactor bits) |  | Max slice segments per picture MaxSliceSegmentsPerPicture | Max # of tile rows MaxTileRows | Max # of tile columns MaxTileCols |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier |  |  |  |
| 1 | 36 864 | 350 | - | 16 | 1 | 1 |
| 2 | 122 880 | 1 500 | - | 16 | 1 | 1 |
| 2.1 | 245 760 | 3 000 | - | 20 | 1 | 1 |
| 3 | 552 960 | 6 000 | - | 30 | 2 | 2 |
| 3.1 | 983 040 | 10 000 | - | 40 | 3 | 3 |
| 4 | 2 228 224 | 12 000 | 30 000 | 75 | 5 | 5 |
| 4.1 | 2 228 224 | 20 000 | 50 000 | 75 | 5 | 5 |
| 5 | 8 912 896 | 25 000 | 100 000 | 200 | 11 | 10 |
| 5.1 | 8 912 896 | 40 000 | 160 000 | 200 | 11 | 10 |
| 5.2 | 8 912 896 | 60 000 | 240 000 | 200 | 11 | 10 |
| 6 | 35 651 584 | 60 000 | 240 000 | 600 | 22 | 20 |
| 6.1 | 35 651 584 | 120 000 | 480 000 | 600 | 22 | 20 |
| 6.2 | 35 651 584 | 240 000 | 800 000 | 600 | 22 | 20 |
| 6.3 | 80 216 064 | 240 000 | 1 600 000 | 600 | 22 | 20 |
| 7 | 142 606 336 | 240 000 | 1 600 000 | 1 800 | 44 | 40 |
| 7.1 | 142 606 336 | 480 000 | 3 200 000 | 1 800 | 44 | 40 |
| 7.2 | 142 606 336 | 960 000 | 6 400 000 | 1 800 | 44 | 40 |

[Ed. Note (AT/GJS/YK): During post-meeting editing, the MaxCPB values for High tier for levels 7, 7.1 and 7.2 were adjusted for sensibility relative to level 6.3. This should be checked and confirmed.]

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |  |
| 1 | 552 960 | 128 | - | 2 | 2 |  |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |
| 7 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |
| 7.1 | 8 556 380 160 | 480 000 | 1 600 000 | 6 | 4 |
| 7.2 | 17 112 760 320 | 960 000 | 3 200 000 | 6 | 4 |

[Ed. Note (AT/GJS/YK): During post-meeting editing, the MaxLumaSr and MaxBR values for level 7 were adjusted for sensibility relative to level 6.3. This should be checked and confirmed.]

We believe that the current definition of Max CPB size for level 7.0 and higher for high tier is fine. However, the value of Max bit rate for level 7.1 and 7.2 for high tier according to the increase of max luma sample rate.

Proposals

This contribution proposes to correct Table A.9 as follows. (Yellow highlighted part)

Table A.9 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |  |
| 1 | 552 960 | 128 | - | 2 | 2 |  |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 6 | 4 |
| 6.3 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |
| 7 | 4 812 963 840 | 320 000 | 1 600 000 | 6 | 4 |
| 7.1 | 8 556 380 160 | 480 000 | 3 200 000 | 6 | 4 |
| 7.2 | 17 112 760 320 | 960 000 | 6 400 000 | 6 | 4 |

Conclusions

This contribution proposes to correct Max bit rate for level 7.1 and 7.2 of HEVC. Max CPB size and Max bit rate for level 6.3 and higher were added in JVET-Y1005. Max CPB size is increased according to increase of max luma sample rate. However, Max bit rate for level 7.1 is not increased. This contribution proposes to increase the value of Max bit rate for level 7.1 and 7.2 for high tier according to the increase of max luma sample rate.

Patent rights declaration(s)

Sony Group Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).