| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0065-v1 |
| --- | --- |

| Title: | EE1-2.1: RPR encoder with multiple scale factors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE1-2.1 from JVET-Y0068, which proposed adaptive selection of picture resolution for VVC RPR functionality without multi-pass coding at the encoder side. Test 2.1.1 allows encoder to adaptively choose one of three scale factors, {x2.0 (half), x1.5 (2/3) and x1.0 (origin)}, at GOP level based on the PSNR and initial QP for the first picture of each GOP. Test 2.1.2 further includes QP adjustments for both luma and chroma components on top of test 2.1.1. The experimental results are reportedly shown as follows,

- For Test 2.1.1,

  - AI: -0.57% / 3.14% / 2.73% / 102% / 89% for Y/U/V/Enc. Time/Dec. Time

  - RA: -0.98% / 2.62% / 2.50% / 92% / 83% for Y/U/V/Enc. Time/Dec. Time

- For Test 2.1.2,

  - AI: 0.02% / 0.45% / -0.03% / 100% / 84% for Y/U/V/Enc. Time/Dec. Time

  - RA: -0.39% / -1.35% / -1.39% / 91% / 80% for Y/U/V/Enc. Time/Dec. Time

Introduction

RPR functionality in VVC can be used as an anchor for the technologies of super resolution category in EE1. To obtain high coding performance on RPR, the resolution and QP for each sequence need to be selected based on the results of multi-pass coding which requires computational complexity. In the last meeting, JVET-Y0068 [1] proposed adaptive resolution selection on RPR without multi-pass coding. The contribution provides the results for JVET-Y0068.

Proposed method

In the context of EE1-2.1, two tests are planned. The test 2.1.1 uses adaptive resolution on RPR in VVC. The test 2.1.2 additionally applies QP adjustments both luma and chroma components. Both tests support three scale factors of x2.0, x1.5 and x1.0. As JVET-Y0068, the scale factor is selected at GOP level, and applied to all pictures in the GOP. To determine the scale factor, a PSNR value and initial QP for the first picture of each GOP are exploited. Specifically, the first picture is downscaled to quarter resolution using down-sampling filters in VTM software and then resampled to the source resolution. The PSNR is calculated between original and down-up scaled pictures. One of scale factors, {x2.0 (half), x1.5 (2/3) and x1.0 (origin)}, is selected based on a threshold which is adaptively varied depending on the initial QP. RPR scale factor (RPR_SF) of i-th GOP is derived as follows,

RPR_SFGOPi=2.0,DnUpPSNRi>(BasePSNR)-QPi-BaseQP*0.5                               1.5,DnUpPSNRi>BasePSNR-OffsetPSNR-QPi-BaseQP*0.51.0, Oherwise

where DnUpPSNRi represents down-up scaled PSNR for the first picture of i-th GOP. BasePSNR and BaseQP indicate predefined criteria for PSNR and QP, respectively. OffsetPSNR is a gap of PSNR to determine a scale factor among x2.0, x1.5 and x1.0. QPi is the initial QP for the first picture of i-th GOP. The threshold is adjusted in multiples of 0.5 for the difference between the initial QP and the BaseQP.

For test 2.1.1, BaseQP and OffsetPSNR are set to 37 and 3. BasePSNR for RA and AI are 45 and 42, respectively. When half or 2/3 resolutions are encoded, QP value is adjusted with offset which is defined to -6 for half and -4 for -2/3 resolutions.

For test 2.1.2, luma QP is adjusted to +1 when DnUpPSNRi is 1.5 dB larger than threshold of each QP point. According to scale factor, chroma QP adjustment is applied to -2 and -1 for x2.0 and x1.5, respectively. To signal the chroma QP offset, chroma QP offset syntax in the slice header is exploited.

Experimental results

The proposed method is implemented on top of NNVC anchor [2] and the tests are conducted by NNVC CTC [3]. For RPR experiments, encoding parameters set as follows,

ScalingRatioHor                     : 2.0
ScalingRatioVer                     : 2.0
ScalingRatioHor2                    : 1.5
ScalingRatioVer2                    : 1.5
FractionNumFrames                  : 1
UpscaledOutput                      : 2

To support multiple scale factors, two encoding parameters are added with “ScalingRatioHor2” and “ScalingRatioVer2”.

Table 1 and Table 2 show the performance of the test 2.1.1 under AI and RA configurations, respectively.

Table 1. Performance of the EE1-2.1.1 in AI

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -1.55% | 2.38% | 2.60% | -4.34% | 2.99% | 4.09% | 110% | 67% | #DIV/0! | 7% |
| Class A2 | -2.81% | 12.66% | 9.69% | -6.91% | 9.98% | 8.04% | 97% | 77% | #DIV/0! | 4% |
| Class B | 0.52% | 2.13% | 2.27% | -1.41% | 3.09% | 3.92% | 101% | 92% | #DIV/0! | 0% |
| Class C | 0.00% | 0.00% | 0.00% | -0.01% | -0.09% | -0.04% | 101% | 107% | #DIV/0! | 0% |
| Class E | 0.09% | 0.25% | 0.27% | -0.36% | 0.68% | 0.42% | 102% | 104% | #DIV/0! | 0% |
| Overall | -0.57% | 3.14% | 2.73% | -2.33% | 3.11% | 3.17% | 102% | 89% | #DIV/0! | 2% |
| Class D | 0.02% | 0.06% | 0.05% | 0.00% | 0.06% | 0.05% | 102% | 105% | #DIV/0! | 0% |
| Class F | 0.00% | 0.00% | 0.00% | 0.06% | -0.01% | 0.03% | 103% | 108% | #DIV/0! | 0% |

Table 2. Performance of the EE1-2.1.1 in RA

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -3.48% | 2.23% | 3.72% | -6.91% | 0.83% | 0.87% | 79% | 55% | #DIV/0! | 1% |
| Class A2 | -1.55% | 7.81% | 6.68% | -4.61% | 8.45% | 6.83% | 87% | 74% | #DIV/0! | -1% |
| Class B | 0.08% | 1.84% | 1.28% | -1.08% | 1.83% | 1.33% | 98% | 93% | #DIV/0! | 0% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | -0.07% | -0.05% | 102% | 105% | #DIV/0! | 0% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.98% | 2.62% | 2.50% | -2.66% | 2.45% | 1.97% | 92% | 83% | #DIV/0! | 0% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% | 101% | 103% | #DIV/0! | 0% |
| Class F | 0.00% | 0.00% | 0.00% | -0.03% | -0.19% | 0.06% | 101% | 104% | #DIV/0! | 0% |

Table 3 and Table 4 show the performance of the test 2.1.2 under AI and RA configurations, respectively. Compared with test 2.1.1, test 2.1.2 compensates chroma loss but makes coding loss for luma.

Table 3. Performance of the EE1-2.1.2 in AI

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -0.30% | -5.61% | -5.13% | -2.77% | -6.88% | -6.34% | 106% | 64% | #DIV/0! | 5% |
| Class A2 | -1.44% | 9.56% | 6.66% | -4.86% | 2.91% | 0.81% | 94% | 74% | #DIV/0! | 5% |
| Class B | 1.04% | -0.72% | -1.01% | -0.71% | -1.19% | -1.39% | 99% | 87% | #DIV/0! | 1% |
| Class C | 0.00% | 0.00% | 0.00% | -0.01% | -0.09% | -0.04% | 99% | 98% | #DIV/0! | 0% |
| Class E | 0.12% | -0.04% | -0.05% | -0.29% | 0.25% | 0.04% | 100% | 95% | #DIV/0! | 0% |
| Overall | 0.02% | 0.45% | -0.03% | -1.52% | -0.97% | -1.31% | 100% | 84% | #DIV/0! | 2% |
| Class D | 0.02% | 0.01% | 0.02% | 0.01% | 0.00% | 0.03% | 99% | 98% | #DIV/0! | 0% |
| Class F | 0.00% | 0.00% | 0.00% | 0.06% | -0.01% | 0.03% | 100% | 98% | #DIV/0! | 0% |

Table 4. Performance of the EE1-2.1.2 in RA

|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | DecT GPU | bit DIFF |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -1.96% | -10.25% | -8.24% | -5.39% | -12.58% | -12.19% | 76% | 53% | #DIV/0! | -2% |
| Class A2 | -0.61% | 4.49% | 3.70% | -2.94% | 2.19% | 0.50% | 85% | 73% | #DIV/0! | -3% |
| Class B | 0.37% | -0.59% | -1.44% | -0.85% | -1.51% | -2.82% | 96% | 89% | #DIV/0! | 1% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | -0.07% | -0.05% | 101% | 99% | #DIV/0! | 0% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.39% | -1.35% | -1.39% | -1.95% | -2.60% | -3.29% | 91% | 80% | #DIV/0! | -1% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | -0.01% | 0.00% | 100% | 99% | #DIV/0! | 0% |
| Class F | 0.00% | 0.00% | 0.00% | -0.03% | -0.19% | 0.06% | 100% | 100% | #DIV/0! | 0% |

Conclusion

This contribution reports the EE test results of JVET-Y0068. The proposed method enables encoder to adaptively select picture resolution for RPR functionality without multi-pass coding. The test 2.1.1 determines a scale factor depending on the PSNR value and initial QP for the first picture of each GOP. To support quality control between luma and chroma, the test 2.1.2 further applies QP adjustments both luma and chroma components on top of test 2.1.1.

References

[1] J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.1-related: RPR encoder with multiple scale factors”, JVET-Y0068, Meeting by Teleconference, January 2022.

[2] https://vcgit.hhi.fraunhofer.de/jvet-x-ee1/VVCSoftware_VTM/-/tree/EE1-2.1

[3] S. Liu, A. Segall, E. Alshina, R-L. Liao,	“Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, Meeting by Teleconference, October 2021.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).