| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0069-v1 |
| --- | --- |

| Title: | AHG12: Longer chroma filters for RPR in ECM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, R. Yu | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson .com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

The chroma interpolation filters in ECM-4.0 are 4-tap both for normal motion compensation (MC) and reference picture resampling (RPR) which is much shorter than the 12-tap luma interpolation filters. It is noted that there are ongoing investigations on using longer chroma filter length for normal motion compensation in the current EE2 and an EE2 related contribution JVET-Z0068. This contribution proposes to increase the chroma interpolation filter lengths to 6 or 8-tap for RPR to be aligned with a possible filter length increase for the normal chroma motion compensation.

The impact on coding efficiency over RPR bugfix version of ECM-4.0 (JVET-Z0062) for test configuration with resolution switch every 16 pictures is reported for RA with 1.5x and 2x scaling for Luma/Cb/Cr as follows:

8-tap for MC and RPR:   RA 2x: -0,06%Y/-6,73%U/-6,99%V

6-tap for MC and RPR:   RA 2x: -0,04%Y/-5,71%U/-5,87%V

Encoding and decoding time is asserted to be similar as the anchor.

Considering the objective benefit it is thus suggested to include one of the proposed 6-tap or 8-tap RPR interpolation filter sets in the next version of ECM.

# Introduction

Longer chroma interpolation filters have shown to improve coding efficiency for normal motion compensation in ECM [1]. While extending the filter length for chroma for normal motion compensation it also make sense to do the same for the current 4-tap chroma RPR filters.

Below the phase and magnitude responses of the current 4-tap chroma RPR filters in ECM are shown. The green straight lines in the top part (a) of the figures show the ideal phase delay responses, and the red curves show the phase delay responses for the ECM filters. Likewise, the green line in the bottom part (b) of the figures shows the ideal magnitude responses and the red curves show the magnitude responses of the ECM filters. It can be noted that the ECM filters have suboptimal phase characteristics at lower frequencies and attenuates the magnitude response also at lower frequencies.

Figure 1: Plots of phase delay responses (a) and magnitude of frequency responses (b) of 4-tap chroma RPR 1.5x interpolation filters in ECM.

Figure 2: Plots of phase delay responses (a) and magnitude of frequency responses (b) of 4-tap chroma RPR 2x interpolation filters in ECM.

# Proposal

It is proposed to increase the chroma interpolation filter lengths to 6 or 8-tap for RPR to be aligned with a possible filter length increase for the normal chroma motion compensation.

The proposed 8-tap filter coefficients for RPR1.5x down-sampling and RPR 2.0x down-sampling are shown in the Table 1 and Table 3, respectively.

The proposed 6-tap filter coefficients for RPR1.5x down-sampling and RPR 2.0x down-sampling are shown in the Table 2 and Table 4, respectively.

It is suggested to select one set of the proposed chroma RPR interpolation filters for down-sampling. For up-sampling we propose the usage of corresponding increased lengths of chroma MC interpolation filters [2].

Table 1: 8-tap chroma interpolation coefficients RPR 1.5x downsampling

| Fractional position | Interpolation filter coefficients |
| --- | --- |
| 0/32 | -4, -20, 68, 168, 68, -20, -4, 0 |
| 1/32 | -4, -20, 64, 167, 73, -19, -5, 0 |
| 2/32 | -1, -20, 58, 164, 77, -19, -4, 1 |
| 3/32 | -1, -20, 53, 165, 81, -18, -5, 1 |
| 4/32 | 0, -19, 49, 161, 84, -15, -5, 1 |
| 5/32 | 0, -19, 45, 160, 89, -14, -6, 1 |
| 6/32 | 1, -20, 42, 157, 95, -13, -8, 2 |
| 7/32 | 1, -19, 37, 157, 98, -12, -8, 2 |
| 8/32 | 1, -18, 33, 154, 103, -10, -9, 2 |
| 9/32 | 1, -18, 31, 151, 107, -8, -11, 3 |
| 10/32 | 1, -19, 26, 152, 113, -7, -12, 2 |
| 11/32 | 1, -19, 23, 151, 116, -6, -12, 2 |
| 12/32 | 3, -17, 18, 145, 120, -4, -13, 4 |
| 13/32 | 3, -17, 16, 142, 123, -1, -14, 4 |
| 14/32 | 4, -17, 11, 141, 127, 0, -14, 4 |
| 15/32 | 4, -17, 9, 137, 131, 4, -16, 4 |
| 16/32 | 4, -16, 8, 132, 132, 8, -16, 4 |
| 17/32 | 4, -16, 4, 131, 137, 9, -17, 4 |
| 18/32 | 4, -14, 0, 127, 141, 11, -17, 4 |
| 19/32 | 4, -14, -1, 123, 142, 16, -17, 3 |
| 20/32 | 4, -13, -4, 120, 145, 18, -17, 3 |
| 21/32 | 2, -12, -6, 116, 151, 23, -19, 1 |
| 22/32 | 2, -12, -7, 113, 152, 26, -19, 1 |
| 23/32 | 3, -11, -8, 107, 151, 31, -18, 1 |
| 24/32 | 2, -9, -10, 103, 154, 33, -18, 1 |
| 25/32 | 2, -8, -12, 98, 157, 37, -19, 1 |
| 26/32 | 2, -8, -13, 95, 157, 42, -20, 1 |
| 27/32 | 1, -6, -14, 89, 160, 45, -19, 0 |
| 28/32 | 1, -5, -15, 84, 161, 49, -19, 0 |
| 29/32 | 1, -5, -18, 81, 165, 53, -20, -1 |
| 30/32 | 1, -4, -19, 77, 164, 58, -20, -1 |
| 31/32 | 0, -5, -19, 73, 167, 64, -20, -4 |

Table 2: 6-tap chroma interpolation coefficients RPR 1.5x downsampling

| Fractional position | Interpolation filter coefficients |
| --- | --- |
| 0/32 | -14, 64, 156, 64, -14, 0 |
| 1/32 | -13, 59, 155, 68, -12, -1 |
| 2/32 | -13, 56, 154, 71, -11, -1 |
| 3/32 | -13, 52, 153, 76, -10, -2 |
| 4/32 | -12, 47, 153, 79, -9, -2 |
| 5/32 | -13, 45, 151, 84, -8, -3 |
| 6/32 | -13, 39, 154, 88, -9, -3 |
| 7/32 | -14, 34, 156, 94, -10, -4 |
| 8/32 | -15, 33, 152, 99, -7, -6 |
| 9/32 | -15, 30, 150, 103, -5, -7 |
| 10/32 | -14, 26, 148, 107, -4, -7 |
| 11/32 | -12, 23, 143, 110, -1, -7 |
| 12/32 | -13, 18, 145, 116, -2, -8 |
| 13/32 | -10, 15, 139, 118, 1, -7 |
| 14/32 | -15, 19, 134, 122, 9, -13 |
| 15/32 | -14, 18, 129, 123, 13, -13 |
| 16/32 | -14, 18, 124, 124, 18, -14 |
| 17/32 | -13, 13, 123, 129, 18, -14 |
| 18/32 | -13, 9, 122, 134, 19, -15 |
| 19/32 | -7, 1, 118, 139, 15, -10 |
| 20/32 | -8, -2, 116, 145, 18, -13 |
| 21/32 | -7, -1, 110, 143, 23, -12 |
| 22/32 | -7, -4, 107, 148, 26, -14 |
| 23/32 | -7, -5, 103, 150, 30, -15 |
| 24/32 | -6, -7, 99, 152, 33, -15 |
| 25/32 | -4, -10, 94, 156, 34, -14 |
| 26/32 | -3, -9, 88, 154, 39, -13 |
| 27/32 | -3, -8, 84, 151, 45, -13 |
| 28/32 | -2, -9, 79, 153, 47, -12 |
| 29/32 | -2, -10, 76, 153, 52, -13 |
| 30/32 | -1, -11, 71, 154, 56, -13 |
| 31/32 | -1, -12, 68, 155, 59, -13 |

Table 3: 8-tap chroma interpolation coefficients RPR 2x downsampling

| Fractional position | Interpolation filter coefficients |
| --- | --- |
| 0/32 | -15, 9, 75, 118, 75, 9, -15, 0 |
| 1/32 | -14, 7, 70, 123, 75, 9, -13, -1 |
| 2/32 | -18, 7, 69, 125, 79, 14, -18, -2 |
| 3/32 | -17, 5, 66, 126, 81, 14, -16, -3 |
| 4/32 | -16, 3, 63, 127, 83, 15, -16, -3 |
| 5/32 | -17, 3, 60, 128, 85, 17, -16, -4 |
| 6/32 | -16, 3, 56, 127, 86, 19, -14, -5 |
| 7/32 | -15, 0, 56, 124, 90, 21, -15, -5 |
| 8/32 | -16, 1, 53, 124, 90, 25, -15, -6 |
| 9/32 | -15, 0, 51, 121, 93, 26, -13, -7 |
| 10/32 | -16, -2, 52, 118, 97, 29, -13, -9 |
| 11/32 | -15, -4, 51, 116, 99, 31, -13, -9 |
| 12/32 | -15, -4, 47, 116, 102, 31, -10, -11 |
| 13/32 | -15, -4, 45, 114, 103, 34, -9, -12 |
| 14/32 | -14, -5, 43, 112, 104, 37, -9, -12 |
| 15/32 | -13, -7, 42, 110, 106, 39, -9, -12 |
| 16/32 | -13, -9, 41, 109, 109, 41, -9, -13 |
| 17/32 | -12, -9, 39, 106, 110, 42, -7, -13 |
| 18/32 | -12, -9, 37, 104, 112, 43, -5, -14 |
| 19/32 | -12, -9, 34, 103, 114, 45, -4, -15 |
| 20/32 | -11, -10, 31, 102, 116, 47, -4, -15 |
| 21/32 | -9, -13, 31, 99, 116, 51, -4, -15 |
| 22/32 | -9, -13, 29, 97, 118, 52, -2, -16 |
| 23/32 | -7, -13, 26, 93, 121, 51, 0, -15 |
| 24/32 | -6, -15, 25, 90, 124, 53, 1, -16 |
| 25/32 | -5, -15, 21, 90, 124, 56, 0, -15 |
| 26/32 | -5, -14, 19, 86, 127, 56, 3, -16 |
| 27/32 | -4, -16, 17, 85, 128, 60, 3, -17 |
| 28/32 | -3, -16, 15, 83, 127, 63, 3, -16 |
| 29/32 | -3, -16, 14, 81, 126, 66, 5, -17 |
| 30/32 | -2, -18, 14, 79, 125, 69, 7, -18 |
| 31/32 | -1, -13, 9, 75, 123, 70, 7, -14 |

Table 4: 6-tap chroma interpolation coefficients RPR 2x downsampling

| Fractional position | Interpolation filter coefficients |
| --- | --- |
| 0/32 | -5, 64, 138, 64, -5, 0 |
| 1/32 | -8, 66, 132, 73, -6, -1 |
| 2/32 | -9, 62, 134, 76, -5, -2 |
| 3/32 | -10, 59, 134, 80, -4, -3 |
| 4/32 | -10, 57, 132, 82, -2, -3 |
| 5/32 | -10, 53, 132, 85, 0, -4 |
| 6/32 | -11, 50, 132, 89, 1, -5 |
| 7/32 | -12, 47, 132, 93, 2, -6 |
| 8/32 | -13, 45, 131, 96, 4, -7 |
| 9/32 | -12, 40, 131, 99, 5, -7 |
| 10/32 | -12, 39, 127, 101, 9, -8 |
| 11/32 | -13, 36, 127, 105, 10, -9 |
| 12/32 | -12, 33, 124, 107, 14, -10 |
| 13/32 | -13, 30, 124, 111, 15, -11 |
| 14/32 | -13, 29, 121, 112, 18, -11 |
| 15/32 | -12, 25, 119, 115, 21, -12 |
| 16/32 | -13, 24, 117, 117, 24, -13 |
| 17/32 | -12, 21, 115, 119, 25, -12 |
| 18/32 | -11, 18, 112, 121, 29, -13 |
| 19/32 | -11, 15, 111, 124, 30, -13 |
| 20/32 | -10, 14, 107, 124, 33, -12 |
| 21/32 | -9, 10, 105, 127, 36, -13 |
| 22/32 | -8, 9, 101, 127, 39, -12 |
| 23/32 | -7, 5, 99, 131, 40, -12 |
| 24/32 | -7, 4, 96, 131, 45, -13 |
| 25/32 | -6, 2, 93, 132, 47, -12 |
| 26/32 | -5, 1, 89, 132, 50, -11 |
| 27/32 | -4, 0, 85, 132, 53, -10 |
| 28/32 | -3, -2, 82, 132, 57, -10 |
| 29/32 | -3, -4, 80, 134, 59, -10 |
| 30/32 | -2, -5, 76, 134, 62, -9 |
| 31/32 | -1, -6, 73, 132, 66, -8 |

The phase and magnitude responses for the proposed filters are shown in below Figures 3 to 6. The proposed filters are closer to the ideal phase and magnitude especially at lower frequencies.

Figure 3: Plots of phase delay responses (a) and magnitude of frequency responses (b) of proposed 8-tap chroma RPR 1.5x interpolation filters.

Figure 4: Plots of phase delay responses (a) and magnitude of frequency responses (b) of proposed 6-tap chroma RPR 1.5x interpolation filters.

,

Figure 5: Plots of phase delay responses (a) and magnitude of frequency responses (b) of proposed 8-tap chroma RPR 2x interpolation filters.

Figure 6: Plots of phase delay responses (a) and magnitude of frequency responses (b) of proposed 6-tap chroma RPR 2x interpolation filters.

# Results

The objective performance is compared against a RPR bug fixed version of ECM-4.0 [3] in contribution JVET-Z0062 [4] for CTC in [5] but with the following RPR configuration parameters enabled for 2x (Table 6) respectively 1.5x (Table 5). The BD-rate PSNR results calculation is based on the PSNR2 (the PSNR value that measures after up-sampling of the decoded picture) [6].

,

Table 5: RPR for 1.5x scaling						       Table 6: RPR for 2x scaling

(Resolution switch after every 16 frames)

ScalingRatioHor : 1.5

ScalingRatioVer : 1.5

FractionNumFrames : 1.0

RPR : 1

UpscaledOutput : 2

SwitchPocPeriod : 16

(Resolution switch after every 16 frames)

ScalingRatioHor : 2.0

ScalingRatioVer : 2.0

FractionNumFrames : 1.0

RPR : 1

UpscaledOutput : 2

SwitchPocPeriod : 16

We also looked at some decoded sequences based on the different filter lengths in comparison to the ECM 4-tap filter and did not find any issue.

## Proposed 8-tap chroma RPR 2x with 8-tap MC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 bugfix [4] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | -1,93% | -3,63% | 100% | 102% |
| Class A2 | -0,06% | -6,38% | -6,99% | 102% | 106% |
| Class B | -0,08% | -7,92% | -7,75% | 97% | 102% |
| Class C | -0,09% | -9,10% | -8,54% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0,06% | -6,73% | -6,99% | 99% | 104% |
| Class D | -0,08% | -9,39% | -11,15% | 85% | 95% |

## Proposed 6-tap chroma RPR 2x with 6-tap MC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 bugfix [4] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | -1,63% | -2,97% | 101% | 105% |
| Class A2 | -0,02% | -5,37% | -5,80% | 102% | 108% |
| Class B | -0,07% | -6,62% | -6,48% | 96% | 102% |
| Class C | -0,07% | -7,88% | -7,34% | 105% | 113% |
| Class E |  |  |  |  |  |
| Overall | -0,04% | -5,71% | -5,87% | 100% | 107% |
| Class D | -0,06% | -8,68% | -9,90% | 86% | 101% |

## Proposed 8-tap chroma RPR 1.5x with 8-tap MC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 bugfix [4] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0,09% | -5,87% | -5,83% | 101% | 108% |
| Class C | -0,09% | -8,01% | -7,31% | 99% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,10% | -8,99% | -10,32% | 86% | 93% |

## Proposed 6-tap chroma RPR 1.5x with 6-tap MC

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 bugfix [4] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0,09% | -4,61% | -4,52% | 101% | 111% |
| Class C | -0,09% | -6,40% | -5,61% | 101% | 110% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0,08% | -7,41% | -8,24% | 87% | 94% |

# Conclusions

It has been shown that increasing the chroma interpolation filter length to 6 or 8-tap can provide some gains for RPR. It is suggested to include one of the proposed 6-tap and 8-tap chroma RPR 1.5x and RPR 2x interpolation filter sets into the next version of ECM.

# References

[1] X. Xie, K. Zhang, L. Zhang, J. Li, M. Wang, S. Wang, “Non-EE2: Long tap interpolation filtering on chroma components”, JVET-Y0172, Meeting by Teleconference, January 2022

[2] K. Andersson, R. Yu, “EE2-2.6 related: Longer chroma interpolation filters”, JVET-Z0068, Meeting by Teleconference, April 2022

[3] ECM-4.0 reference software,  https://vcgit.hhi.fraunhofer.de/ecm/ECM.git

[4] R. Yu, K. Andersson, “AHG12: RPR luma filter modifications and RPR enabling fixes in ECM”, JVET-Z0062, Meeting by Teleconference, April 2022

[5] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Meeting by Teleconference, January 2022

[6] J. Luo, V. Sergin, “JVET functionality confirmation test conditions for reference picture resampling”, JVET-Q2015, Jan. 2020

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).