| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0071-v1 |
| --- | --- |

| Title: | Cross-check of JVET-Z0065: EE1-2.1: RPR encoder with multiple scale factors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

This document verifies reported PSNR and bitrate numbers in EE1-2.1 (JVET-Z0065).

# Introduction

The software was obtained from https://vcgit.hhi.fraunhofer.de/jvet-y-ee1/VVCSoftware_VTM.git at branch EE1-2.1 commit 4fb6c1d87891bf36931c84887f25a63dd98f7c32.

Following defines can be seen:

#define JVET_Y_EE1_2_1_1                                  1 // Adaptive RPR

#define JVET_Y_EE1_2_1_2                                  0 // Adaptive RPR w/ QP adjustment

#define JVET_Y_EE1_VIEWING                          0 // RPR w/ scale 2.0 only

Where Test 2.1.1 corresponds to defining JVET_Y_EE1_2_1_1 to 1 and others to 0.

Where Test 2.1.2 corresponds to also defining JVET_Y_EE1_2_1_2 to 1.

For viewing scaling was restricted to only apply for 2.0 and this was used, in combination with test 2.1.1. This resulted that two out of 4 sequences was identical with the anchor instead of being encoded in reduced resolution.

Command line parameters for all cases:

ScalingRatioHor = 2.0

ScalingRatioVer = 2.0

ScalingRatioHor2 = 1.5

ScalingRatioVer2 = 1.5

UpscaledOutput = 2

FractionNumFrames = 1

The approach is to select downscaling by 2x or 1.5x based on evaluation of re-scaling performance (upscaledPSNR) on the first picture of a GOP and then apply the selected resolution for encoding all pictures in the GOP. The slice QP is then reduced by 6 if downscaled by 2 and reduced by 4 if downscaled by 1.5x. The heuristics for selection of downscaling ratio is shown below.

int basePSNR = 45 - (m_gopBasedTemporalFilterEnabled ? 0 : 3); // RA case 0, AI case 3

if (((basePSNR)-(m_iQP - 37)*0.5) < upscaledPSNR)

{

Encode GOP after 2x downscaling

}

else

{

if (((basePSNR - 3) - (m_iQP - 37)*0.5) < upscaledPSNR)

{

Encode GOP after 1.5x downscaling

}

else

{

Encode in original resolution.

}

}

# Results

We confirm the reported bitrate and PSNR for both Tests. For Test 2.1.2 we only tested class A. Encoding and decoding times are not accurate.

## Test 2.1.1

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -3,48% | 2,23% | 3,72% | 83% | 84% | 4% |
| Class A2 | -1,55% | 7,81% | 6,68% | 93% | 95% | 2% |
| Class B | 0,08% | 1,84% | 1,28% | 105% | 109% | 0% |
| Class C | 0,00% | 0,00% | 0,00% | 105% | 103% | 0% |
| Class E |  |  |  |  |  |  |
| Overall | -0,98% | 2,62% | 2,50% | 98% | 99% | 1% |

## Test 2.2.2

|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | bit DIFF |
| --- | --- | --- | --- | --- | --- | --- |
| Class A1 | -1,96% | -10,25% | -8,24% | 80% | 81% | 5% |
| Class A2 | -0,61% | 4,49% | 3,70% | 90% | 89% | 3% |

# References

[1] 	J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.1: RPR encoder with multiple scale factors”, JVET-Z0065, on-line meeting, April 2022

[2] E. Alshina, S. Liu, W. Chen, F. Galpin, Y. Li, Z. Ma, H. Wang , “Exploration Experiments on Neural Network-based Video Coding (EE1)”, JVET-Y2023, on-line meeting, January 2022

[3] S. Liu, A. Segall, E. Alshina, R.-L. Liao , “Common Test Conditions and evaluation procedures for neural network-based video coding technology”, JVET-X2016, on-line meeting, October 2021

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).