| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20 – 29 April 2022 | Document: JVET-Z0077 |
| --- | --- |

| Title: | AHG11: Extension of DOVC to Regular 2D Videos |  |  |
| --- | --- | --- | --- |
| Status: | [Input document to JVET] |  |  |
| Purpose: | [Proposal] |  |  |
| Author(s) or Contact(s): | Qipu Qin Cheolkon Jung Dan Zou Ming Li | Email: | kippqin@stu.xidian.edu.cn zhengzk@xidian.edu.cn zoudan@oppo.com myron.li@oppo.com |
| Source: | [Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd.] |  |  |

_____________________________

- Abstract

This contribution reports extension of the deep omnidirectional video compression framework (DOVC) to regular 2D videos. Two DOVC frameworks were provided in JVET-X0043 [1] and JVET-Y0051 [2], namely DOVC-RGB and DOVC-YUV, respectively, and DOVC-YUV is conversion of DOVC-RGB from the RGB 4:4:4 domain to the YUV 4:2:0 domain. In this contribution, we apply DOVC-YUV to the regular 2D video dataset after retraining to verify its effectiveness. We generate 5 different models for both luma and chroma components according to 5 lambdas, i.e. 256, 512, 1024, 2048, and 4096. Compared with VTM-16.0 under RA configuration, DOVC-YUV achieves average BD-rate reductions of {15.525% (Y), 18.948% (U), and 14.995% (V)} and average BD-PSNR gains of {-0.2045dB (Y), -0.3541dB (U), and -0.2761dB (V)} on the all dataset. Furthermore, DOVC-YUV takes advantage of GPU parallel processing and thus the average encoding time of DOVC-YUV is only 0.022 times that of VTM-16.0.

Introduction

Recently, deep learning-based video compression (DLVC) has achieved great advances in improving coding efficiency. However, most of DLVC frameworks mainly focus on 2D videos for compression and use unidirectional motion estimation.

This proposal inherits the deep omnidirectional video compression frameworks proposed in JVET-X0043 [1] and JVET-Y0051 [2], namely DOVC-RGB and DOVC-YUV. They suggest taking the consecutive adjacent frames together as input into consideration and jointly predict motion vector and compensation frame at once. To verify the universality of DOVC-YUV, this contribution retrains the DOVC-YUV model on the regular 2D video dataset and tests accordingly on Class A1, Class B, Class C and Class D.

Proposed Method

Network Architecture (Encoder)

The network framework of DOVC-YUV is shown in Figure 1. DOVC-YUV is mainly composed of the following modules: projection, separate channels, bidirectional motion estimation, motion encoder/ decoder, bidirectional motion compensation, residual encoder/decoder, quality enhancement and entropy coding. The details of each module are described in the following sub-sections.

Figure 1. Overall framework of DOVC-YUV. Note that the main contributions of this framework lie in taking omnidirectional video with YUV 4:2:0 color space as the compression target, bidirectional motion estimation and motion compensation (i.e. blue blocks) and quality enhancement (i.e. purple block) components.

Channel Separation

(a) DOVC-YUV in JVET-Y0051 [2]          (b) DOVC-RGB in JVET-X0043 [1]

Figure 2. Illustration of the difference between DOVC-YUV and DOVC-RGB.

The traditional video compression standards (such as HEVC/H.265 and VVC/H.266) adopt the YUV color space, especially, YUV 4:2:0 format, for compression operations instead of encoding videos in the RGB color space. Compared with the RGB format, encoding in the YUV 4:2:0 format is less complicated in terms of memory with less bits because the size of the chroma channels is only one-fourth of the luma channel. Figure 2 illustrates the difference between DOVC-YUV and DOVC-RGB. DOVC-YUV converts the video sequence into the luma and chroma channels, and generates models for them separately.

DOVC-YUV Decoder

For I frame, DOVC adopts BPG tool [5] for image compression and decompression. The bitstream of I frame, B/P frame and sequence parameters are transmitted together from the encoder side to the decoder side as shown in Figure 3. The DOVC decoder utilizes a stand-alone version of the arithmetic coder, as shown in Figure 4. This library encodes a feature map into a bitstream or decodes a bitstream into a feature map.

Figure 3. Schematic diagram of the data transmission process.

Figure 4. Network structure of DOVC-YUV decoder.

Inference Stage

DOVC-YUV uses PyTorch framework to perform the inference. The network information in the inference stage is provided in Table 1.

Table 1. Network information for DOVC-YUV in the inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory |  |  |
|  | GPU Type | GeForce GTX3090 (24G) |
|  | Framework: | Python3.9, PyTorch1.6, CUDA10.1 |
|  | Number of GPUs per Task | 1 |
|  | Total Parameter Number | Including five model, i.e. λ= 256, 512, 1024, 2048 and 4096 for luma and chroma, respectively. Each luma’s model is 10.8M and each chroma’s model is 5.4M. |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 67.24 MB for each model |
|  | Test data information: | Class A1, Class B and Class D (CTC) |
|  | MAC (Giga) | kernel2×in_channels×out_channels |
| Optional | Total Conv. Layers | DOVC-YUV is composed of a series of CNNs, including a total of 7 modules. |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) | / |
|  | Batch size: | 4 |
|  | Patch size | Whole frame |
|  | Changes to network configuration or weights required to generate rate points | / |
|  | Peak Memory Usage | / |
|  | Other information: | / |

Training Stage

DOVC-YUV also uses PyTorch framework to train all models. The network information in the training stage is provided in Table 2.

Table 2. Network information for DOVC-YUV in the training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Nvidia Tesla V100, 2×32G |
|  | Number of GPUs per Task | 2 |
|  | Framework: | Python3.6, PyTorch1.6, CUDA10.0 |
|  | Epoch: | 100 |
|  | Batch size: | 4 |
|  | Loss function: | RDO: R +λ∙MSE Note that R represents the total number of bits for encoding, λ is Lagrangian multiplier, and MSE is a measure of distortion. |
|  | Training time: | About 48 hours / luma model About 20 hours / chroma model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | λ = 256, 512, 1024, 2048, 4096 |
| Optional | Number of iterations: | 2000000 |
|  | Patch size: | 256 × 256 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | See description in main text |
|  | Mini-batch selection process: | / |
|  | Other information: | / |
|  | Preprocessing: | Channel separation |

Experimental Results

This contribution compares DOVC-YUV with VTM-16.0 under RA configuration. In the experiment, we use a PC with Intel Xeon CPU (dual processor) and 32G RAM for VTM-16.0, GeForce GTX3090 (24G) GPU for test DOVC-YUV and NVIDIA Tesla V100 GPUs (2×32G) for training DOVC-YUV.

R-D Curve

Figure 5 shows the rate-distortion comparison on different 2D video datasets. It is obvious that DOVC-YUV (retrain) achieves better performance than DOVC-YUV (no-retrain) in terms of the PSNR at all bit-rates. On the all 2D video datasets, DOVC-YUV (retrain) performs slightly worse than the latest video coding standard VTM16.0.

Figure 6. Rate-distortion curves on ClassA1, Class B, Class C and Class D.

BD-BR and BD-PSNR

The BDBR and BD-PSNR of DOVC-YUV are provided in Tables 3 in comparison with VTM16.0. It can be observed from Table 3 that DOVC-YUV (retrain) achieves average {15.53% (Y), 18.95% (U) and 15.00% (V)} reductions in BDBR and average {-0.2045dB (Y), -0.3541dB (U), and -0.2761dB (V)} gains in BD-PSNR when compare with the latest video coding standard VTM16.0 in terms of BDBR and BD-PSNR.

Table 3. Performance comparison with VTM16.0 on different 2D video datasets in terms of BDBR (%) and BD-PSNR (dB).

|  | BD-BR (%) |  |  | BD-PSNR (dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A1 | 17.22 | 17.12 | 13.48 | -0.4018 | -0.3842 | -0.2148 |
| Class B | 25.44 | 29.16 | 19.06 | -0.5721 | -0.5901 | -0.4337 |
| Class C | 9.14 | 12.78 | 13.08 | -0.1804 | -0.2146 | 0.2156 |
| Class D | 10.30 | 16.73 | 14.36 | -0.2357 | -0.2273 | -0.2302 |
| Average | 15.53 | 18.95 | 15.00 | -0.2045 | -0.3541 | -0.2761 |

Runtime Comparison

We also provide runtime comparison of encoding and decoding with DOVC-YUV (retrain) and VTM16.0 on 2D video sequences. Since VTM16.0 only work in CPU due to the block-level sequential processing, we test them on a PC with Intel-Xeon(R) dual CPU and 32G RAM. However, DOVC-YUV (retrain) performs global optimization at the frame level and takes advantage of implementing GPU parallel processing for encoding and decoding. Thus, we test DOVC-YUV (retrain) on a DGX Station with GTX3090 for encoding and decoding. It can be observed that the average encoding time of DOVC-YUV (retrain) is only 0.022 times that of VTM16.0 thanks to the GPU parallel processing. For decoding time, DOVC-YUV (retrain) is a bit higher than VTM16.0 because the GPU-based high performance computing is not very effective for decoding with low complexity.

Table 5: Runtime comparison of encoding and decoding with VTM16.0 on 2D video sequences.

|  | VTM-16.0 |  | DOVC-YUV |  |
| --- | --- | --- | --- | --- |
|  | Enc T (s) | Dec T (s) | Enc T (s) | Dec T (s) |
| Class A1 | 246714.75 | 232.48 | 3629.61 | 241.57 |
| Class B | 84096.71 | 216.75 | 2174.73 | 233.18 |
| Class C | 12910.19 | 107.52 | 1253.25 | 106.47 |
| Class D | 3427.18 | 68.17 | 587.38 | 85.23 |
| Average | 86787.21 | 156.23 | 1911.24 | 166.61 |

Conclusion

This contribution presents DOVC-YUV (retrain) for regular 2D video compression, which is an extension of DOVC-RGB to the YUV domain. Experimental results demonstrate that DOVC-YUV achieves outstanding performance on regular 2D video datasets in terms of quantitative measurements and runtime for encoding. In conclusion, DOVC-YUV can be successfully applied to the omnidirectional video and regular 2D video compression.

Therefore, we recommend including DOVC-YUV in EE1 as a test case and further study on it.

References

[1] Q. Qin, C. Jung, D. Zou, and M. Li, “[AHG11 & AHG6] DOVC: Deep Omnidirectional Video Compression,” JVET-X0043, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 24th Meeting, by teleconference, 6 – 15 Oct. 2021.

[2] Q. Qin, C. Jung, D. Zou, and M. Li, “[AHG11] Deep Omnidirectional Video Compression in YUV domain,” JVET-Y0051, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 25th Meeting, by teleconference, 12 – 21 Jan. 2022.

Patent rights declaration(s)

Xidian University and Guangdong OPPO Mobile Telecommunications Corp., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).