| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0083-v3 |
| --- | --- |

| Title: | Non-EE2: Fix on MHP parsing condition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution is to address a potential issue in ECM4.0. Specifically, AMVR index is referenced before it is parsed. Therefore, in order to resolve the problem, two solutions are proposed.

- Solution #1: remove dependency between AMVR and MHP

- Solution #2: relocate MHP parsing behind AMVR syntax and remove dependency between MHP and MVSD

For Solution #1, simulation results reportedly show no changes in coding performance and run-time compared to ECM4.0 as anchor in CTC [2] because combination of MHP and AMVR for affine inter mode is not permitted from fast encoder option. But in case of disabling fast encoder option (BcwFast = 0) in cfg file, ECM 4.0 software works by the proposed modification, otherwise parsing error is observed.

For Solution #2, simulation results reportedly show -0.01%, -0.03% BD-rate changes compared to ECM4.0 as anchor in RA, LB configurations.

Problem statement

In the previous meeting, MVD sign prediction [1] was adopted in ECM4.0. There were several syntaxes reordered between MVD sign prediction and Multi-Hypothesis Prediction (MHP). For example, mh_pred_data() syntaxes were moved to in front of mvd_coding() syntaxes. As a result, it causes mh_pred_data() to reference the AMVR index(imv), which is not parsed yet.

In ECM software, there are many encoder optimization techniques to speed up by skipping to check possible modes. Hence, ECM4.0 does not generate parsing errors or mismatches in current CTC coding environment. However, when BCW Fast encoder option is turned off, decoder does not work because of parsing error. Therefore, referencing a not decoded syntax should be fixed to prevent from the potential bugs.

Figure 1-1 shows part of parsing order of inter mode and dependencies between inter tools.

Figure 1-1 parsing order and dependencies

Figure 1 describes two dependency cases referring syntaxes. Dependency #1 shows mvd_coding() and mvsd_idx(index for MVD sign prediction) are referencing the mh_pred_data().Dependency #2 is about for mh_pred_data() referencing imv.

Dependency #1 : mvsd index is parsed when explicitly signaled MHP data is not present. Also, sign of mvd is not parsed at same condition. Dependency #2 : mhp data is not parsed when current block is affine mode and imv is greater than 0. : wrong referencing

Proposed method

MHP has constraints as below table considering the trade-off between performance and complexity. If one of conditions is satisfied, MHP syntaxes are not parsed.

slice_type != B IBC mode CIIP Mode GPM-MERGE TM-MERGE mode BM-MERGE mode !merge_flag && bcw_idx == Default (Equal weight) !merge_flag && affine && imv != 0 cbWidth * cbHeight <=64 ¦¦ min(cbWidth, cbHeight) < 8

But, there is a problem on MHP parsing conditions of ECM4.0 software as above mentioned, to resolve the problem, two solutions are proposed.

Solution #1: remove dependency between AMVR and MHP

In this solution, it is proposed to remove the dependency between MHP and AMVR. Therefore, a condition marked as stricken red font is removed.

slice_type != B IBC mode CIIP Mode GPM-MERGE TM-MERGE mode BM-MERGE mode !merge_flag && bcw_idx == Default (Equal weight) !merge_flag && affine && imv != 0 cbWidth * cbHeight <=64 ¦¦ min(cbWidth, cbHeight) < 8

Solution #2: relocate MHP parsing behind AMVR syntax and remove dependency between MHP and MVSD

In this solution, mh_pred_data() syntaxes are followed AMVR syntax and dependency between MHP and MVSD is removed. It means that dependency #1 in Figure 1-1 is removed while retaining conditions between MHP and AMVR with correct dependency. Figure 2-1 shows the proposed solution to fix the problem.

Figure 2-1 relocation of mh_pred_data()

Simulation Results

The proposed method has been implemented on ECM4.0.

For Solution #1, the experimental results reportedly show no changes in coding performance and run-time compared to ECM4.0 as anchor in RA(LB) configurations.

|  | Over ECM-4.0 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 98% | 99% |
| Class E |  |  |  |  |  | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 102% | 101% | 0.00% | 0.00% | 0.00% | 101% | 101% |

For Solution #2, simulation results reportedly show -0.01%, -0.03% BD-rate changes compared to ECM4.0 as anchor in RA, LB configurations.

|  |  |  | Over ECM-4.0 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Random access Main10 |  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y |  |  | U | V | EncT | DecT | Y |  | U | V | EncT | DecT |
| Class A1 | 0.00% |  |  | 0.01% | 0.00% | 101% | 101% |  |  |  |  |  |  |
| Class A2 | 0.00% |  |  | -0.02% | -0.01% | 101% | 101% |  |  |  |  |  |  |
| Class B | -0.01% |  |  | 0.01% | -0.04% | 100% | 101% | 0.00% |  | -0.04% | -0.37% | 102% | 101% |
| Class C | -0.01% |  |  | 0.01% | -0.10% | 100% | 100% | -0.03% |  | 0.15% | -0.29% | 100% | 101% |
| Class E |  |  |  |  |  |  |  | -0.08% |  | -0.13% | -0.52% | 100% | 101% |
| Overall | -0.01% |  |  | 0.00% | -0.04% | 101% | 101% | -0.03% |  | 0.00% | -0.38% | 101% | 101% |
| Class D | 0.02% |  |  | -0.01% | -0.02% | 100% | 100% | -0.01% |  | 0.72% | 0.82% | 99% | 99% |
| Class F | 0.00% |  |  | -0.05% | -0.01% | 100% | 100% | 0.11% |  | 0.09% | -0.20% | 100% | 101% |

Conclusion

In this contribution, it is raised an issue on parsing condition. Since encoder fast option is used in CTC, ECM software does not generate mismatch issues in current CTC, but there is potential bug referencing syntax which is not parsed yet. To resolve the problem, two solutions are proposed. Solution #1 is to remove dependency between MHP and AMVR on condition check for MHP parsing is proposed and Solution #2 is to relocate the MHP syntaxes behind AMVR and to remove dependency between MHP and MVSD. And it is recommended to adopt this modification in the next ECM.

Reference

- Mehdi Salehifar, et al., “EE2-3.9 and EE2-3.10: TM based reordering for MMVD and affine MMVD and MVD sign prediction”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 JVET-Y0067, 25th Meeting, by teleconference, 12–21 January 2022.

- Marta Karczewicz, et al., “Common test conditions and evaluation procedures for enhanced compression tool testing”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 JVET-Y2017, 25th Meeting, by teleconference, 12–21 January 2022.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).