| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0085-v2 |
| --- | --- |

| Title: | Non-EE2 : amvpMerge without decoder side mv refinement process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Jaehyun Lim Seung-hwan Kim | Tel: Email: | hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the ECM-4.0, amvpMerge is disabled when DMVD is disabled by the SPS flag since the bilateral matching based Merge candidate reordering and mv refinement are to be applied as default. However, even though DMVD is disabled, amvpMerge still can generate amvpMerge candidate list without bilateral matching based merge list reordering and mv refinement. In this proposal, it is suggested to enable amvpMerge without merge candidate reordering process and mv refinement process when the DMVD is disabled in SPS.

The simulation result of the proposed method shows (-0.14%, -0.20%, -0.16%) over ECM-4.0 with DMVD off for RA. The amvpMerge is automatically disabled at low delay case in the slice level therefore there is no changes for LDB configuration.

Problem Statement

Current amvpMerge list construction includes bilateral matching based merge list reordering and also mv refinement process. From this reason, amvpMerge is disabled when DMVD is off in SPS. The amvpMerge mode constructs bi-directional motion information by AMVP and MERGE for each reference list. Additionally TM and BM based processing is applied to achieved the improved prediction performance. Therefore amvpMerge mode can be applied without TM/BM based decoder side refinement when the DMVD is disabled. If DMVD is disabled, the coding performance decrement is noticeable since many coding tools are disabled and amvpMerge can give benefit in terms of coding performance without decoder side TM/BM based operation for this scenario.

Proposed method

The proposed method suggests to generate amvpMergeList without reordering process and refinement process by bypassing the process when DMVD is disabled.

The proposed merge candidate reordering based on BM cost is applied adaptively based on DMVD flag as shown in Figure 1.

Figure 1

And also the mv refinement process for the final motion information of amvpMerge is bypassed when the DMVD is off in SPS as shown in Figure 2.

Figure 2

Simulation result

In order to evaluate the compression benefit of the amvpMerge without DMVD process, the anchor is considered as ECM-4.0 CTC with DMVD off.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 (DMVD off) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.20% | -0.13% | 104% | 100% |
| Class A2 | -0.14% | -0.25% | -0.20% | 103% | 100% |
| Class B | -0.15% | -0.21% | -0.15% | 102% | 100% |
| Class C | -0.12% | -0.15% | -0.18% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.20% | -0.16% | 103% | 100% |
| Class D | -0.23% | -0.07% | -0.36% | 103% | 99% |
| Class F | -0.07% | -0.01% | -0.04% | 101% | 100% |
| Class TGM | 0.00% | -0.01% | -0.02% | 101% | 100% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 (DMVD off) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 96% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 97% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 97% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class TGM | 0.00% | 0.00% | 0.00% | 97% | 98% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).