| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0086-v1 |
| --- | --- |

| Title: | EE1-1.4: ALF improvement for NNVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenjie Zou, Yi Zhou, Cheng Huang, and Yaxian Bai | Email: | wjzou@xidian.edu.cn yi.zhou@stu.xidian.edu.cn huang.cheng5@zte.com.cn bai.yaxian@zte.com.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution presents an ALF modification in NNVC. When a neural network-based filter is used as in-loop filter, two ALF flags are introduced to indicate ALF enabled/disable for luma and chroma components, respectively. It is reported that, with JVET-X0065 as anchor and test platform, the overall performance is as follows:

- AI: 0.03%/-0.96%/-1.04% with 100%EncT/100%DecT

By setting chroma QP offset to 1, the overall performance is as follows:

- AI: 0.03%/-1.07%/-1.09% with 100%EncT/100%DecT

Introduction

As shown in Figure 1b, in current solutions for NNVC, the NN filter generally replaces the DBF and SAO modules. ALF module can improve the coding performance after the NN filter.

(a)

(b)

Figure 1: (a) Original in-loop filter process. (b) in-loop filter process after adding NN filter

In VVC [1], shown below and in Figure 2, there is an ALF Flag to indicate whether ALF is enable or disable. When this flag is on, luma ALF is always used, and then the chroma ALF and CCALF, are further be controlled by their own flags. If the AFL flag is off, the chroma ALF and CCALF will be forcibly closed.

Figure 2: Syntax structure in VVC

Benefitted from the advantage of ALF, the ALF Flag is always on using VVC encoder. So, subsequent chroma ALF and CCALF can obtain the optimal decision. In the NN filter + ALF combination, a large number of ALF Flags are turned off. This process will limit the optimization of the chroma ALF and CCALF and thus reduce the overall performance.

Proposed ALF-SPLIT method

A method called ALF-SPLIT is proposed to optimize the ALF module used in NNVC [2]. As shown in Figure 3, the changes of the ALF flags are as follows：

- The existing ALF flag is used to indicate whether luma ALF is on or off.

- An additional ALF flag is introduced to indicate whether chroma ALF and CCALF are on or off.

In this way, the ALF flag for the luma and chroma components can be determined independently.

Figure 3: Modified Syntax structure

Experimental results

Test 1.4.1： JVET-X0065 anchor with the ALF-SPLIT tool presented in JVET-Y0046

Based on the platform in JVET-X0065 [3], coding performance experiment was conducted following the common test conditions defined in [4]. Experimental results are shown in Table 1. The proposed method can obtain 0.03%, -0.96%, and -1.04% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 1. Performance of Test1.4.1 (the proposed method on top of JVET-X0065 CTC)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 CTC |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.06% | -0.72% | -2.75% | 100% | 101% |
| Class A2 | 0.00% | -0.17% | -0.21% | 100% | 101% |
| Class B | 0.04% | -1.68% | -1.02% | 101% | 100% |
| Class C | 0.06% | -1.55% | -1.17% | 100% | 101% |
| Class E | 0.01% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.03% | -0.96% | -1.04% | 100% | 100% |
| Class D | 0.12% | -1.53% | -1.27% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Test 1.4.2： Test 1.4.1 with larger chroma QP offset

Based on the platform in JVET-X0065, an additional experiment was conducted by setting chroma QP offset to 1. Experimental results are shown in Table 2 and Table 3.

Compared with the anchor of JVET-X0065 using CTC, the larger chroma QP offset method brings -1.44%, 12.25%, and 11.74% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 2. Performance of JVET-X0065 with larger chroma QP offset vs. JVET-X0065 CTC

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 CTC |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.89% | 13.84% | 10.20% | 99% | 101% |
| Class A2 | -2.79% | 12.83% | 11.63% | 98% | 100% |
| Class B | -1.03% | 13.17% | 13.22% | 99% | 100% |
| Class C | -1.07% | 10.68% | 11.18% | 98% | 100% |
| Class E | -0.80% | 10.66% | 11.67% | 99% | 100% |
| Overall | -1.44% | 12.25% | 11.74% | 98% | 100% |
| Class D | -1.00% | 10.06% | 11.02% | 98% | 100% |
| Class F | -1.17% | 7.01% | 7.43% | 100% | 102% |

Compared with the anchor of JVET-X0065, by introducing ALF_SPLIT, the proposed method with larger chroma QP offset can obtain -1.41%, 10.99%, and 10.49% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 3. Performance of the proposed method + JVET-X0065 with larger chroma QP offset

vs. JVET-X0065 CTC

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 CTC |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.83% | 12.95% | 7.07% | 99% | 101% |
| Class A2 | -2.79% | 12.59% | 11.28% | 98% | 101% |
| Class B | -1.00% | 10.88% | 11.93% | 99% | 100% |
| Class C | -1.02% | 8.71% | 9.76% | 98% | 100% |
| Class E | -0.80% | 10.66% | 11.67% | 99% | 100% |
| Overall | -1.41% | 10.99% | 10.49% | 99% | 100% |
| Class D | -0.90% | 8.41% | 9.69% | 99% | 100% |
| Class F | -1.17% | 7.01% | 7.43% | 100% | 102% |

Further, compared with JVET-X0065 with larger chroma QP offset, the proposed method can obtain 0.03%, -1.07%, and -1.09% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 4. Performance of Test 1.4.2 (the proposed method on top of JVET-X0065 with larger chroma QP offset)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 with larger chroma QP offset |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.06% | -0.79% | -2.94% | 100% | 99% |
| Class A2 | 0.00% | -0.20% | -0.29% | 100% | 102% |
| Class B | 0.04% | -1.92% | -1.05% | 100% | 100% |
| Class C | 0.05% | -1.67% | -1.18% | 100% | 100% |
| Class E | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.03% | -1.07% | -1.09% | 100% | 100% |
| Class D | 0.10% | -1.40% | -1.12% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

These results verify that a similar coding gain of ALF-SPLIT can be achieved with larger chroma QP offset.

- 4 Conclusions

ALF_SPLIT scheme provides more flexibility for luma ALF and chroma ALF. The results from Test 1.4.1 demonstrated that the proposed ALF-SPLIT shows coding gains on BD-Rate compare with the anchor of JVET-X0065. It is also reported that coding gains of ALF-SPLIT are maintained with larger chroma QP offset in Test 1.4.2.

- 5 References

- ITU-T Study Group, Series H: Audiovisual and multimedia systems: Infrastructure of audiovisual services–coding of moving video-versatile video coding, in General Secretariat and Telecom Radiocommunication (ITU-R) Standardization (ITU-T), sec. H 266, 2020.

- Wenjie Zou, Yi Zhou，AHG11: ALF improvement for NNVC. Document JVET-Y0046, January online, October 2022.

- Y. Li, K. Zhang, L. Zhang, EE1-1.2: Test on Deep In-Loop Filter with Adaptive Model Selection and External Attention, Document JVET-X0065, online, October 2021.

- S. Liu, A. Segall, E. Alshina, R. -L. Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology, Document JVET-W2016, July 2021.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).