| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0086-v3 |
| --- | --- |

| Title: | EE1-1.4: ALF improvement for NNVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenjie Zou, Yi Zhou, Cheng Huang, and Yaxian Bai | Email: | wjzou@xidian.edu.cn yi.zhou@stu.xidian.edu.cn huang.cheng5@zte.com.cn bai.yaxian@zte.com.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

- Abstract

This contribution presents an ALF modification in NNVC. When a neural network-based filter is used as in-loop filter, two ALF flags are introduced to indicate ALF enabled/disable for luma and chroma components, respectively.

It is reported that, compared with JVET-X0065, the overall performance is as follows:

Test1.4.1:  AI: 0.03%/-0.96%/-1.04% with 100%EncT/100%DecT; RA: 0.05%/-0.30%/-0.41% with 100%EncT/100%DecT.

Test1.4.2(By setting chroma QP offset to 1): AI: 0.03%/-1.07%/-1.09% with 100%EncT/100%DecT; RA: 0.xx%/-0.xx%%/-0.xx%% with xx%EncT/xx%DecT.

Compared with VTM-11.0 + NewMCTF, the overall performance is as follows:

Test1.4.1: AI: -7.38%/-20.74%/-20.95% with 124%EncT/27071%DecT; RA: -10.03%/-23.91%/-23.50% with 165%EncT/62196%DecT.

Test1.4.2(By setting chroma QP offset to 1): AI: -8.71%/-11.85%/-12.24% with 122%EncT/27092%DecT; RA: -10.41%/-13.73%%/-13.27%% with 163%EncT/63064%DecT.

Introduction

As shown in Figure 1b, in current solutions for NNVC, the NN filter generally replaces the DBF and SAO modules. ALF module can improve the coding performance after the NN filter.

(a)

(b)

Figure 1: (a) Original in-loop filter process. (b) in-loop filter process after adding NN filter

In VVC [1], shown below and in Figure 2, there is an ALF Flag to indicate whether ALF is enable or disable. When this flag is on, luma ALF is always used, and then the chroma ALF and CCALF, are further be controlled by their own flags. If the ALF flag is off, the chroma ALF and CCALF will be forcibly closed.

Figure 2: Syntax structure in VVC

Benefitted from the advantage of ALF, the ALF Flag is always on using VVC encoder. So, subsequent chroma ALF and CCALF can obtain the optimal decision. In the NN filter + ALF combination, a large number of ALF Flags are turned off. This process will limit the optimization of the chroma ALF and CCALF and thus reduce the overall performance.

Proposed ALF-SPLIT method

Architecture

A method called ALF-SPLIT is proposed to optimize the ALF module used in NNVC [2]. As shown in Figure 3, the changes of the ALF flags are as follows：

- The existing ALF flag is used to indicate whether luma ALF is on or off.

- An additional ALF flag is introduced to indicate whether chroma ALF and CCALF are on or off.

In this way, the ALF flag for the luma and chroma components can be determined independently.

Figure 3: Modified Syntax structure

Inference

Libtorch is used for performing the inference of the proposed CNN filters in VTM. The network informati-on in the inference stage is provided in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | LibTorch v1.10.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | ~1.53M/model (24 models) |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | ~6.1MB/model, 24 models in total |
|  | MAC (Giga) | 520K/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 21+16 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128×128, 256×256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

Since ALF-SPLIT is designed after the enhancement filter proposed in JVET-X0065 [3], the training model of JVET-X0065 is directly used. The network information in the training stage in [3] is provided in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage [3]

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.6 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: | 90 |
|  | Batch size: | 64 |
|  | Training time: | 60h/model |
|  | Training data information: | DIV2K, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {17, 22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Experimental results

Test 1.4.1： JVET-X0065 CTC with the ALF-SPLIT tool presented in JVET-Y0046

Based on the platform in JVET-X0065, coding performance experiment was conducted following the common test conditions defined in [4]. Experimental results are shown in Table 3. The proposed method can obtain 0.03%, -0.96%, and -1.04% BD-rate for AI configuration, 0.05%, -0.30%, and -0.41% BD-rate for RA configuration on average for Y, Cb, and Cr components, respectively.

Based on the platform in VTM11.0+newMCTF, experimental results are shown in Table 4. The proposed method can obtain -7.38%, -20.74%, and -20.95% BD-rate for AI configuration, -10.03%, -23.91%, and -23.50% BD-rate for RA configuration on average for Y, Cb, and Cr components, respectively.

Table 3. Performance of Test1.4.1 (JVET-X0065 CTC+ALF-SPLIT vs. JVET-X0065 CTC)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 CTC |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.06% | -0.72% | -2.75% | 100% | 101% |
| Class A2 | 0.00% | -0.17% | -0.21% | 100% | 101% |
| Class B | 0.04% | -1.68% | -1.02% | 101% | 100% |
| Class C | 0.06% | -1.55% | -1.17% | 100% | 101% |
| Class E | 0.01% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.03% | -0.96% | -1.04% | 100% | 100% |
| Class D | 0.12% | -1.53% | -1.27% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 CTC |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.11% | -0.46% | -0.96% | 100% | 100% |
| Class A2 | 0.05% | -0.08% | -0.56% | 100% | 100% |
| Class B | 0.02% | -0.39% | -0.07% | 100% | 100% |
| Class C | 0.04% | -0.23% | -0.33% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.30% | -0.41% | 100% | 100% |
| Class D | 0.04% | -0.31% | -0.46% | 100% | 100% |
| Class F | 0.02% | -0.10% | -0.08% | 100% | 100% |

Table 4. Performance of Test1.4.1 (JVET-X0065 CTC+ALF-SPLIT vs. AhG11anchor)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM11.0+newMCTF |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -6.40% | -17.46% | -19.81% | 144% | 34086% |
| Class A2 | -6.57% | -20.14% | -18.66% | 127% | 27732% |
| Class B | -6.59% | -22.18% | -21.45% | 122% | 27390% |
| Class C | -7.71% | -20.10% | -21.74% | 112% | 19564% |
| Class E | -10.03% | -23.08% | -22.47% | 123% | 31731% |
| Overall | -7.38% | -20.74% | -20.95% | 124% | 27071% |
| Class D | -7.76% | -20.59% | -22.17% | 109% | 15786% |
| Class F | -3.65% | -12.56% | -10.59% | 115% | 12882% |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM11.0+newMCTF |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.23% | -18.61% | -19.73% | 168% | 61651% |
| Class A2 | -10.46% | -23.02% | -22.01% | 159% | 59091% |
| Class B | -9.60% | -26.41% | -24.64% | 177% | 68380% |
| Class C | -10.86% | -25.44% | -21.74% | 153% | 63035% |
| Class E |  |  |  |  |  |
| Overall | -10.03% | -23.91% | -23.50% | 165% | 62196% |
| Class D | -12.74% | -26.29% | -27.39% | 148% | 54847% |
| Class F | -4.99% | -15.78% | -15.49% | 231% | 38560% |

Test 1.4.2： Test 1.4.1 with larger chroma QP offset

Based on the platform in JVET-X0065, an additional experiment was conducted by setting chroma QP offset to 1. Experimental results are shown in Table 5 and Table 6.

Compared with JVET-X0065 using CTC, the larger chroma QP offset method brings -1.44%, 12.25%, and 11.74% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 5. Performance of JVET-X0065 with larger chroma QP offset vs. JVET-X0065 CTC

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 CTC |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.89% | 13.84% | 10.20% | 99% | 101% |
| Class A2 | -2.79% | 12.83% | 11.63% | 98% | 100% |
| Class B | -1.03% | 13.17% | 13.22% | 99% | 100% |
| Class C | -1.07% | 10.68% | 11.18% | 98% | 100% |
| Class E | -0.80% | 10.66% | 11.67% | 99% | 100% |
| Overall | -1.44% | 12.25% | 11.74% | 98% | 100% |
| Class D | -1.00% | 10.06% | 11.02% | 98% | 100% |
| Class F | -1.17% | 7.01% | 7.43% | 100% | 102% |

Compared with JVET-X0065 using CTC, by introducing ALF_SPLIT, the proposed method with larger chroma QP offset brings -1.41%, 10.99%, and 10.49% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 6. Performance of JVET-X0065 with larger chroma QP offset + ALF-SPLIT vs.

JVET-X0065 CTC

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 CTC |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -1.83% | 12.95% | 7.07% | 99% | 101% |
| Class A2 | -2.79% | 12.59% | 11.28% | 98% | 101% |
| Class B | -1.00% | 10.88% | 11.93% | 99% | 100% |
| Class C | -1.02% | 8.71% | 9.76% | 98% | 100% |
| Class E | -0.80% | 10.66% | 11.67% | 99% | 100% |
| Overall | -1.41% | 10.99% | 10.49% | 99% | 100% |
| Class D | -0.90% | 8.41% | 9.69% | 99% | 100% |
| Class F | -1.17% | 7.01% | 7.43% | 100% | 102% |

Further, in Table 7, compared with JVET-X0065 with larger chroma QP offset, the proposed method can obtain 0.03%, -1.07% and -1.09% BD-rate for AI configuration on average for Y, Cb and Cr components, respectively. These results verify that a similar coding gain of ALF-SPLIT can be achieved with larger chroma QP offset.

Table 7. Performance of Test 1.4.2 (JVET-X0065 with larger chroma QP offset + ALF-SPLIT vs.

JVET-X0065 with larger chroma QP offset)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 with larger chroma QP offset |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.06% | -0.79% | -2.94% | 100% | 99% |
| Class A2 | 0.00% | -0.20% | -0.29% | 100% | 102% |
| Class B | 0.04% | -1.92% | -1.05% | 100% | 100% |
| Class C | 0.05% | -1.67% | -1.18% | 100% | 100% |
| Class E | 0.01% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.03% | -1.07% | -1.09% | 100% | 100% |
| Class D | 0.10% | -1.40% | -1.12% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-X0065 with larger chroma QP offset |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.09% | -0.43% | -1.00% | 100% | 100% |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.35% | -0.02% | 100% | 100% |
| Class C | 0.04% | -0.24% | -0.25% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.04% | -0.37% | -0.31% | 100% | 100% |
| Class F | 0.01% | -0.04% | -0.05% | 101% | 101% |

In addition, the coding performance of Test 1.4.2 compared with VTM11.0+newMCTF is provided in Table 8.

Table 8.  Performance of Test 1.4.2 (JVET-X0065 with larger chroma QP offset + ALF-SPLIT vs. AhG11anchor)

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM11.0+newMCTF |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.19% | -6.52% | -12.34% | 142% | 34177% |
| Class A2 | -9.18% | -11.11% | -9.68% | 124% | 27963% |
| Class B | -7.55% | -13.00% | -11.56% | 120% | 27463% |
| Class C | -8.69% | -12.29% | -13.52% | 111% | 19505% |
| Class E | -10.75% | -15.40% | -14.11% | 122% | 31518% |
| Overall | -8.71% | -11.85% | -12.24% | 122% | 27092% |
| Class D | -8.69% | -12.83% | -13.89% | 108% | 15825% |
| Class F | -4.77% | -6.62% | -4.17% | 114% | 13039% |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM11.0+newMCTF |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.96% | -8.22% | -12.45% | 166% | 62042% |
| Class A2 | -7.25% | -13.18% | -10.66% | 155% | 57989% |
| Class B | -10.33% | -15.94% | -13.13% | 176% | 67883% |
| Class C | -11.73 | -15.51% | -16.04% | 152% | 62011% |
| Class E |  |  |  |  |  |
| Overall | -10.41% | -13.73% | -13.27% | 163% | 63064% |
| Class D | -13.51% | -16.53% | -17.13% | 147% | 55555% |
| Class F | -6.30% | -8.70% | -8.24% | 230% | 39248% |

- 4 Conclusions

ALF_SPLIT scheme provides more flexibility for luma ALF and chroma ALF. The results from Test 1.4.1 demonstrated that the proposed ALF-SPLIT shows coding gains on BD-Rate compare with JVET-X0065. It is also reported that coding gains of ALF-SPLIT are maintained with larger chroma QP offset in Test 1.4.2.

- 5 References

- ITU-T Study Group, Series H: Audiovisual and multimedia systems: Infrastructure of audiovisual services–coding of moving video-versatile video coding, in General Secretariat and Telecom Radiocommunication (ITU-R) Standardization (ITU-T), sec. H 266, 2020.

- Wenjie Zou, Yi Zhou，AHG11: ALF improvement for NNVC. Document JVET-Y0046, January online, October 2022.

- Y. Li, K. Zhang, L. Zhang, EE1-1.2: Test on Deep In-Loop Filter with Adaptive Model Selection and External Attention, Document JVET-X0065, online, October 2021.

- S. Liu, A. Segall, E. Alshina, R. -L. Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology, Document JVET-W2016, July 2021.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).