| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0087-v1 |
| --- | --- |

| Title: | EE1-related: ALF-SPLIT based on JVET-Z0070 and JVET-Y0078 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenjie Zou, Yi Zhou, Chengming Gu Cheng Huang, and Yaxian Bai | Email: | wjzou@xidian.edu.cn yi.zhou@stu.xidian.edu.cn cmgu@stu.xidian.edu.cn huang.cheng5@zte.com.cn bai.yaxian@zte.com.cn |
| Source: | Xidian University, ZTE Corporation |  |  |

_____________________________

- Abstract

An ALF split scheme was proposed in JVET-Y0046 which indicates ALF enabled/disable for luma and chroma components, respectively. To verify the coding performance, ALF-SPLIT was integrated on top of JVET-Z0070 and JVET-Y0078, respectively. It is reported that, with JVET-Z0070 as anchor and test platform, the overall performance is as follows:

- AI: 0.02%/-0.62%/-0.79% with 100%EncT/100%DecT

With JVET-Y0078 as anchor and test platform, the overall performance is as follows:

- AI: 0.02%/-0.69%/-0.43% with 100%EncT/100%DecT

Introduction

In VVC [1], shown below and in Figure 1, there is an ALF Flag to indicate whether ALF is enable or disable. When this flag is on, luma ALF is always used, and then the chroma ALF and CCALF, are further be controlled by their own flags. If the AFL flag is off, the chroma ALF and CCALF will be forcibly closed.

Figure 1: Syntax structure in VVC

A method called ALF-SPLIT was proposed in JVET-Y0046[2] as shown in Figure 2. When a neural network-based filter is used as in-loop filter, two ALF flags, which indicate ALF enabled/disable, are introduced for luma and chroma components, respectively.

Figure 2: Modified Syntax structure

Experimental results

To verify the performance of ALF-SPLIT, extensive tests were conducted based on JVET-Z0070 and JVET-Y0078.

Test 1：JVET-Z0070 anchor with the ALF-SPLIT tool presented in JVET-Y0046

Based on the platform in JVET-Z0070[3], coding performance experiment was conducted following the common test conditions defined in [5]. Experimental results are shown in Table 1. The proposed method can obtain 0.02%, -0.62%, and -0.79% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 1. Performance of JVET-Z0070 with the ALF-SPLIT tool vs. JVET-Z0070 [5]

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-Z0070 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.05% | -0.78% | -2.49% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | 0.01% | -0.39% | -0.39% | 100% | 100% |
| Class C | 0.05% | -1.69% | -1.20% | 100% | 102% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.02% | -0.62% | -0.79% | 100% | 100% |
| Class D | 0.12% | -1.72% | -1.38% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Test 2：JVET-Y0078 anchor with the ALF-SPLIT tool presented in JVET-Y0046

Based on the platform in JVET-Y0078 [4], coding performance experiment was conducted following the common test conditions defined in [5]. Experimental results are shown in Table 2. The proposed method can obtain 0.02%, -0.69%, and -0.43% BD-rate for AI configuration on average for Y, Cb, and Cr components, respectively.

Table 2. Performance of JVET-Y0078 with the ALF-SPLIT tool vs. JVET-Y0078 CTC

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over JVET-Y0078 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.01% | -0.04% | -0.05% | 100% | 99% |
| Class A2 | 0.01% | -0.02% | -0.09% | 101% | 100% |
| Class B | 0.01% | -0.48% | -0.29% | 100% | 99% |
| Class C | 0.07% | -2.45% | -1.47% | 100% | 100% |
| Class E | 0.01% | 0.00% | 0.00% | 101% | 101% |
| Overall | 0.02% | -0.69% | -0.43% | 100% | 100% |
| Class D | 0.13% | -2.55% | -1.71% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |

These results show that the proposed ALF-SPLIT brings coding gains in different NNVC frameworks including JVET-Z0070 and JVET-Y0078.

- 4 Conclusions

This contribution provides more flexibility for luma ALF and chroma ALF. The results from this contribution demonstrated that the proposed ALF-SPLIT shows coding gains on BD-Rate compare with the anchor of JVET-Z0070 and JVET-Y0078. It is also reported that coding gains of ALF-SPLIT are maintained in these networks.

- 5 References

- ITU-T Study Group, Series H: Audiovisual and multimedia systems: Infrastructure of audiovisual services–coding of moving video-versatile video coding, in General Secretariat and Telecom Radiocommunication (ITU-R) Standardization (ITU-T), sec. H 266, 2020.

- Wenjie Zou, Yi Zhou, AHG11: ALF improvement for NNVC, Document JVET-Y0046, online, January 2022.

- K. Andersson, J. Ström, D. Liu, R. Sjöberg, EE1-1.3: Combination of deblocking and NN, Document: JVET-Z0070, online, April 2022.

- Liqiang Wang, EE1-1.1: neural network based in-loop filter with constrained storage and low complexity, Document: JVET-Y0078, online, January 2022.

- S. Liu, A. Segall, E. Alshina, R. -L. Liao, JVET common test conditions and evaluation procedures for neural network-based video coding technology, Document JVET-W2016, July 2021.

- 6 Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).