| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0088-v1 |
| --- | --- |

| Title: | AHG11: A CNN-based Super Resolution Method Combined with Existing RPR Functionality |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shuang Peng, Dong Jiang, Jucai Lin Cheng Fang, Xue Zhang | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution presents a super-resolution method that combines CNN and existing RPR functionality. In this method, the CNN model is derived from the previous contribution JVET-Y0087, and the image up-sampled by RPR is added to the output of CNN. Compared with VTM-11.0-NNVC, the experimental results show 7.55% and 6.14% BD-rate gains on average for luma, under AI and RA configurations, respectively.

Introduction

A super-resolution method based on CNN is used to improve the compression efficiency in JVET-Y0087 [1]. However, the coding gain is not significant enough, especially for RA configuration. To further improve the coding performance, the low-resolution reconstructed image is first up-sampled by RPR and then added to the network.

Proposed method

Details of the model architecture, training and inference process are described below.

Architecture

The architecture of the proposed CNN model is shown in Fig.1. The network contains 16 advanced residual blocks(ARB), several convolutional layers, a concatenate layer, a shuffle layer and a shortcut connection from conv1 to conv2. In addition, a global connection from input to output is included.

The parameters of each convolution layer are [cin, k, k, cout], where cin, k and cout represent the number of input channels, the size of convolution kernel and the number of output channels respectively.

The YRpr represents reconstruction up-sampled by RPR, YLR-Rec represents reconstruction, YLR-Pred represents prediction, and QP represents quantization parameter, which are used as the input of the model. QP is expanded to a matrix of the same shape as YLR-Rec before entering the concatenate layer. The output of the model is the super-resolution reconstruction YSR.

In addition, the architecture of the designed ARB is shown in Fig.2, where ReLU, GAP and Softmax are activation function, global average pooling layer and normalization function, respectively. The channel attention extraction module is circled by a dotted line in Fig.2.

Architecture of the proposed CNN model

Architecture of the designed ARB

Training and Inference

The training stage use BVI_DVC 4K dataset [2]. The data is down-sampled by RPR and then compressed by VTM.

Other details of the training and inference stage are as shown in Table 1 and Table 2, respectively.

Network Information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | NVIDIA A40 |
|  | Framework: | PyTorch v1.9.1 |
|  | Number of GPUs per Task | 4 |
|  | Epoch: | ~500 |
|  | Batch size: | 4Kx16 |
|  | Loss function: | L1 |
|  | Training time: | ~100h |
|  | Training data information: | BVI_DVC 4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37, 42 |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 128x128 |
|  | Learning rate: | 1.00E-04 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | randomly cropped, and rotation |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

Network Information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | No GPU, Intel(R) Xeon(R) Gold 6242R CPU @ 3.10GHz |
|  | Framework: | Libtorch 1.3.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1,445,249/model |
|  | Total Number of Parameters (All Models) | 2,890,498 |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 11.02637482 |
|  | Multiply Accumulate (kMAC/pixel) | 361 |
| Optional |  |  |
|  | Total Conv. Layers | 68 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 4Kx1 |
|  | Patch size | 512x512 |
|  | Changes to network configuration or weights required to generate rate points | (e.g. ) |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling | padding |
|  | Other information: |  |

Experimental results

The proposed CNN-based up-sampling filter is tested on top of VTM-11.0-NNVC[3] with the common test conditions[4].

For the anchor and the test, the QP sets are both {22, 27, 32, 37, 42}.

Table 3 and Table 4 shows the test results for AI and RA configurations, respectively. Under AI configuration, the proposed method can bring about 7.55% BD-rate gains on average for luma. Under RA configuration, the proposed method can bring about 6.14% BD-rate gains on average for luma.

Results of the proposed method under AI configuration

|  |  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 | Tango2 | -8.73% | 1.32% | 7.95% | 119% | 45% |
|  | FoodMarket4 | -4.08% | 5.32% | 8.23% | 112% | 45% |
|  | Campfire | -10.31% | 100.00% | 32.38% | 64% | 39% |
| Class A2 | CatRobot | -7.42% | 21.72% | 28.89% | 77% | 43% |
|  | DaylightRoad2 | -1.67% | -0.07% | 2.55% | 73% | 40% |
|  | ParkRunning3 | -13.07% | 125.44% | 48.30% | 56% | 41% |
| Overall |  | -7.55% | 42.29% | 21.38% | 83% | 42% |

Results of the proposed method under RA configuration

|  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over VTM-11.0_NNVC |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Class A1 | Tango2 | -9.76% | -2.75% | 7.00% | 69% | 31% |
|  | FoodMarket4 | -7.94% | 2.91% | 4.14% | 74% | 30% |
|  | Campfire | -17.39% | 72.22% | 21.89% | 49% | 32% |
| Class A2 | CatRobot | 0.96% | 32.56% | 49.63% | 66% | 31% |
|  | DaylightRoad2 | 9.57% | 5.07% | 10.88% | 67% | 46% |
|  | ParkRunning3 | -12.30% | 225.39% | 73.77% | 44% | 39% |
| Overall |  | -6.14% | 55.90% | 27.88% | 62% | 35% |

Conclusions

This contribution proposes to use the up-sampled images as the input of the neural network, which effectively reduces the difficulty of training and improves the overall effect. The proposed method shows a promising coding gain.

References

- AHG11: An improved CNN-based Super Resolution Method, JVET-Y0087, S. Peng, D. Jiang, J. Lin, C. Fang, X. Zhang.

- https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn , Di Ma, Fan Zhang, and David R. Bull.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0-NNVC.

- JVET common test conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).