| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0089-v1 |
| --- | --- |

| Title: | AHG11: An Improved Unet-Based In-Loop Filter Method |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang, Jucai Lin, Dong Jiang, Xue Zhang, Shuang Peng, | Tel: Email: | lin_jucai@dahuatech.com jiang_dong@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution presents a convolutional neural network-based in-loop filtering method with QP based models. In this contribution, we modify the training strategy and Unet-Based model from JVET-Y0086 for the inter frame.

Compared with VTM-11.0-NNVC, the proposed method reportedly shows on average {6.03%, 20.34%, 21.13%} BD-rate reductions for {Y, Cb, Cr} under AI configuration, and { } BD-rate reductions for {Y, Cb, Cr}, under RA configuration

Introduction

A Unet-Based based NN in-loop filter was proposed to improve the compression efficiency in JVET-Y0086 [1]. In JVET-Y0086, we did not have enough time to test the experiment for RA configuration. Later we tested the experiment under RA configuration but the coding gain is not significant enough. To further improve the performance for the inter frame, we extend the resolution of the dataset, and modify the residual block.

Proposed method

Architecture

The basic architecture of Unet-Based neural network is shown in Fig.1[2].

The input size for luma NN is 160x160, including the current CTU and 16 neighboring samples to each side of the current CTU. The chroma samples are up-sampled to 160x160, as an additional channel of the input tensor.

The input size for chroma NN is 80x80, including the current CTU and 8 neighboring samples to each side of the current CTU. The luma samples are down-sampled to 80x80, as an additional channel of the input tensor.

The architecture has 12 residual blocks in total. The kernel of the convolution layer in residual block is 3x3, and the channel number of the convolution layser in residual block is 64.

Other details related to the network architecture are listed as below:

- Different groups of models are trained for luma and chroma components, the two chroma components share the same network parameters.

- The CNN in-loop filter tool is applied after SAO and before ALF. An enable flag in SPS is set to control CNN in-loop filter tool, and a frame level enable flag for each component is also set to decide whether to utilize NN filtering method to it or not.

Fig.1. Architecture of Unet-Based neural network.[1]

3. For the inter frame, the channel number of the residual block is added to 96, and the 1x1 convolution layer is added before every 3x3 convolution layer, as shown in Fig.2.

Fig.2. Architecture of residul block for inter model

Training and Inference Information

In this contribution, individual models are trained with QPs in {22, 27, 32, 37, 42}. The corresponding CNN model is selected for the test sequence according to the given QP.

The NN is implemented by PyTorch for performing training process, and Libtorch for inference process in VTM11.0_NNVC.

Last time we only used BVI4K to train the inter model, but in this contribution , we use all the resolution in BVI for training.

The network information in the training stage is provided in Table 1 as suggested by [3].

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GeForce GTX 1080 Ti |
|  | Framework: | PyTorch v1.3.0 |
|  | Number of GPUs per Task | 1 GPU was used for training |
|  | Epoch: | 20000 |
|  | Batch size: | 16 |
|  | Training time: | 150h/model |
|  | Training data information: | DIV2K (intra model), BVI (inter model) |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | QP = 22, 27, 32, 37, 42 |
| Optional | Learning rate | 1e-4 |
|  | Number of iterations |  |
|  | Patch size | 160 x 160 (Luma) 80 x 80 (Chroma) |
|  | Learning rate: | 1.00E-04 |
|  | Optimizer: | ADAM |
|  | Loss function: | MSE |
|  | Preprocessing: | randomly cropped |

The network information in the inference stage is provided in Table 2 as suggested by [3].

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | Libtorch 1.3.0 |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | intra model: 1650210 /model (luma) x 5 = 8.25M 1648482 /model (chroma) x 5 = 8.24M 16.49 M (total) inter model: 2636706 /model (luma) x 5 = 13.18M 2636130 /model (chroma) x 5 = 13.18M 26.36M (total) |
|  | Parameter Precision (Bits) in Float | 32 |
|  | Memory Parameter (MB) | intra model: 6.30 /model (luma), 6.29 /model (chroma) inter model: 10.06 /model (luma), 10.06 /model (chroma) |
|  | MAC | intra model: 537.6 kMAC/pixel (Luma) 534.9 kMAC/pixel (Chroma) 805.1 kMAC/pixel (total) inter model : 729.6 kMAC/pixel (Luma) 728.7 kMAC/pixel (Chroma) 1094.0kMAC/pixel (total) |
| Optional | Total Conv. Layers | intra model: 149 (Luma) ,149 (Chroma) inter model: 173 (Luma) ,173 (Chroma) |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Experimental results

The proposed Unet-based in-loop filtering method is tested on top of VTM-11.0-NNVC[4] according to the common test conditions defined in [3]. The proposed method is applied after SAO and before ALF.

Test results are shown in Table 3. Under AI configurations, the proposed method can bring about 6.08%, 20.34%, 21.13% BD-rate reductions on average for Y, Cb, and Cr, respectively; Under RA configurations, the proposed method can bring about  BD-rate reductions on average for Y, Cb, and Cr, respectively

Table 3. Performance of proposed method on top of VTM11.0-NNVC

|  | AI |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -5.50% | -17.80% | -21.46% | 185% | 56797% |
| Class A2 | -5.52% | -20.47% | -24.49% | 145% | 43460% |
| Class B | -5.28% | -23.73% | -21.71% | 216% | 55619% |
| Class C | -6.33% | -15.22% | -17.46% | 239% | 63728% |
| Class E | -8.23% | -23.94% | -21.38% | 226% | 65229% |
| Overall | -6.08% | -20.34% | -21.13% | 203% | 56697% |
| Class D | -6.53% | -15.72% | -18.46% | 230% | 66795% |
| Class F | -3.65% | -13.19% | -8.53% | 190% | 53718% |

|  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-11.0-NNVC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Conclusions

This contribution presents a Unet-based in-loop filtering method. Under AI configurations, the proposed method can bring about 6.08%, 20.34%, 21.13% BD-rate reductions on average for Y, Cb, and Cr, respectively; Under RA configurations, the proposed method can bring about       BD-rate reductions on average for Y, Cb, and Cr, respectively

References

- AHG11: A Unet-Based Deep In-Loop Filter, JVET-Y0086, X. Zhang, D. Jiang, J. Lin, C. Fang, S. Peng .

- X Chen, Liu Y , Zhang Z , et al. HDRUNet: Single Image HDR Reconstruction with Denoising and Dequantization[J]. IEEE, 2021.

- Common Test Conditions and evaluation procedures for neural network-based video coding technology, JVET-X2016, S. Liu, A. Segall, E. Alshina, R. -L. Liao.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-11.0-NNVC.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).