| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0092-v4 |
| --- | --- |

| Title: | EE1-related: additional results on Test 1.1 and Test 1.2 with 8-bit quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liqiang Wang Xiaozhong Xu Shan Liu Franck Galpin | Email: | liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com franck.galpin@interdigital.com |
| Source: | Tencent, Interdigital |  |  |

_____________________________

- Abstract

In this contribution, model quantization is further studied based on EE1-1.1 [] and EE1-1.2 [] using 8-bit quantization. Two neural network based in-loop filters with the smaller memory size are proposed in this contribution, compared with the implementations in EE1 tests. In all, only two or even one single model is used in the proposed filter design. In addition to the reconstruction image, other side information, such as the prediction image, partition image, slice QP and based QP, is fed into the network. Based on the EE1 anchor, the related results are shown in order of RA, LB and AI configurations as follows.

Filter 1 (based on EE1-1.1, 2 models, 8-bit, SADL implementation):

{-8.24%, -18.96%, -18.14%}, {-6.94%, -16.33%, -15.25%}, {-6.28%, -14.98%, -16.14%}

Filter 2 (based on EE1-1.2, 1 model, 8-bit, SADL implementation):

{-8.63%, -18.09%, -18.43%}, {-7.45%, -15.46%, -16.50%}, {-6.34%, -14.36%, -15.41%}

Introduction

Based on the network architecture in JVET-W0113[], more side information is utilized. The inference complexity and memory size are also taken into consideration. Compared the filters in EE1-1.1 [1] and EE1-1.2 [2], additional test results using 8-bit quantization are provided in this contribution.

Proposed method

The reconstructed image after LMCS is fed into the network. Then, the output of NN filter is processed by ALF and CCALF. In detail, as shown in Fig. 1, U and V channels are up-sampled and down-sampled in the pre-processing and post-processing, respectively. For simplicity, the resampling is done by the Nearest method.

Fig. 1 the pre-processing and post-processing units

Network architecture

Along with the reconstructed image (rec_yuv), the prediction image (pred_yuv) [] and the partition image (par_yuv) [] may be fed into the network. Based on the un-filtered reconstructed image, the partition image is generated by setting each sample value into the average value of the whole partition block.

Filter 1: The network structure is shown in Fig. 2. Note that the partition image is only fed for the I slice, and the base QP is fed only for the network of the B slice.

Filter 2: The network structure is shown in Fig. 3.

Compared with JVET-W0113[3], the ResBlock is also modified as shown in Fig. 3. With the same spirit in JVET-W0113, the number of channels firstly goes up before the activation layer, and then goes down after the activation layer. In addition, and the convolution in the skip-layer connection is removed.

Fig. 2: The network architecture for the filter 1

Fig. 3: The network architecture for the filter 2

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | Tesla A100 40GB |
|  | Framework: | Torch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1000 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time: | ~150h |
|  | Training data information: | DIV2K, TVD, BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations | 1200 |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flipped, rotated |
|  | Mini-batch selection process: | random cropped |
|  | Other information: |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Test items | Filter 1 |  | Filter 2 |
| Mandatory | HW environment: |  |  |  |
|  | GPU Type |  | CPU only |  |
|  | Framework: |  | SADL |  |
|  | Number of GPUs per Task |  | 0 |  |
|  | Number of Parameters (Each Model) |  | 1.54M | 1.9M |
|  | Total Number of Parameters (All Models) |  | 3.08M | 1.9M |
|  | Parameter Precision (Bits) |  | 8 |  |
|  | Memory Parameter (MB) |  | 1.54MB/model, 2 models in all | 1.9MB/model, 1 model in all |
|  | Multiplay Accumulate (MAC)/pixel, worst-case |  | 401K | 485K |
| Optional |  |  |  |  |
|  | Total Conv. Layers |  | 78 | 101 |
|  | Total FC Layers |  | 0 |  |
|  | Total Memory (MB) |  |  |  |
|  | Batch size: |  | 1 |  |
|  | Patch size |  | 144x144 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |  |
|  | Peak Memory Usage (Total) |  |  |  |
|  | Peak Memory Usage (per Model) |  |  |  |
|  | Border handling |  |  |  |
|  | Other information: |  |  |  |

Implementation details

- For I slices and B slices, Deblock and SAO are both enabled. The optimal filtered result is decided between the two outputs from the proposed filter and the SAO.

- A scaling operation is carried out to refine the result of NN filter. The scaling factors are signaled in the slice header for each component. The specific process is the same as the scaling process used in []. Additionally, there are 3 fixed weights to blend the results of the proposed filter and the conventional filter. These fixed weights are 1, 0.75 and 0.5, which are similar as those in [].

- The proposed filter can be turned on/off at the CTU level and slice level.

Results

All the test results are provided as follows, which are based on the EE1 anchor [][].

Results of filter 1 (based on EE1-1.1, 2 models, 8-bit, SADL implementation)

The comparison results with the EE1 anchor are provided in Table 3 to Table 5. The comparison results with the EE1-1.1.1, which is implemented by Libtorch using flt32, are provided in Table 6 to Table 8.

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -10.62% | -8.84% | -14.66% | -17.24% | 218% | 175435% |
| Class A2 | -10.60% | -8.76% | -17.81% | -14.44% | 209% | 169634% |
| Class B | -10.66% | -7.85% | -19.72% | -18.44% | 220% | 184679% |
| Class C | -11.32% | -7.88% | -22.10% | -21.24% | 185% | 168014% |
| Class E |  |  |  |  |  |  |
| Overall | -10.82% | -8.24% | -18.96% | -18.14% | 208% | 175236% |
| Class D | -11.77% | -8.79% | -19.94% | -21.44% | 192% | 170133% |
| Class F | -5.37% | -3.42% | -11.67% | -10.74% | 295% | 73414% |

Table 4. Performance of the proposed method (LB)

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -9.73% | -8.09% | -12.82% | -16.45% | 211% | 172903% |
| Class A2 | -9.13% | -7.21% | -16.03% | -13.77% | 206% | 165499% |
| Class B | -8.56% | -6.57% | -14.77% | -14.25% | 207% | 169624% |
| Class C | -9.69% | -7.12% | -18.27% | -16.49% | 181% | 158468% |
| Class E | -8.79% | -7.29% | -14.46% | -12.14% | 378% | 139226% |
| Overall | -9.15% | -6.94% | -16.33% | -15.25% | 230% | 157833% |
| Class D | -10.04% | -8.03% | -14.96% | -17.16% | 186% | 166251% |
| Class F | -4.93% | -3.59% | -10.86% | -7.04% | 293% | 69440% |

Table 5. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.44% | -5.27% | -11.36% | -16.52% | 335% | 126037% |
| Class A2 | -7.53% | -5.22% | -15.40% | -13.53% | 227% | 103488% |
| Class B | -8.15% | -5.61% | -15.50% | -16.01% | 211% | 102701% |
| Class C | -9.62% | -6.97% | -16.47% | -18.68% | 164% | 77642% |
| Class E | -10.21% | -8.54% | -15.30% | -15.17% | 235% | 121950% |
| Overall | -8.60% | -6.28% | -14.98% | -16.14% | 222% | 102893% |
| Class D | -9.19% | -6.93% | -13.92% | -18.05% | 160% | 84747% |
| Class F | -5.54% | -3.94% | -9.86% | -10.80% | 154% | 70550% |

Table 6. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-1.1.1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.48% | 1.22% | 1.89% | 179% | 585% |
| Class A2 | 0.72% | 2.00% | 0.83% | 174% | 575% |
| Class B | 0.61% | 1.28% | 2.09% | 180% | 594% |
| Class C | 0.48% | 0.60% | 1.28% | 162% | 581% |
| Class E |  |  |  |  |  |
| Overall | 0.57% | 1.23% | 1.58% | 174% | 585% |
| Class D | 0.52% | 1.58% | 1.39% | 168% | 593% |
| Class F | 0.23% | 0.39% | 0.47% | 218% | 594% |

Table 7. Performance of the proposed method (LB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-1.1.1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.41% | 0.03% | 1.82% | 177% | 583% |
| Class A2 | 0.44% | 0.76% | 0.47% | 174% | 565% |
| Class B | 0.41% | 0.42% | 1.23% | 175% | 576% |
| Class C | 0.44% | 0.39% | 1.01% | 160% | 578% |
| Class E | 1.00% | 0.64% | 3.02% | 253% | 556% |
| Overall | 0.56% | 0.41% | 1.13% | 186% | 572% |
| Class D | 0.66% | 0.80% | 1.57% | 166% | 589% |
| Class F | 0.27% | 0.62% | 0.83% | 215% | 574% |

Table 8. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-1.1.1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 1.37% | 4.33% | 1.35% | 232% | 634% |
| Class A2 | 1.42% | 2.83% | 1.20% | 184% | 622% |
| Class B | 1.04% | 1.77% | 1.25% | 176% | 615% |
| Class C | 0.90% | 1.02% | 0.84% | 147% | 584% |
| Class E | 1.71% | 2.81% | 3.60% | 191% | 623% |
| Overall | 1.24% | 2.38% | 1.56% | 181% | 613% |
| Class D | 0.93% | 2.50% | 1.36% | 145% | 606% |
| Class F | 0.52% | 1.06% | 0.57% | 140% | 588% |

Results of filter 2 (based on EE1-2.1, 1 model, 8-bit, SADL implementation)

The comparison results with the EE1 anchor are provided in Table 9 to Table 11. The comparison results with the EE1-1.2.1, which is implemented by Libtorch using flt32, are provided in Table 12 to Table 14.

Table 9. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -11.10% | -9.19% | -15.72% | -17.95% | 238% | 211906% |
| Class A2 | -10.90% | -9.23% | -17.23% | -14.60% | 226% | 200049% |
| Class B | -10.88% | -8.27% | -18.36% | -19.06% | 241% | 220104% |
| Class C | -11.28% | -8.20% | -20.18% | -20.88% | 201% | 198411% |
| Class E |  |  |  |  |  |  |
| Overall | -11.04% | -8.63% | -18.09% | -18.43% | 226% | 208457% |
| Class D | -11.75% | -9.23% | -18.08% | -20.54% | 209% | 203885% |
| Class F | -5.71% | -3.74% | -11.73% | -11.47% | 330% | 96659% |

Table 10. Performance of the proposed method (LB)

|  | Low delay B Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -10.00% | -8.43% | -12.29% | -17.15% | 230% | 208381% |
| Class A2 | -9.39% | -7.61% | -15.19% | -14.28% | 225% | 196309% |
| Class B | -9.10% | -7.17% | -14.14% | -15.64% | 226% | 205118% |
| Class C | -10.17% | -7.78% | -17.13% | -17.57% | 195% | 187623% |
| Class E | -9.02% | -7.49% | -12.78% | -14.41% | 424% | 175325% |
| Overall | -9.59% | -7.45% | -15.46% | -16.50% | 252% | 191451% |
| Class D | -10.79% | -8.73% | -15.49% | -18.49% | 202% | 199208% |
| Class F | -5.69% | -4.09% | -10.69% | -10.28% | 324% | 94138% |

Table 11. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |
|  | YUV-PSNR | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -8.03% | -5.98% | -13.06% | -15.34% | 356% | 142558% |
| Class A2 | -7.50% | -5.54% | -14.60% | -12.18% | 240% | 114852% |
| Class B | -8.01% | -5.73% | -14.21% | -15.54% | 221% | 113155% |
| Class C | -8.91% | -6.41% | -15.35% | -17.49% | 172% | 85730% |
| Class E | -10.09% | -8.45% | -14.33% | -15.73% | 248% | 133525% |
| Overall | -8.48% | -6.34% | -14.36% | -15.41% | 234% | 113937% |
| Class D | -8.71% | -6.45% | -14.05% | -16.90% | 167% | 93771% |
| Class F | -5.76% | -4.00% | -10.70% | -11.33% | 159% | 96910% |

Table 12. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-1.2.1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.17% | 0.36% | 1.00% | 192% | 609% |
| Class A2 | 0.09% | 0.79% | 0.29% | 185% | 591% |
| Class B | 0.03% | 0.82% | 0.81% | 195% | 628% |
| Class C | 0.00% | 0.72% | 0.53% | 174% | 621% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.70% | 0.67% | 186% | 615% |
| Class D | -0.22% | 1.28% | 1.06% | 181% | 627% |
| Class F | 0.02% | 0.42% | 0.20% | 238% | 640% |

Table 13. Performance of the proposed method (LB)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-1.2.1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.30% | -0.24% | 1.07% | 188% | 597% |
| Class A2 | 0.16% | 0.28% | -0.10% | 184% | 582% |
| Class B | 0.10% | 0.56% | 0.93% | 186% | 589% |
| Class C | 0.06% | -0.05% | 0.39% | 172% | 594% |
| Class E | 0.24% | 1.23% | 0.80% | 274% | 621% |
| Overall | 0.12% | 0.29% | 0.69% | 199% | 599% |
| Class D | 0.14% | 1.97% | -0.12% | 179% | 622% |
| Class F | 0.07% | 0.54% | -0.23% | 231% | 600% |

Table 14. Performance of the proposed method (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over EE1-1.2.1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.21% | 0.51% | 0.91% | 241% | 631% |
| Class A2 | 0.28% | 0.19% | -0.16% | 192% | 607% |
| Class B | 0.14% | 0.73% | 0.66% | 183% | 625% |
| Class C | 0.03% | 0.70% | 0.57% | 154% | 623% |
| Class E | 0.26% | 0.97% | 1.65% | 200% | 643% |
| Overall | 0.17% | 0.64% | 0.71% | 188% | 625% |
| Class D | 0.02% | 1.08% | 1.14% | 151% | 622% |
| Class F | 0.06% | 0.59% | 0.13% | 145% | 639% |

Conclusions

With a constrained memory size and lower complexity, additional results for the two neural network based in-loop filters proposed in EE1 tests are presented in this contribution. Good trade-off between performance and complexity can be achieved by applying the proposed method. It seems that, compared with the implementation using the float or 16-bit quantization in EE1 tests, the trade-off can be further optimized by the 8-bit quantization, especially for the filter with the single model (the filter 2).

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.

[] L. Wang, X. Xu, S. Liu, F. Galpin. EE1-1.1: neural network based in-loop filter with 2 models. JVET-Z0094, 26th Meeting, by teleconference, 20–29 April 2022.

[] L. Wang, X. Xu, S. Liu, F. Galpin. EE1-1.2: neural network based in-loop filter with a single model. JVET-Z0091, 26th Meeting, by teleconference, 20–29 April 2022.

[] L. Wang, W. Jiang, X. Xu, S. Liu. AHG11: neural network based in-loop filter. JVET-W0113, 23rd Meeting, by teleconference, 7–16 July 2021.

[]  H. Zhu, X. Xu, and S. Liu. Deep learning based in-loop filter with QP and residual distribution information. Doc. AVS-M5654, Aug. 2020.

[] W. Lin, X. He, X. Han, D. Liu, J. See, J. Zou, H. Xiong, and F. Wu, “Partition-Aware Adaptive Switching Neural Networks for Post-Processing in HEVC,” IEEE Transactions on Multimedia, vol. 22, no. 11, pp. 2749–2763, Nov. 2020.

[] H. Wang, J. Chen, K. Reuze, A. M. Kotra, M. Karczewicz. EE1-1.4: Test on Neural Network-based In-Loop Filter with Large Activation Layer. JVET-W0130, 23rd Meeting, by teleconference, 7–16 July 2021.

[] K. Andersson, J. Ström, D. Liu, R. Sjöberg. EE1-related: Combination of VVC deblocking and NN loop filtering. JVET-Y0098, 25th Meeting, by teleconference, 12–21 January 2022.

[] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/VTM-11.0_nnvc.

[] S. Liu, A. Segall, E. Alshina, R.-L. Liao. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-X2016, 24th Meeting, by teleconference, 6–15 October 2021.