| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0095-v1 |
| --- | --- |

| Title: | EE2-3.6: Combined tests involving EE2-3.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert K. Naser T. Poirier Y. Chen F. Galpin | Email: | antoine.robert@interdigital.com karam.naser@interdigital.com tangi.poirier@interdigital.com ya.chen@interdigital.com franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 combined tests involving test 3.4. This 3.4 test proposes to use Template Matching in IBC merge and AMVP modes. But three other tools deal also with IBC. Test 3.2 enlarges the IBC reference region. Test 3.3 increases the number of HMVP candidates involved in the IBC merge list prior to reordering. And test 3.5 clips the IBC motion vectors into the reference region and replaces the ending zero motion by some valid candidates.

The reported simulation results of the combined tests on top of the ECM-4.0 in SCC test conditions show gains for Y in AI, RA and LDB respectively of:

- {-6.52%, -4.21%, -3.18%} with an increased encoder runtime of 5% to 7% and a slight complexity increase of the decoder of 3% for test 3.6b = 3.4+3.2.

- {-1.19%, -1.06%, -0.98%} with an increased encoder runtime of 4% to 6% and a slight complexity increase of the decoder of 2% to 3% for test 3.6e = 3.4+3.3.

- {-0.65%, -0.44%, -0.34%} with an increased encoder runtime of 3% to 5% and a slight complexity increase of the decoder of 1% to 2% for test 3.6g = 3.4+3.5c.

- {-7.11%, -4.81%, -3.71%} with an increased encoder runtime of 5% to 7% and a slight complexity increase of the decoder of 3% to 4% for test 3.6h = 3.4+3.2+3.3.

The three tools 3.4, 3.2 and 3.3 seem orthogonal since all their gains are additional.

# Introduction

In EE2 [1], several tools have been proposed on IBC in addition to the test 3.4. This test 3.4, based on JVET-Y088 [2], proposes to use Template Matching for the IBC merge and AMVP modes.

The additional tests are:

- 3.2: it extends the IBC reference region to cover more CTUs that have been used by other coding modes in the current and above CTU rows.

- 3.3: the HMVP table size for IBC is increased to a number larger than 5 (such as 30). And the size of IBC merge list could be enlarged correspondingly. All the IBC merge candidates can be involved in the reordering process. After reordering, the first M candidates with the lowest costs can be selected as the final candidates in the IBC merge list.

- 3.5: two modifications to the IBC Merge/AMVP list construction are tested. First, before checking the validity of the BVP candidates, an adjustment of the candidates pointing outside of the reference region to the nearest boundary of the IBC reference region is applied. Second, a method is tested to replace the zero-padding candidates included to complete the candidate list when there are no sufficient valid candidates by a set of valid candidates distributed in the IBC reference region.

These three tests can be combined with test 3.4 as:

| # | Test |
| --- | --- |
| 3.6b | Test 3.2 + Test 3.4 |
| 3.6e | Test 3.3 + Test 3.4 |
| 3.6g | Test 3.4 + Test 3.5c |
| 3.6h | Test 3.2 + Test 3.3 + Test 3.4 |
| 3.6j | Test 3.2 + Test 3.4 + Test3.5c |
| 3.6k | Test 3.3 + Test 3.4 + Test 3.5c |
| 3.6l | Test 3.2 + Test 3.3 + Test 3.4 + Test 3.5c |

# Simulation results

The proposed combinations have been tested on top of the ECM-4.0 software [3] under the CTC conditions [4] and in SCC configuration.

## Test 3.4 remainder

As a reminder, EE2 test 3.4 results are depicted in Table 1.

Table 1: EE2 test 3.4 on top of ECM-4.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.65% | -0.25% | -0.37% | 105% | 102% |
| Class TGM | -0.56% | -0.52% | -0.48% | 105% | 103% |
| Overall | -0.60% | -0.38% | -0.43% | 105% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.43% | -0.67% | -0.51% | 104% | 100% |
| Class TGM | -0.49% | -0.37% | -0.38% | 104% | 102% |
| Overall | -0.46% | -0.52% | -0.45% | 104% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.32% | -0.08% | -0.57% | 103% | 100% |
| Class TGM | -0.39% | -0.32% | -0.28% | 102% | 101% |
| Overall | -0.36% | -0.20% | -0.43% | 103% | 101% |

## Test 3.6b = 3.2 + 3.4

The simulation results of test 3.6b combining test 3.4 with test 3.2 are depicted in Table 2.

Table 2: Test 3.6b of test 3.4 combined with test 3.2 on top of ECM-4.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.51% | -4.29% | -4.28% | 108% | 103% |
| Class TGM | -8.53% | -7.87% | -7.85% | 107% | 103% |
| Overall | -6.52% | -6.08% | -6.06% | 107% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.83% | -2.84% | -2.91% | 105% | 103% |
| Class TGM | -5.60% | -5.30% | -5.45% | 106% | 103% |
| Overall | -4.21% | -4.07% | -4.18% | 105% | 103% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.92% | -1.05% | -2.82% | 105% | 103% |
| Class TGM | -4.45% | -3.98% | -4.18% | 105% | 103% |
| Overall | -3.18% | -2.51% | -3.50% | 105% | 103% |

By using EE2 test 3.2 as anchor, this test 3.6b can show the gains brought by test 3.4 as presented in Table 3. This shows some improved gains in AI (-0.11% Y) and especially in LDB (-0.77% Y).

Table 3: Test 3.6b of test 3.4 combined with test 3.2 on top of EE2 test 3.2

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2 test 3.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.77% | -0.82% | -0.72% |  |  |
| Class TGM | -0.65% | -0.55% | -0.56% |  |  |
| Overall | -0.71% | -0.69% | -0.64% |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2 test 3.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.42% | -0.47% | -0.31% |  |  |
| Class TGM | -0.49% | -0.38% | -0.40% |  |  |
| Overall | -0.45% | -0.43% | -0.35% |  |  |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over EE2 test 3.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -1.09% | -1.00% | -1.89% |  |  |
| Class TGM | -1.17% | -1.02% | -1.02% |  |  |
| Overall | -1.13% | -1.01% | -1.45% |  |  |

## Test 3.6e = 3.3 + 3.4

The simulation results of test 3.6e combining test 3.4 with test 3.3 are depicted in Table 4.

Table 4: Test 3.6e of test 3.4 combined with test 3.3 on top of ECM-4.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.69% | -0.44% | -0.61% | 106% | 103% |
| Class TGM | -1.68% | -1.42% | -1.41% | 106% | 103% |
| Overall | -1.19% | -0.93% | -1.01% | 106% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.62% | -0.76% | -0.63% | 104% | 101% |
| Class TGM | -1.50% | -1.44% | -1.39% | 105% | 103% |
| Overall | -1.06% | -1.10% | -1.01% | 104% | 102% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.76% | 0.67% | -0.42% | 104% | 101% |
| Class TGM | -1.21% | -1.08% | -1.12% | 103% | 102% |
| Overall | -0.98% | -0.20% | -0.77% | 104% | 102% |

## Test 3.6g = 3.4 + 3.5c

The simulation results of test 3.6g combining test 3.4 with test 3.5c are depicted in Table 5.

Table 5: Test 3.6g of test 3.4 combined with test 3.5c on top of ECM-4.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.74% | -0.35% | -0.39% | 104% | 102% |
| Class TGM | -0.56% | -0.47% | -0.43% | 105% | 103% |
| Overall | -0.65% | -0.41% | -0.41% | 105% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.42% | -0.51% | -0.44% | 104% | 101% |
| Class TGM | -0.46% | -0.37% | -0.36% | 104% | 102% |
| Overall | -0.44% | -0.44% | -0.40% | 104% | 101% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.34% | 1.22% | -0.45% | 103% | 101% |
| Class TGM | -0.33% | -0.31% | -0.02% | 102% | 101% |
| Overall | -0.34% | 0.45% | -0.24% | 103% | 101% |

As the test 3.6g that combines test 3.4 with test 3.5c does not improve the gains compared to test 3.4 alone (-0.05% in AI, +0.02% in RA and +0.02% in LDB) due to some overlapping,

- Test 3.6j that combines test 3.4 with tests 3.2 and 3.5c will give similar results as test 3.6b combining test 3.4 with test 3.2 (Table 1).

- Test 3.6k that combines test 3.4 with tests 3.3 and 3.5c will give similar results as test 3.6e combining test 3.4 with test 3.3 (Table 2).

- Test 3.6l that combines test 3.4 with tests 3.2, 3.3 and 3.5c will also give similar results as test 3.6h combining test 3.4 with tests 3.2 and 3.3 (Table 4).

So, all these 3 tests (3.6j, 3.6k and 3.6l) do not need to be tested.

## Test 3.6h= 3.2 + 3.3 + 3.4

The simulation results of test 3.6h combining test 3.4 with tests 3.2 and 3.3 are depicted in Table 6.

Table 6: Test 3.6h of test 3.4 combined with tests 3.2 and 3.3 on top of ECM-4.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -4.63% | -4.18% | -4.40% | 107% | 104% |
| Class TGM | -9.60% | -8.80% | -8.77% | 107% | 103% |
| Overall | -7.11% | -6.49% | -6.59% | 107% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -3.01% | -3.27% | -2.96% | 105% | 103% |
| Class TGM | -6.62% | -6.33% | -6.47% | 105% | 104% |
| Overall | -4.81% | -4.80% | -4.72% | 105% | 104% |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -2.08% | -1.30% | -2.55% | 105% | 103% |
| Class TGM | -5.35% | -5.10% | -5.44% | 104% | 104% |
| Overall | -3.71% | -3.20% | -4.00% | 104% | 104% |

# Conclusion

In this contribution, the EE2 combined tests involving test 3.4 have been tested. All combinations improve the coding efficiency and seem orthogonal since the gains are additive. The best combination of EE2-3.4 with tests 3.2 and 3.3 shows a coding efficiency of {-7.11%, -4.81%, -3.71%} for Y in AI, RA and LDB along with a limited increase in encoder runtime of 5% to 7%, and in decoding time of 3% to 4%.

# References

- V. Seregin, J. Chen, L. Li, K. Naser, J. Ström, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2)”, JVET-Y2024, January 2022.

- A. Robert, K. Naser, T. Poirier, Y. Chen, F. Galpin, “EE2-related: IBC with Template Matching”, JVET-Y0088, January. 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, January. 2022.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).