| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0098 |
| --- | --- |

| Title: | EE1-2.4: CNN-based Super Resolution for Video Coding with GOP Level Adaptive Resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Yue Li2, Kai Zhang2, Li Zhang2 (Bytedance) 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA Junghak Nam3, Sunmi Yoo3, Jaehyun Lim 3, Seung Hwan Kim 4 (LGE) 319 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea 410680 Treena St, San Diego, CA 92131, USA | Tel: Email: | linchaoyi.cy@bytedance.com yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com junghak.nam@lge.com |
| Source: | Bytedance Inc., LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution reports the EE1-2.4 test results, which is a combination of JVET-Y0070 and JVET-Y0068. At each GOP, the encoder can adaptively select a scale factor from ×1.0 and ×2.0 and CNN-based super-resolution is utilized for is the latter case. Compared with VTM-11.0 with new MCTF enabled, the simulation results show {-2.95%, -1.42%, -1.68%} and {-2.71%, -0.80%, -1.50%%} BD-rate savings for {Y, Cb, Cr} under RA and AI configurations, respectively.

# Introduction

The JVET-Y0070 [1] shows significant coding gains by using the Convolutional Neural Network (CNN)-based super-resolution. In addition, it has been demonstrated by JVET-Y0068 [2] that RPR encoder control at GOP level may provide a better coding performance. This contribution tests the combination of JVET-Y0070 and JVET-Y0068.

# Proposed method

## GOP level encoding resolution decision

In this proposal, the resampling factors ×2.0 (half size) and ×1.0 (original size) are selected at GOP level and the scale factor is applied on all pictures in the GOP. To determine the resolution, the first picture is downscaled to quarter resolution and then resampled to the original resolution. The PSNR is calculated between the original picture and the down-up scaled picture. The scale factor is selected according to a threshold which is adaptively varied based on the initial QP. The pre-defined threshold is the same as that in JVET-Y0068.

## CNN-based super-resolution

In this contribution, CNN-based super-resolution models are the same as JVET-Y0070 that separate models are used for Y, U, and V components, respectively. The up-sampling model for the luma component is illustrated in Fig. 1. The input to the model consists of three parts, i.e., the low-resolution reconstruction YLRrec, the luma component of prediction frame YLRpred, and the QP map. The QP map is a matrix filled with the QP value.

Fig. 1 The proposed model for luma up-sampling.

Fig. 2 shows the network structure of up-sampling models for the chroma components. To utilize the cross-component correlation, the luma component of reconstruction is taken as input. Specifically, the luma component YLRrec is first down-sampled to the same resolution as chroma components by a convolution layer with the stride of 2 (denoted by s2 in Fig. 2) for the 4:2:0 color format.

- Network structure for up-sampling U components.

- Network structure for up-sampling V components.

Fig. 2 The proposed network structures for up-sampling the U and V components, respectively.

## Inference

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | Framework: | LibTorch v1.8.1 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | luma up-sampling model for I slice: 2.7 M luma up-sampling model for B slice: 1.37M U up-sampling model: 1.44M V up-sampling model: 1.44M |
|  | Total Parameter Number | 6.95 M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 4 models in total: 26.51 MB |
|  | Multiply Accumulate (MAC) | 854 kMAC/pixel |
| Optional | Total Conv. Layers | 35 for up-sampling the luma, 37 for up-sampling the chroma |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | Whole frame |

## Training details

DIV2K [3] and BVI_DVC [4] datasets are used for training. The VTM-11.0 [5] + new MCTF [6] is utilized to compress these images with QPs {22, 27, 32, 37, 42}. In the context of RPR scheme, the training data will be down-sampled by 2x before encoding. Then, we use the decoded low-resolution images and the corresponding original images for training.

Table 2 Network information for the proposed model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.8 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 120 |
|  | Batch size: | 16 |
|  | Training time: | 64h/model |
|  | Training data information: | DIV2K, BVI_DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 + new MCTF, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

# Experimental results

The proposed CNN-based up-sampling filter is tested on top of VTM-11.0 + new MCTF, according to the common test conditions [7]. The QP values are {22, 27, 32, 37,42} for both anchor and test. The test results for RA and AI configurations are shown in Table 3 and Table 4, respectively.

Under RA configurations, the proposed method can bring {-2.95%, -1.42%, -1.68%} BD-rate changes on average for {Y, U, V} components, respectively.

Under AI configurations, the proposed method can bring {-2.71%, -0.80%, -1.50%%} BD-rate changes on average for {Y, U, V} components, respectively.

Table 3 Results of the proposed method for RA configurations (QPs are {22, 27, 32, 37, 42}).

|  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27,32,37,42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -8.03% | -9.59% | -8.79% | 182% | 65% |
|  | FoodMarket4 | -6.49% | 1.10% | -1.04% |  |  |
|  | Campfire | -0.40% | -0.07% | -0.24% |  |  |
| Class A2 4K | CatRobot1 | -1.48% | -1.41% | -0.49% | 187% | 89% |
|  | DaylightRoad2 | 0.00% | 0.00% | 0.00% |  |  |
|  | ParkRunning3 | -1.29% | 1.47% | 0.51% |  |  |
| Average on A1 |  | -4.97% | -2.86% | -3.36% | 182% | 77% |
| Average on A2 |  | -0.92% | 0.02% | 0.00% | 187% | 77% |
| overall |  | -2.95% | -1.42% | -1.68% | 184% | 77% |

Table 4 Results of the proposed method for AI configurations (QPs are {22, 27, 32, 37, 42}).

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-11.0 + New MCTF (QP 22, 27,32,37,42) |  |  |  |  |
|  |  | Y | U | V | EncT | DecT |
| Cass A1 4K | Tango2 | -5.11% | -4.85% | -5.08% | 329% | 76% |
|  | FoodMarket4 | -4.21% | -0.70% | -2.53% |  |  |
|  | Campfire | 0.00% | 0.00% | 0.00% |  |  |
| Class A2 4K | CatRobot1 | -0.92% | -0.92% | -0.69% | 219% | 87% |
|  | DaylightRoad2 | 0.00% | 0.00% | 0.00% |  |  |
|  | ParkRunning3 | -6.04% | 1.66% | -0.73% |  |  |
| Average on A1 |  | -3.11% | -1.85% | -2.54% | 329% | 76% |
| Average on A2 |  | -2.32% | 0.25% | -0.47% | 219% | 87% |
| overall |  | -2.71% | -0.80% | -1.50% | 274% | 81% |

# Conclusion

In this contribution, a combination of CNN-based super-resolution and adaptive GOP level encoding resolution decision is tested. The adaptive encoding resolution decision enables the encoder to select the encoding resolution between a full size and a half size. This method shows a promising coding performance.

# References

- C. Lin, Y. Li, K. Zhang, L. Zhang, “EE1-2.4: CNN-based Super Resolution for Video Coding Using Separate Networks for Chroma Components”, JVET-Y0070, Jan. 2022.

- J. Nam, S.Yoo, J. Lim, S.  Kim, E1-2.1-related: RPR encoder with multiple scale factors, JVET-Y0068, Jan. 2022.

- Timofte, R., Gu, S., Wu, J., Van Gool. “Ntire 2018 challenge on single image super-resolution: Methods and results”, In Proceedings of the IEEE conference on computer vision and pattern recognition workshops, 2018.

- Di Ma, Fan Zhang, David R. Bull, BVI-DVC: A Training Database for Deep Video Compression (https://data.bris.ac.uk/data/dataset/3hj4t64fkbrgn2ghwp9en4vhtn ).

- VTM-11.0 https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-11.0

- P. Wennersten, C. Hollmann, J. Ström, “GOP-based temporal filter improvements”, JVET-V0056, Apr. 2022.

- S. Liu, A. Segall, E. Alshina, R.-L. Liao, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-Y2016, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).