| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0099-v1 |
| --- | --- |

| Title: | AHG10: Deblocking in RDO and beta offset minus 2 for VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Enhorn, R. Sjöberg, J. Ström, L. Litwic | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

Deblocking can provide a significant subjective benefit for VVC. At the last meeting the deblocking parameters were changed to be aligned with ECM since they provided an objective benefit while keeping subjective quality. To further improve objective benefits with maintained subjective quality, this contribution proposes to enable deblocking in RDO in CTC for RA and LD in VTM. It is also proposed to allow for more deblocking in VTM than ECM by a less restrictive setting. The proponents claim that this provides an additional objective benefit in VTM together with deblocking in RDO. This is also claimed to be identical with the deblocking settings in the alternative configurations for VTM (high complexity setting).

The reported results from enabling deblocking in RDO for VTM are as follows:

RA: -0.51%Y, -0.87%U, -0.84%V

LDB: -0.27%Y, -0.90%U, -1.00%V

Enabling deblocking in RDO and with the less restrictive settings (tC offset set to 0 and beta offset set to -2) for VTM:

RA:   -0.56%Y,	-0.99%U, -0.89%V

LDB: -0.41%Y, -0.73%U, -0.96%V

The objective benefit is reported to result in 5% encoding time increase.

# Introduction

Deblocking in VVC allow for control of the deblocking strength by tC parameter and control of how much spatial activity can be present for deblocking to be applied by beta parameter. The tC parameter mainly controls the clipping which determines how much a deblocked sample can vary from the sample value before deblocking. The beta parameter is mainly used as a basis for thresholds that spatial activities are compared to and deblocking can only be applied if the spatial activities are below thresholds based on the beta parameter.

At the last meeting a change to the deblocking parameters was made to change both tC and beta offset parameters which is the current setting in VTM-16. For random access a tC offset of -5 and a beta offset of -2 is set for the intra picture and for inter pictures a further adjustment of tC offset with increasing level is made. For low-delay configurations both beta and tC offset are set to -3.

The VVC software is also configurable to apply deblocking in RDO which was proposed in [1]. When running the high complexity setting in VTM that was adopted at the last meeting [2] the proposed setting in this proposal was used, e.g. enabling deblocking in RDO and using a beta offset of -2.

# Proposal

To further improve objective performance with maintained subjective performance we propose to make the following changes to the VTM config files for random access and low-delay configurations [3]:

EncDbOpt: 1   (this enables deblocking in RDO)

DeblockingFilterBetaOffset_div2: -2 (this set beta parameter for luma)

DeblockingFilterCbBetaOffset_div2: -2 (this set beta parameter for chroma component Cb)

DeblockingFilterCrBetaOffset_div2: -2 (this set beta parameter for chroma component Cr)

DeblockingFilterTcOffset_div2: 0 (this set beta parameter for luma)

DeblockingFilterCbTcOffset_div2: 0 (this set beta parameter for chroma component Cb)

DeblockingFilterCrTcOffset_div2: 0 (this set beta parameter for chroma component Cr)

The GOP based parameter  tC offset in the random access config is also set to 0.

The reason to enable deblocking in RDO is that it can then naturally compete as other tools in VVC to improve coding efficiency. The deblocking parameter changes make the deblocking a bit less restricted than the current setting to make proper deblocking when needed, e.g. for more flat areas.

# Results

We provide results for VTM CTC [4]. A significant improvement to the objective performance can be noted. Most benefit comes from enabling deblocking in RDO. The encoding and decoding time numbers provided are not accurate. Separate test for RA for class C and D on same machine for anchor and test indicated 5% increase in encoding time which should be ok in comparison to the gain provided.

We have also verified the subjective quality on a wide range of sequences. Sometimes we see some slight benefit but mostly similar as the anchor.

## Deblocking in RDO enabled

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,72% | -1,19% | -0,98% | 107% | 95% |
| Class A2 | -0,56% | -0,65% | -0,74% | 107% | 92% |
| Class B | -0,52% | -1,03% | -0,82% | 104% | 94% |
| Class C | -0,32% | -0,61% | -0,82% | 106% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0,51% | -0,87% | -0,84% | 106% | 94% |
| Class D | -0,20% | -0,69% | -1,02% | 100% | 82% |
| Class F | -0,15% | -0,55% | -0,63% | 101% | 90% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,37% | -0,72% | -0,46% | 109% | 93% |
| Class C | -0,17% | -0,50% | -0,69% | 106% | 94% |
| Class E | -0,26% | -1,75% | -2,34% | 104% | 85% |
| Overall | -0,27% | -0,90% | -1,00% | 107% | 92% |
| Class D | -0,19% | -0,78% | -0,67% | 103% | 94% |
| Class F | -0,13% | -0,40% | -0,03% | 104% | 91% |

## Deblocking in RDO and beta offset -2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,77% | -1,50% | -1,36% | 116% | 106% |
| Class A2 | -0,63% | -1,06% | -0,85% | 114% | 104% |
| Class B | -0,55% | -1,16% | -0,86% | 113% | 105% |
| Class C | -0,36% | -0,34% | -0,60% | 113% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0,56% | -0,99% | -0,89% | 114% | 107% |
| Class D | -0,26% | -0,47% | -0,60% | 107% | 97% |
| Class F | -0,18% | 0,31% | 0,44% | 113% | 109% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0,49% | -0,78% | -0,83% | 104% | 97% |
| Class C | -0,28% | -0,15% | -0,23% | 99% | 88% |
| Class E | -0,44% | -1,42% | -2,17% | 99% | 91% |
| Overall | -0,41% | -0,73% | -0,96% | 101% | 93% |
| Class D | -0,38% | -1,15% | -0,94% | 101% | 93% |
| Class F | 0,01% | 0,35% | 0,01% | 101% | 99% |

# Conclusions

It this contribution we have shown that it can be beneficial to enable the config parameter EncDbOpt and set the deblocking beta offsets config parameters to -2.

We therefore suggest enabling the proposed configuration parameters in RA and LDB CTC for VTM.

# References

[1] N. Hu, V. Seregin, W-J Chien, M. Karczewicz, “Encoder optimization with deblocking filter”, JVET-M0428, Marrakech, January 2019

[2] D.Rusanovskyy, M.Karczewicz (Qualcomm), K.Andersson, R.Sjöberg, L.Litwic (Ericsson), P.Nikitin, G.Martin-Cocher (InterDigital), A.Wieckowski, J.Brandenburg, B.Bross (HHI), “AHG10: VTM encoder configurations for tests targeting improved coding performance”, JVET-Y0126, Meeting by Teleconference, January 2022

[3] VTM-16.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

[4] F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-Y2010, Meeting by Teleconference, January 2022

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).