| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0100-v1 |
| --- | --- |

| Title: | Non-EE2: Weighted Chroma Prediction (WCP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Hongqing Du, Zhenyao Zhang, YanZhuo Ma, Fuzheng Yang, Ming Li, Yang Liu | Email: | jyhuo@mail.xidian.edu.cn hqdu@stu.xidian.edu.cn zhyzhang@stu.xidian.edu.cn yzma@mail.xidian.edu.cn fzhyang@mail.xidian.edu.cn myron.li@oppo.com serena@oppo.com |
| Source: | Xidian University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

This contribution proposes a weighted chroma prediction (WCP) method in which predicted chroma samples of the current block are derived based on the weights and neighboring chroma samples. It is proposed that the weights are designed according to the difference between neighboring luma samples and the reconstructed luma samples of a coding block. The test results on top of ECM-4.0 are as follows:

AI(Y/U/V): -0.09%/-0.70%/-0.67%, EncT 98%, DecT 101%

Introduction

In ECM4.0, both cross-component linear model prediction(CCLM) and multi-model linear model prediction(MMLM) i.e., linear model(LM) approaches use correlation between luma and chroma components to obtain predicted chroma samples. When LM mode is used, the chroma samples are predicted from the reconstructed luma samples of co-located block as follows:

PredCxy=α×CurL'xy+β

where PredC indicates predicted chroma samples in a block and CurL indicates the reconstructed luma samples in the block. Parameters α and β are derived from neighboring reconstructed samples of the current block. For subsampled color format, downsampled reconstructed luma CurL' is necessary for LM modes to match size of chroma signal.

Proposed method

In this contribution, a weighted chroma prediction (WCP) method is proposed. Fig. 1 depicts the procedure of WCP. For each sample CurL'xy in the downsampled luma block, the luma difference vector DeltaL is first obtained according to the absolute value of the difference between CurL'xy and the neighboring luma vector; second, the normalized weight vector Weight is derived according to a nonlinear mapping model related to DeltaL; third, with the weight vector, the predicted chroma sample PredCxy can be obtained by performing vector multiplication based on the weight vector and the neighboring chroma vector.

Fig. 1 The procedure of weighted chroma prediction (WCP)

Specifically, the predicted chroma samples PredCxy with x = 0…W − 1, y = 0…H − 1 are derived as follows:

PredCxy=k=0K-1Weightxyk×RefCk

Experimental results

The proposed WCP method is implemented on ECM-4.0[1] as a new chroma prediction mode(WCP_IDX), tests are performed following test conditions specified in JVET-Y2017[2]. The results of All Intra configuration are summarized in the Table 1. More detail results can be found in the accompanying Excel tables.

Table 1. Test results of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -1.15% | -0.92% | 91% | 96% |
| Class A2 | -0.05% | -0.74% | -0.70% | 94% | 100% |
| Class B | -0.07% | -0.73% | -0.81% | 97% | 96% |
| Class C | -0.10% | -0.57% | -0.56% | 104% | 107% |
| Class E | -0.04% | -0.36% | -0.30% | 103% | 107% |
| Overall | -0.09% | -0.70% | -0.67% | 98% | 101% |
| Class D | -0.07% | -0.52% | -0.47% | 97% | 95% |
| Class F | -0.29% | -0.56% | -0.58% | 94% | 98% |

Conclusion

In this contribution, a weighted chroma prediction mode is proposed to improve the coding efficiency of chroma intra prediction. The contribution achieves BD-rate saving of {0.09% (Y), 0.70% (U), 0.67% (V)} for the AI configuration. The encoding and decoding time is similar to the anchor scheme.

References

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM.

[2] M. Karczewicz, Y. Yen, "Common Test Conditions and evaluation procedures for enhanced compression tool testing", JVET-Y2017, January 2022.

Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).