| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0101-v1 |
| --- | --- |

| Title: | AHG11: post-process filter based on fusion of CNN and transformer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tianhong Liu, Wenxue Cui, Chen Hui, Feng Jiang, Ying Gao, Shaowei Xie and Ping Wu | Email: | 20s103271@stu.hit.edu.cn wenxuecui@stu.hit.edu.cn 20b903013@stu.hit.edu.cn fjiang@hit.edu.cn gao.ying@zte.com.cn xie.shaowei@zte.com.cn ping.wu@zte.com.cn |
| Source: | Harbin Institute of Technology, ZTE Corporation |  |  |

_____________________________

# Abstract

This contribution presents a post-process filter based on fusion network, which combines convolutional neural network and transformer by channel-wise attention mechanism. The performance is evaluated using PSNR and MS-SSIM based DB-rate reduction in VVC Test Model 11.0 with Neural Network for Video Coding (NNVC) technology 1.0. It is reported that the average DB-rate reduction of class B, C, D is {-1.99%, -7.31%, -5.19%} on PSNR and {-1.41%, -5.86%, -3.38%} on MS-SSIM under random access configuration. It is also reported that the average DB-rate reduction of class F is {10.60%, 0.19%, 1.80%} on PSNR and	{-0.97%, -1.69%, 0.23%} on MS-SSIM under random access configuration. Due to time reasons, class A has not been fully tested, the result will be supplemented in the latter version.

# Introduction

In this contribution, a post-process filter based on the combination of CNN and transformer is reported, named DFNN. DFNN aims to learn effective residual information and improve the quality of the input lossy frame. DFNN is composed of several basic modules DFB to extract restoration features from the input frame. Each DFB includes a convolutional-based module LAB and a transformer-based module GAB to extract feature from different perspectives. After these two modules, DFB applies channel-wise attention to fuse their output features. Besides, DFNN uses frame QP as additional input information and is trained on BVI-DVC [2] dataset.

# Proposed method

## Network Architecture

The network architecture is shown in Fig. 1. It includes three parts: head, backbone and reconstruction. The head part consists of one 3×3 convolutional layer, which is used to extract the shallow features of the input frame. The backbone consists of M deep fusion blocks (DFBs) containing 6 residual blocks (RBs). The reconstruction part consists of one 3×3 convolutional layer. In addition, a residual connection is used between input lossy frame and output enhanced frame to learn global residual information.

Fig. 1 Architecture of DFNN

One of the inputs to DFNN is 3-channels lossy frame, of which UV components are resized to the same size as Y component, and another input is the QP map of the lossy frame. Thus, the input is a 4-channels tensor, and the output is a 3-channel tensor.

The architecture of proposed DFB is shown in Fig. 2. DFB includes RBs to pre-process input features, GAB as well as LAB comprised of convolutional layers and Swin Transformer blocks [1] respectively to extract restoration features from different perspectives, and linear layers to calculate channel wise fusion weight of GAB and LAB. The output of DFB is the sum of weighted LAB feature and GAB feature.

Fig. 2 Architecture of DFB.

Fig. 3 (a) Architecture of RB. (b) Architecture of LAB. (c) Architecture of GAB.

The architecture of the proposed residual block (RB) is shown in Fig. 3(a). Each RB has two 3×3 convolutional layers and a Parametric Rectified Linear Unit (PReLU) as activation function between these two layers. Residual connection is also applied to RB. LAB consists of three cascaded 3×3 convolutional layer and PReLU activation function. GAB consists of two shift window transformer blocks [1] in the middle. In order to reshape the input feature to be accessible by Swin Block, GAB uses a 4×4 convolutional layer with same stride to embed input feature, then it flattens width and height into one dimension. In addition, a layer normalization function is followed. After the process of Swin Block, reverse operations have to be applied, which contain layer normalization function, viewing operation to recover the dimension and pixel shuffle operation as up sample function.

## Training

The proposed network is trained with BVI-DVC [2] dataset. The number M of DFNN’s DFB block is set to 4. All the frames are compressed under the common test conditions (RA configuration and QP = {22, 27, 32, 37, 42}) with VTM-11.0. All frames are randomly cropped into 256*256 patches and applied with data augmentation operations. For training, the network uses weighted Charbonnier loss. the weight of YUV is set to 10:1:1. Since DFNN is a post-processing filter used after the VTM decoder, there is no need to transmit the network parameters from the encoder to the decoder through bitstream or generate them in the decoder side. More information related with the network architecture are reported in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3060 12GB |
|  | Framework: | PyTorch v1.10 |
|  | Number of GPUs per Task | 2 |
|  | Epoch: |  |
|  | Batch size: | 6 |
|  | Training time: | 35h |
|  | Training data information: | BVI-DVC |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0 CTC NNVC |
| Optional |  |  |
|  | Number of iterations | 200,000 |
|  | Patch size | 256*256 |
|  | Learning rate: | 2E-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Resize UV components to the same size of Y, normalize to [0, 1]. Concatenate frame qp map. |
|  | Other information: |  |

## Inference

Table 2 reports the network information at the inference stage. Class B, C, D, F are inferenced in whole frame format. Class A1, A2 are inferenced by 960*960 tiles with 32 overlapping pixels.

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | GPU: GeForce RTX 3060 12GB |
|  | Framework: | PyTorch v1.10 |
|  | Number of GPUs per Task | 1 |
|  | Total Parameter Number | 5.28M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (MB) | 25M |
|  | MAC (Giga) | 2528K/pixel |
| Optional |  |  |
|  | Total Conv. Layers | 70 |
|  | Total FC Layers | 44 |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size |  |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

# Experimental results

## Overall

Table 3 reports the BD-rates of the proposed filtering over VTM 11.0 NNVC 1.0

Table 3 BD-rate of DFNN over VTM-11.0 NNVC in RA configuration

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-1.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 | -1.54% | -6.75% | -2.47% | -1.45% | -5.30% | -1.35% |  |  |
| Class B | 0.02% | -5.22% | -3.65% | -0.84% | -4.59% | -2.56% |  |  |
| Class C | -1.91% | -8.13% | -5.53% | -1.40% | -6.47% | -3.67% |  |  |
| Overall |  |  |  |  |  |  |  |  |
| Class D | -4.09% | -8.58% | -6.39% | -1.99% | -6.53% | -3.91% |  |  |
| Class F | 10.60% | 0.19% | 1.80% | -0.97% | -1.69% | 0.23% |  |  |

## Class F research

It is reported in table 3 that the proposed network performs poorly on class F. Table 4 reports detail results on class F. Sequence SlideEditing performs worst in table 4, followed by SlideShow and BasketbalDrillText. Visible frames of these sequences are shown in fig. 4. There are lots of text textures in these sequences, especially in SlideEditing which performs worst. However, the dataset BVI-DVC on which DFNN is trained lacks text texture data. In the following research, data that full of text texture need to be added into training dataset to get more results.

Table 4 BD-rate of DFNN over VTM-11.0 NNVC in RA configuration on class F

|  | BD-rate5 q12345 (piecewise cubic) |  |  | BD-rate5 q12345 (piecewise cubic) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| BasketballDrillText | 4.94% | 7.75% | 5.46% | -2.30% | -1.05% | -0.27% |
| ArenaOfValor | -5.65% | -5.99% | -5.68% | -6.39% | -5.54% | -5.15% |
| SlideEditing | 32.28% | 3.19% | 2.27% | 4.50% | 3.10% | 2.97% |
| SlideShow | 10.84% | -4.19% | 5.16% | 0.31% | -3.28% | 3.39% |

Fig. 4 (a) 60th frame of SlideEditing. (b) 60th frame of SlideShow. (c) 60th frame of BasketballDrillText.

# References

- Z. Liu, Y. Lin, Y. Cao, H. Hu, Y. Wei, Z. Zhang, S. Lin, and B. Guo. Swin transformer: Hierarchical vision transformer using shifted windows. arXiv preprint arXiv:2103.14030, 2021.

- Di Ma, Fan Zhang, and David Bull, “BVI-DVC: A Training Database for Deep Video Compression”, arXiv:2003.13552, 2020.

# Patent rights declaration(s)

Harbin Institute of Technology may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)