| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0103-v1 |
| --- | --- |

| Title: | On ARMC improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Patrice Onno | Tel: Email: | 33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution presents some modifications of the Adaptive Merge Candidates list Reordering process (ARMC). The proposed method, applied after the first reordering process, reorders the list according to the distortion computed during the first ordering to target a more diverse list of Merge candidates for the final list. In addition, the merge derivation is adapted to this process to obtain additional coding gains. It is also reported that this particular merge derivation adaptation does not provide any gain when directly applied on top of ECM4.0.

Compared to ECM-4.0, the average BDR gains and runtimes reported in this contribution and combined with JVET-Z0102 are as follows:

-0.27%/-0.24%/-0.18%  - 101%, 102% for the RA configuration,

-0.47%/-0.58%/-0.70%  - 102%, 102% for the Low Delay B configuration.

# Problem Statement

In ECM-3.0, the Merge candidates with an adaptive reordering based on a template matching (TM) technique has been adopted. The reordering method was applied to regular Merge mode, template matching (TM) Merge mode, and affine Merge mode. The reordering was limited to a subgroup of candidates. The subgroup size is set to 5 for regular merge mode and TM Merge mode and to 3 for affine Merge mode.

In ECM-4.0, AMRC was updated and the reordering is applied to all Merge candidates. In addition, the Merge candidates derivation was changed by introducing many additional temporal and non adjacent positions. This modification multiplies the number of Merge positions by 2. Moreover, a first pass of ARMC on a list of temporal and on a list of non-adjacent candidates was added. Furthermore, in ECM-4.0, the motion vector threshold for the duplicate check for these subgroups of Merge candidates is different and is also different compared to the motion vector thresholds of the other Merge candidates. These specific tunings of independent orderings and specific motion vector thresholds are efficient as it creates a kind of diversity among the final list of Merge candidates in ECM-4.0 as some gains were observed for RA configuration compared to methods without these specific tunings.

In this contribution, we propose another way to control the diversity inside the Merge candidates set.

# Proposed method

## Diversity reordering

To create diversity inside the Merge candidates list, the proposed method looks for candidates which are too redundant in the rate-distortion (RD) sense. In the algorithm, a candidate is considered as redundant if the cost difference between a candidate and its predecessor is inferior to a lambda value e.g. |D1-D2| < λ, where D1 and D2 are the costs obtained during the first ARMC ordering and λ is the Lagrangian parameter used in the RD criterion at encoder side.

The proposed algorithm is defined as the following:

-Determine the minimum cost difference between a candidate and its predecessor among all candidates in the list

-If the minimum cost difference is superior or equal to λ, the list is considered diverse enough and the reordering stops.

-If this minimum cost difference is inferior to λ, the candidate is considered as redundant and it is moved at a further position in the list. This further position is the first position where the candidate is diverse enough compared to its predecessor.

-	The algorithm stops after a finite number of iterations (if the minimum cost difference is not inferior to λ).

This algorithm is applied to the Regular, TM and BM merge modes of ECM-4.0. A similar algorithm is applied to the Merge MMVD and sign MVD prediction methods which also use ARMC for the reordering.

The value of λ is set equal to the λ of the rate distortion criterion used to select the best merge candidate at the encoder side for low delay configuration and to the value λ corresponding to a another QP for Random Access configuration.

## Merge Mode modifications

The following modifications have been applied to regular Merge list:

- The same Motion vector threshold is used for Temporal, non-adjacent and other candidates.

-No ARMC reordering for sub-groups is performed

-Pairwise candidates are added after the first ARMC reordering for regular, TM and BM Merge modes.

# Results

The proposed modifications are implemented on top of the ECM-4.0 [1] and tested according to the common test conditions (CTC) for ECM-4.0 [2].

The modifications have been tested independently as summarized in the following table.

The diversity criterion includes also the modification described in JVET-Z0102.

Table 1 Summary of experiments

| Tests | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| A: JVET-Z0102 | -0.04% | -0.04% | -0.03% | 99% | 98% | -0.11% | -0.14% | -0.16% | 100% | 97% |
| B: JVET-Z0102 +Diversity | -0.12% | -0.18% | -0.11% | 101% | 100% | -0.31% | -0.45% | -0.43% | 102% | 97% |
| C: Merge modifications | 0.24% | 0.18% | 0.26% | 101% | 99% | 0.33% | 0.55% | 0.56% | 103% | 96% |
| D: JVET-Z0102+ Diversity+ Merge modifications | -0.27% | -0.24% | -0.18% | 101% | 102% | -0.47% | -0.58% | -0.70% | 102% | 102% |

The test A is a reminder of the result of the contribution JVET-Z0102 where zero candidates are excluded for the ARMC process.

The test B regarding the diversity reordering algorithm gives about -0.1% BDR gain for RA configuration and significant gains for LDB configuration with around -0.2% BDR compared to the method proposed in JVET-Z0102.

This gain is interesting as the proposed method does not change the first candidate of Merge list which is the most selected candidate.

As per the test C, the merge mode modification shows only losses on top of ECM4.0 as there is no control of the diversity in the final list of merge candidates even if ARMC reordering orders the candidates according to their template distortion.

Yet, the combination of these merge modifications and the proposed diversity reordering algorithm in test D shows significant gains with about -0.3% and -0.5% BDR gains for respectively RA and LDB configuration.

The result details per classes of this last test D are reported in the following table.

Table 2 Coding performances of the proposed modification corresponding to test D.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0_ANCHOR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.31% | -0.25% | 101% | 99% |
| Class A2 | -0.19% | -0.20% | -0.17% | 102% | 100% |
| Class B | -0.29% | -0.22% | -0.15% | 102% | 100% |
| Class C | -0.30% | -0.25% | -0.17% | 101% | 106% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.24% | -0.18% | 101% | 102% |
| Class D | -0.22% | -0.09% | -0.15% | 98% | 105% |
| Class F | -0.07% | -0.27% | -0.15% | 99% | 100% |
| Class TGM |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |
|  | Over ECM-4.0_ANCHOR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.39% | -0.46% | -0.79% | 102% | 101% |
| Class C | -0.50% | -0.44% | -0.82% | 101% | 106% |
| Class E | -0.57% | -0.96% | -0.38% | 102% | 100% |
| Overall | -0.47% | -0.58% | -0.70% | 102% | 102% |
| Class D | -0.50% | -0.37% | -0.32% | 104% | 110% |
| Class F | -0.18% | 0.17% | -0.82% | 102% | 101% |
| Class TGM |  |  |  |  |  |

# Conclusion

In this contribution, some modifications regarding the ARMC process are proposed and dedicated merge derivation modifications are added. The proposed diversity reordering algorithm is applied after the first ARMC reordering process to obtain a more diverse, list of Merge candidates in a RD-sense.

The results of test D in the section above show a noticeable coding performance both in RA and the LDB configuration.

We suggest considering the proposed changes for investigation in the next round of EE2 in the ECM-5.0.

# References

- ECM-4.0 reference software: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-4.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).