| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0109 |
| --- | --- |

| Title: | Evaluation of objective quality metrics on HEVC, VVC and AV1 contents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Joel Jung José Gonzalez Lopez Xiang Li Shan Liu | Tel: Email: | joeljung@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution evaluates objective quality metrics on a recent database provided by Bristol University, with both HD and UHD content, encoded with recent encoders (HEVC, AV1, VVC) and with mean opinion scores (MOS) obtained from subjective experiments made in laboratory. The results are reported as typical correlations and average errors, compared to the MOS. Classical and learning-based metrics have been evaluated. It is reported that VMAF and AVQT are the best performing metrics, far above other usual metrics, especially the PSNR. More detailed results are provided, analyzing metrics’ efficiency according to the spatial resolution, to the encoder used, to the spatial and temporal information level, and to the bitrate range.

# Introduction

The evaluation of objective metrics is largely addressed in the literature, especially when direct rating scales are used to obtain mean opinion scores (MOS), allowing the computation of classical correlations (Pearson, Spearman, Kendall). In AG5, similar interesting preliminary results were presented in m57993 [m57993], providing correlation results for three public datasets. With a slightly different goal, the AG5 contribution m58816 [m58816] and m59383 [m59383] focus on the question “can objective metrics make correct decisions when comparing an anchor to a proposal”, using a subjective methodology with an indirect scale, and so deriving correct decisions rates according to a comparative mean opinion score (CMOS).

This contribution focuses on the ability to report correct quality levels, using a subjective methodology with a direct scale. Given that objective metrics efficiency is sensitive to the dataset, type of encoder used, range of bitrates, etc., especially since machine learning-based techniques have been introduced, we pay particular attention to use one of the most recent dataset publicly available. It is using HEVC, AV1 and VVC, at HD and UHD resolutions. A detailed analysis is performed based on the spatial resolution, on the encoder used, on the spatial and temporal information level, and on the bitrate.

The goal of this contribution is not to advertise any metric or any category of metric, but rather to investigate how much we can rely on them, under different practical scenarios, close to JVET needs.

# Subjective test methodology and content

The BVI-CC dataset [BRI1][BRI2] has been selected for this study because it is one of the most recent dataset available, considering recent encoders, and adopting a reliable subjective evaluation methodology. This ensures a reliable evaluation of the objective metrics, and conclusions more in line with the recent coding standardization context. The dataset is provided by Bristol University. It is made of 18 sequences, 9 of them in UHD native resolution, 9 of them in full-HD downscaled resolution, using Lanczos-3 filter). All sequences are 60 fps, 10 bits. The processed video sequences (PVS) were obtained with three encoders: HEVC (HM16.18), VVC (VTM4.01) and AV1 (0.1.0-9647-ga6fa0877f), all in random access main 10 profile, and at four different rate points.

The subjective experiments were performed in laboratory, following the different requirements of ITU-R BT.500-12 for experimental setup, such as room environment, monitor characteristics, viewing distance, etc. A Double Stimulus Continuous Quality Scale (DSCQS) methodology has been used, with a direct rating scale ranging from 0 to 100. A total of 62 subjects took part to the tests.

# Objective metrics evaluation methodology

The BVI dataset uses a DSCQQ methodology with a 0-100 rating scale. It corresponds to a direct-scaling approach, that allows to compute classical indicators. The following indicators were computed:

- Pearson linear correlation

- Spearman rank order correlation

- Kendall correlation

- Root mean square error, after applying a linear regression of the metrics’ scores, to fit with the 0-100 subjective scale.

# Evaluated objective metrics

13 full-reference metrics are evaluated:

- PSNR, SSIM, MS-SSIM and VMAF (version 1.3.14, with v0.6.1 model for HD content, and v0.6.1_4k model for UHD content).

- 3SSIM (Three Component SSIM) [3SSIM]. It is computing a weighted average of SSIM for three different categories: edges, textures and smooth regions.

- VQM [VQM]: DCT-based Video Quality Metric, an 8x8 DCT is used to get the frequencies in the image, the DCT coefficients are converted to local contrast, further converted to just noticeable differences to derive the score.

- VIF (Visual Information Fidelity) [VIF]. It is based on natural scene statistics and the notion of image information extracted by the human visual system.

- WaDiQAM (Weighted Average Deep Image QuAlity Measure) [WADIQAM]. It is a deep neural network-based approach for image quality assessment. The network is trained end-to-end and comprises ten convolutional layers and five pooling layers for feature extraction, and two fully connected layers for regression.

- LPIPS (Learned Perceptual Image Patch Similarity) [LPIPS], and two variants, LPIPS modified with a squeezeNet network, and LPIPS modified with a VGG network. LPIPS is a family of deep learning based objective metrics, that evaluates the similarity of an image with a reference image, rather than the quality itself.

- AVQT (Advanced Video Quality Tool) [AVQT]

- DISTS (Deep Image Structure and Texture Similarity) [DISTS]. It is a full-reference image quality assessment model. It is based on a CNN that combines texture similarity spatial averages and structure similarity. It is designed to provide explicit tolerance to texture resampling, and insensitivity to geometric transformations.

None of the evaluated metrics was re-trained. The metrics were applied on each frame of the sequence, and the score of each frame averaged with equal weight.

# Experimental results

## Full dataset

Figure 1 shows the Pearson, Spearman and Kendall correlations for each metric, ordered from left to right by decreasing correlation, and figure 2 shows the RMSE, ordered by decreasing accuracy. HD and UHD content are considered. The following observations can be made:

- For all indicators, the two best metrics are AVQT and VMAF, offering good correlations (PLCC of 0.83 and 0.84 respectively), and reasonable RMSE (of 7.13 and 7.04 respectively on a 100-grade scale).

- All other metrics have correlations below 0.8, illustrating the difficulty of the metrics to report an accurate overall quality level. Only DISTS gets close with 0.77, without having been retrained on similar kind of content.

- The PSNR has a very poor correlation, and a higher error level than most of the other metrics.

| Figure 1: correlation between MOS and metrics on HD and UHD content, full reference metrics | Figure 2: RMSE between MOS and metrics on HD and UHD content, full reference metrics |
| --- | --- |

## Split by resolution

In this section, the dataset is split into two parts, based on the resolution (HD, UHD). Figure 3 and 4 show the Pearson correlation and Spearman rank order correlation for each metric, for each resolution separately, and figure 5 shows the RMSE for each metric. The following observations can be made:

- For all performance indicators, the best metrics are VMAF and AVQT, followed by DISTS.

- There is no significant difference for metrics, between HD and UHD, for the best performing metrics. There is a difference, metrics facing more difficulties with UHD content, for the LPIPS family, probably because it has not been trained on such resolutions, and for the PSNR and SSIM.

- The PSNR has a poor correlation and a quite high RMSE compared to most of the other metrics.

| Figure 3: Pearson correlation between MOS score and metrics, by resolution | Figure 4: Spearman rank order between MOS and metrics, by resolution |
| --- | --- |

Figure 5: RMSE between MOS and metrics, by resolution

## Split by encoder

In this section, the dataset is split into three categories, each category corresponding to an encoder, among AV1, VTM, and HM. HD and UHD content is considered. Figure 6 shows the Pearson correlation for each metric, for each encoder separately, and figure 7 shows the RMSE for each metric. The following observations can be made:

- VMAF and AVQT are the best metrics for all encoders.

- DISTS comes right after in the third position, but with a significantly lower correlation for AV1 and HM.

- MS-SSIM has a correct correlation only with VTM.

- For VTM, several metrics have a correlation above 0.8, and VMAF, AVQT, MS-SSIM and DISTS are very reliable, with correlations of 0.93, 0.90, 0.84 and 0.89 respectively.

- For HM, VMAF, AVQT and DISTS are the only reliable metrics.

- For AV1, while AVQT is the best metric, significantly outperforming VMAF and DISTS, none of the metrics have sufficient correlation to be considered as reliable. The quality is more difficult to predict by metrics for AV1 than for other encoders.

- The correlation of the PSNR is too low, even for VTM.

| Figure 6: Pearson correlation between MOS score and metrics on ALL content, by encoder | Figure 7: RMSE between MOS and metrics on ALL content, by encoder |
| --- | --- |

## Split by spatial information

In this section, the dataset is split into two parts, based on the level of spatial information. Only the HD content is considered. The spatial information (SI), described in ITU-T P.910 Recommendation, indicates the amount of spatial details in a picture. It is based on the Sobel filter: each luminance frame is first filtered with the Sobel filter. The standard deviation over the pixels in each Sobel-filtered frame is computed, for each frame. The maximum value is chosen to represent the spatial information content of the scene., It is computed with the implementation from [SITI].

The selected threshold between the two classes is 60. As show in figure 8, the sequences classified as low SI are S11, S14, S17, S18, S19, and the sequences classified as high SI are S12, S13, S15, S16.

Figure 8: spatial information of source content (HD)

Figures 9 and 10 show the Pearson correlations and RMSE of each metric, split by SI class. The following observations can be made:

- The metrics are more reliable for low SI than for high SI. Given that the human visual system (HVS) is less sensitive in textured areas than in smooth areas (well-known texture masking effect, also called entropy masking by Watson in 1997 [WAT97]), a possible explanation could be that human scores, MOS, do not take into account artifacts in textured areas, while these artifacts would be counted by objective metrics, not handling properly the texture masking effect.

- At low SI, the best metrics are VMAF, AVQT and DISTS. They are reliable with correlations of 0.89, 0.89 and 0.89 respectively.

- At high TI, VMAF and AVQT, as well as VIF, have a higher correlation than other metrics but none of them can be considered as reliable for this class of content. DISTS is not reliable either. This might result from the dataset used to train DISTS.

| Figure 9: Pearson correlation between MOS and metrics on HD content, by SI | Figure 10: RMSE between MOS and metrics on HD content, by SI |
| --- | --- |

## Split by temporal information

In this section, the dataset is split into two parts, based on the level of temporal information. Only the HD content is considered. The temporal information (TI), described in ITU-T P.910 Recommendation, is based on motion difference feature, the difference between the luminance pixel values at the same location in space for consecutive frames.

The measure of temporal information (TI) is computed as the maximum over time of the standard deviation of Mn. It is computed with the implementation from [SITI]. The threshold between the two classes is 17. As show in figure 11, the sequences classified as low TI are S11, S12, S13, S17, S18, and the sequences classified as high TI are S14, S15, S16, S19.

Figure 11: spatial information of source content (HD)

Figures 12 and 13 show the Pearson correlations and RMSE of each metric, split by TI class. The following observations can be made:

- Metrics are more reliable for high TI than for low TI. The HVS properties reduces the sensitivity in fast moving areas (temporal masking effect). With a similar reasoning as for SI, we would suppose that metrics count temporal artifacts, and that the correlation would be lower at high TI. This is not observed however, and we currently have no valid explanation of this result.

- At high TI, AVQT, VMAF, MS-SSIM and DISTS are reliable, with very good correlations of 0.96, 0.96, 0.93 and 0.94 respectively. Several other metrics have a reasonable correlation.

- At low TI, only AVQT has a sufficient correlation of 0.82.

| Figure 12: Pearson correlation between MOS and metrics on HD content, by TI | Figure 13: RMSE between MOS and metrics on HD content, by TI |
| --- | --- |

## Split by bitrate

In this section, the dataset is split into four parts, based on the bitrate, more precisely based on the quantization level. Only the HD content is considered. Figures 15 and 16 show the Pearson correlations and RMSE of each metric, split by bitrate class. The following observations can be made:

- As a general trend, metrics are more efficient at low-bitrate (R1) -except VMAF-, and less efficient at high bitrate, considering both Pearson and RMSE indicators.

- Again, we can observe that AVQT and VMAF are quite robust, with small efficiency differences from one rate point to another. This is not the case of DISTS (2nd performer for R1) and LPIPS family. This set of DNN based metrics are generally not robust, and most likely were not trained on high bitrates.

| Figure 15: Pearson correlation between MOS and metrics on HD content, by bitrate | Figure 16: RMSE between MOS and metrics on HD content, by bitrate |
| --- | --- |

# Discussion and conclusions

According to the different evaluations performed in this contribution, VMAF and AVQT are by far the best performers. They are robust in the sense that they are consistently providing the highest correlations and lowest RMSE, no matter the tested configuration. Their correlation is around 0.9 for VTM, which is an excellent correlation in this specific context. DISTS also provides good correlation in several situations but seems a bit more sensitive to changes of conditions. In contrary, the PSNR is always among the worst metrics, for all tested conditions. In particular, it is one of the few metrics with a correlation below 0.8 for VTM.

Additional observations are that metrics are generally more reliable at low SI and high TI, and that full reference metrics are not reliable with AV1.

As a step forward, a simple recommendation would be to add metrics like VMAF to the JVET result template.

Another step, that could be conducted by AG5, would be to organize a large-scale viewing session, applying a viewing methodology with a direct scale (ACR, extended ACR, etc.), based on representative HRCs (source content), and relevant PVS (processed video sequences) using recent VTM software. This would provide a large dataset with MOS, perfectly aligned with JVET needs, and would allow to train several machine learning-based metrics (including VMAF) and most likely achieve much higher level of correlation for several of them, making decisions based on objective metrics more reliable.

# References

[m57993]		L. Yu,  Y. Ye, S. Wang, Z. Wang, “Investigation of learning-based video quality metrics for 2D video based on three public datasets”, ISO/IEC JTC 1/SC 29/AG 5 m57993, Online – October 2021.

[m58816]		J. Jung, G. Teniou, X. Li, S. Liu, "Evaluation of 15 objective quality metrics on JVET 2D contents", ISO/IEC JTC1/SC29/AG5 M58816, January 2022, Virtual.

[m59383] 		J. Jung, J.G. Lopez, X. Li, S. Liu, “Evaluation of 17 objective quality metrics on JVET contents”, ISO/IEC JTC1/SC29/AG5 M59383, April 2022, Virtual.

[WADIQAM]	S. Bosse, D. Maniry, K. Müller, T. Wiegand and W. Samek, "Deep Neural Networks for No-Reference and Full-Reference Image Quality Assessment," in IEEE Transactions on Image Processing, vol. 27, no. 1, pp. 206-219, Jan. 2018, doi: 10.1109/TIP.2017.2760518.

[REV] 			J. Jung, M. Wien, V. Baroncini, Guidelines for remote experts viewing sessions, ISO/IEC JTC 1/SC 29/AG 5 N00040 Online, October 2021.

[VQM]			F. Xiao, “DCT-based Video Quality Evaluation”, final project of the EE392J, 2000.

[AVQT]		https://developer.apple.com/videos/play/wwdc2021/10145/

[VIF]				H. Sheikh, A. Bovik, "Image Information and Visual Quality". IEEE Transactions on Image Processing. 15 (2): 430–444.

[LPIPS]		R. Zhang, P. Isola, A.A. Efros, E. Shechtman, O. Wang, “The Unreasonable Effectiveness of Deep Features as a Perceptual Metric”, CVPR 2018.

[DISTS]		K. Ding, K. Ma, S. Wang, E.P. Simoncelli, Image Quality Assessment: Unifying Structure and Texture Similarity, IEEE Transactions on Pattern Analysis and Machine Intelligence, 2021, 10.48550/arXiv.2004.07728.

[3SSIM]		C. Li, A. Bovik, Content-Weighted Video Quality Assessment Using a Three-Component Image Model. Journal of Electronic Imaging. 29. 10.1117/1.3267087.

[SITI]			https://github.com/Telecommunication-Telemedia-Assessment/SITI.

[BRI]			A. V. Katsenou, F. Zhang, M. Afonso, and D. R. Bull, “A Subjective Comparison of AV1 and HEVC for Adaptive Video Streaming”, 2019 IEEE International Conference on Image Processing (ICIP), Taipei, Taiwan, 2019, pp. 4145-4149

[BRI2]			F. Zhang, A. V. Katsenou, M. Afonso, G. Dimitrov and D. R. Bull, “Comparing VVC, HEVC and AV1 using Objective and Subjective Assessments”, arXiv:2003. 10282 [eess.IV], 2020.

[WAT97]		A. B. Watson, R. Borthwick, and M. Taylor, “Image quality and entropy masking,” in Human Vision, Visual Processing, and Digital Display VIII, 3016, (San Jose, CA, USA), 1997.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.