| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0110-v1 |
| --- | --- |

| Title: | On low delay configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Li Guichun Li Shan Liu | Tel: Email: | xlxiangli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, the coding efficiency and coding speed by low delay B (LDB) and low delay P (LDP) are compared and reported:

- LDB is 6+% more efficient (luma) than LDP with 29% longer runtime under VTM CTC. When only 1 reference picture is used for LDB and LDP, the average luma efficiency and runtime gaps are reduced to 3.61% and 21%, respectively. If a coding tool (GPM) which cannot be applied in P picture is disabled, the efficiency and runtime gap are further reduced to 2.06% and 15%, respectively.

- LDB and LDP with multiple reference pictures are much more efficient than those with single reference picture. The luma efficiency gaps of 20.36% and 16.37% by LDB and LDP are reported for 4 reference pictures vs 1 reference picture.

- LDP with 4 reference pictures is averagely 10.54% (luma) better than LDB with 1 reference picture with 11% less runtime.

# Introduction

In this contribution, the coding efficiency and coding speed by low delay B (LDB) and low delay P configurations are compared under different conditions.

# Simulation results

In this section, the performance of LDB and LDP with one single reference picture is studied. The 1-ref test was conducted by modifying active reference number to 1 in VTM configuration files. Please note that all other settings including quantization hierarchy and GOP structure were kept unchanged.

## LDP-4ref vs LDB-4ref

The table below compares the results of LDP with 4 reference pictures over LDB with 4 reference pictures. Both LDP and LDB are VTM CTC settings. As expected, LDB is more efficient than LDP though the runtime is much higher.

|  | VTM-16.0 LDP Over VTM-16.0 LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 7.40% | 9.60% | 9.42% | 71% | 99% |
| Class C | 5.86% | 7.71% | 7.81% | 72% | 102% |
| Class E | 6.84% | 5.85% | 5.80% | 71% | 99% |
| Overall | 6.75% | 8.03% | 7.98% | 71% | 100% |
| Class D | 6.22% | 7.46% | 7.73% | 73% | 99% |
| Class F | 5.17% | 6.56% | 6.57% | 81% | 101% |

## LDP-1ref vs LDB-1ref

In this section, LDP with 1 reference picture is compared to LDB with 1 reference picture. Interestingly, the coding efficiency gap is close to half of that of 4-reference case. From the results, the number of reference pictures has a big impact on the coding efficiency.

|  | VTM-16.0-1ref LDP Over VTM-16.0-1ref LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 3.46% | 4.85% | 4.38% | 79% | 102% |
| Class C | 2.95% | 3.94% | 4.34% | 79% | 101% |
| Class E | 4.75% | 4.88% | 4.16% | 79% | 99% |
| Overall | 3.61% | 4.55% | 4.31% | 79% | 101% |
| Class D | 2.42% | 3.71% | 3.51% | 81% | 103% |
| Class F | 1.96% | 2.88% | 2.55% | 87% | 101% |

## LDP-1ref vs LDB-1ref-GPM0

It should be noted that the comparison in section 2.2 is not very fair if the focus is on the efficiency difference between B and P pictures. In fact, GPM has a big impact and this tool cannot be used in LDP case. In this section, the performance of single reference picture with GPM off is further tested. As shown below, the efficiency gap between LDP and LDB with single reference picture and GPM off is 2.06% on average with 15% encoding time difference.

|  | VTM-16.0-1ref-geo0 LDP Over VTM-16.0-1ref-geo0 LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.53% | 3.61% | 3.12% | 85% | 96% |
| Class C | 1.44% | 2.01% | 1.81% | 87% | 98% |
| Class E | 2.11% | 3.24% | 2.62% | 84% | 97% |
| Overall | 2.06% | 2.98% | 2.56% | 85% | 97% |
| Class D | 1.03% | 1.24% | 1.61% | 89% | 96% |
| Class F | 1.40% | 2.58% | 2.48% | 92% | 100% |

## LDB-1ref vs LDB-4ref

The table below shows the results of LDB with 1 reference picture over LDB with 4 reference pictures (CTC setting). It can be observed that 4 reference pictures are much more efficient than single reference picture though the encoding time of single reference picture is only 80% of that of 4 reference pictures.

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-16.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 18.55% | 30.02% | 34.58% | 79% | 100% |
| Class C | 25.43% | 32.75% | 32.06% | 83% | 101% |
| Class E | 16.64% | 15.40% | 16.43% | 76% | 100% |
| Overall | 20.36% | 27.27% | 29.20% | 80% | 100% |
| Class D | 37.71% | 53.66% | 60.12% | 86% | 98% |
| Class F | 19.42% | 24.68% | 24.74% | 88% | 102% |

## LDP-1ref vs LDP-4ref

The table below shows the results of LDP with 1 reference picture over LDP with 4 reference pictures (CTC setting). Similar to the case of LDB, 4 reference pictures are much more efficient than single reference picture. But for LDP, the runtime saving is relatively not that significant.

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-16.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 13.93% | 24.19% | 28.15% | 88% | 102% |
| Class C | 21.93% | 28.07% | 27.66% | 93% | 99% |
| Class E | 14.46% | 14.53% | 14.91% | 85% | 100% |
| Overall | 16.73% | 23.07% | 24.68% | 89% | 101% |
| Class D | 31.35% | 47.74% | 51.79% | 96% | 102% |
| Class F | 15.73% | 20.21% | 19.87% | 95% | 102% |

## LDP-5ref vs LDB-1ref

It is well recognized that LDB is much more efficient than LDP when testing conditions are the same. In this section, different numbers of reference pictures by LDB and LDP are compared. In the table below, results of LDP with 4 reference pictures over LDB with 1 reference picture are shown. It is observed that LDP with 4 reference pictures is averagely 10+% better than LDB with 1 reference picture and more than 10% faster. In this regard, LDP with more reference pictures seems a better choice for low complexity use cases.

|  | VTM-16.0 LDP Over VTM-16.0-1ref LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -8.60% | -13.44% | -15.02% | 89% | 100% |
| Class C | -14.58% | -17.62% | -17.01% | 86% | 102% |
| Class E | -8.37% | -8.35% | -9.22% | 94% | 99% |
| Overall | -10.54% | -13.56% | -14.24% | 89% | 100% |
| Class D | -17.57% | -21.58% | -22.14% | 85% | 101% |
| Class F | -11.25% | -13.52% | -13.48% | 91% | 98% |

# Conclusions

In this contribution, the coding efficiency and coding speed by LDB and LDP are compared. It is confirmed that LDB is more efficient than LDP under VTM CTC. However, when only 1 reference picture is allowed and GPM (cannot be applied in P pictures) are further disabled, the efficiency gap becomes much smaller. It is further reported that multiple reference pictures have significant efficiency impact on both LDB and LDP. The gaps between using 4 reference pictures and 1 reference picture is 16+%. Using LDP with 4 reference pictures shows 10+% luma gain over LDB with 1 reference with about 10% less encoding time. It is proposed to use LDP instead of LDB for low delay and low-complexity test.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.