| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0111-v2 |
| --- | --- |

| Title: | Adaptively bypass affine ME in VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Kuang Xiang Li Guichun Li Shan Liu | Tel: Email: | xlxiangli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, an encoder only method is proposed to adaptively bypass affine ME. Following results are reported

- Option A: 0.24%/0.15%/0.14% loss with 94% encoding time for RA, 0.25%/0.29%/0.11% loss with 91% encoding time LDB, 0.21%/0.26%/0.30% loss with 94% encoding time LDP

- Option B: 0.21%/0.14%/0/14% loss with 95% encoding time for RA, 0.19%/0.22%/-0.06% loss with 94% encoding time LDB, 0.13%/0.12%/0.05% loss with 97% encoding time LDP

# Introduction

In this contribution, an encoder only method is proposed to adaptively bypass affine ME. The basic idea is as follows. Before performing affine ME for a CU, the coding modes of its five spatial neighbours (above, left, above-right, left-bottom, above-left) are checked. If the number of available neighbours is greater than or equal to 4 and none of them are coded as affine, affine ME is bypassed.

To further speedup while keeping coding efficiency, two additional conditions are added.

- If a CU is no larger than 16x16, affine ME is not bypassed.

- If the best mode for a CU is affine merge so far, and the current picture does not have symmetric reference pair (SMVD condition) or the absolute temporal distance between the current picture and SMVD reference is larger than 1, affine ME is not bypassed.

# Simulation results

Two options are tested in this section under VTM CTC [1].

- Option A: The number of neighbours in affine mode (i.e., excluding SbTmvp) is checked

- Option B: The number of neighbours with affine flag on (i.e., including SbTmvp) is checked.

In general, option A bypasses more affine ME, showing faster speed with bigger loss.

## Option A

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.21% | 0.20% | 93% | 100% |
| Class A2 | 0.37% | 0.11% | 0.12% | 93% | 100% |
| Class B | 0.22% | 0.17% | 0.12% | 93% | 100% |
| Class C | 0.18% | 0.11% | 0.12% | 95% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.24% | 0.15% | 0.14% | 94% | 100% |
| Class D | 0.15% | 0.00% | -0.04% | 97% | 100% |
| Class F | 0.20% | 0.15% | 0.24% | 96% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | 0.18% | 0.09% | 91% | 101% |
| Class C | 0.27% | 0.44% | 0.05% | 94% | 106% |
| Class E | 0.38% | 0.28% | 0.23% | 87% | 102% |
| Overall | 0.25% | 0.29% | 0.11% | 91% | 103% |
| Class D | 0.17% | 0.00% | 0.32% | 97% | 99% |
| Class F | 0.32% | 0.52% | 0.29% | 92% | 99% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.18% | 0.22% | 0.43% | 94% | 101% |
| Class C | 0.18% | 0.31% | -0.01% | 96% | 100% |
| Class E | 0.30% | 0.27% | 0.51% | 91% | 100% |
| Overall | 0.21% | 0.26% | 0.30% | 94% | 100% |
| Class D | 0.20% | 0.22% | 0.50% | 98% | 101% |
| Class F | 0.31% | 0.44% | 0.24% | 94% | 103% |

## Option B

Comparing with option A, smaller loss is observed by option B, especially for low delay cases.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.21% | 0.21% | 0.16% | 94% | 100% |
| Class A2 | 0.32% | 0.18% | 0.20% | 94% | 100% |
| Class B | 0.20% | 0.15% | 0.17% | 94% | 100% |
| Class C | 0.15% | 0.03% | 0.03% | 97% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.21% | 0.14% | 0.14% | 95% | 100% |
| Class D | 0.15% | -0.05% | -0.02% | 98% | 100% |
| Class F | 0.18% | 0.11% | 0.16% | 98% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | 0.28% | -0.07% | 94% | 101% |
| Class C | 0.18% | 0.16% | -0.08% | 96% | 106% |
| Class E | 0.28% | 0.19% | -0.03% | 93% | 103% |
| Overall | 0.19% | 0.22% | -0.06% | 94% | 103% |
| Class D | 0.16% | 0.17% | 0.56% | 98% | 99% |
| Class F | 0.24% | 0.57% | 0.32% | 95% | 100% |
|  |  |  | Low delay P Main10 |  |  |
|  |  |  | Over VTM-16.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | 0.14% | 0.24% | 96% | 102% |
| Class C | 0.08% | 0.32% | -0.31% | 98% | 100% |
| Class E | 0.19% | -0.17% | 0.22% | 96% | 96% |
| Overall | 0.13% | 0.12% | 0.05% | 97% | 100% |
| Class D | 0.10% | 0.54% | 0.10% | 99% | 101% |
| Class F | 0.25% | 0.58% | 0.37% | 97% | 97% |

# Conclusions

In this contribution, an encoder only method is proposed to adaptively bypass affine ME. Two options are provided with different trade off. It is proposed to adopt one option into VTM software.

# Reference

[1] F. Bossen, X. Li, V. Seregin, K. Sharman, K. Sühring, “VTM common test conditions and software reference configurations for SDR video”, JVET-Y2010.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.