| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0114-v1 |
| --- | --- |

| Title: | AHG7: low delay configuration for cloud gaming |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | S. Puri, P. Le Guyadec, K. Naser, G. Martin-Cocher, T. Poirier | Email: | first.last@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution proposes a new low delay configuration (named LDB-1Ref-asymmetric) where the number of reference frames is restricted to 1 frame per list, and reference frame list is modified such that reference list 0 refers to the previous picture in the GOP and reference list 1 refers to the picture at the start of the GOP.

This configuration is proposed as a refinement of the LLCC baseline configuration proposed during the last meeting (JVET-Y0043) where it was agreed to restrict the number of reference frames to 2 in LD-B configuration.

Following set of tests are performed on one intra period. Test 1 and 2 compares the LDB-2Ref and LDB-1Ref (named LDB-1Ref-symmetric) configuration with the proposed LDB-1Ref-asymmetric configuration for class F, TGM and Gaming. Test 3 and 4 compares the proposed LDB-1Ref-asymmetric with LDP-4Ref and LDB-2Ref respectively.

Following results are reported for various tests conducted on 1IP:

Test 1: (LDB-2Ref vs LDB-1Ref-asymmetric)   0.71%,  0.85%,  1.06%,  94%,  103%
Test 2: (LDB-1Ref vs LDB-1Ref-asymmetric) -7.19%, -8.84%, -8.77%, 102%, 110%
Test 3: (LDP-4Ref vs LDB-1Ref-asymmetric)   1.95%,  1.50%,   1.53%, 110%, 103%
Test 4: (LDP-2Ref vs LDB-1Ref-asymmetric) -2.00%, -3.11%,  -3.04%, 120%, 106%

It is asserted that the proposed LDB-1Ref-asymmetric configuration provides a better trade-off between the LDP-4Ref and LDB-2Ref configuration.

It is proposed to keep the LDB-2Ref configuration for the class B to E and to use the proposed LDB-1Ref-asymmetric configuration for at least class F, TGM and the newly proposed Gaming Class of sequences [1].

# Discussion

More and more low latency video applications such as cloud gaming are being deployed in the real world. Due to their inherent unicast nature, they are mostly deployed using high density ASIC encoders in the cloud. Following are some of the key factors that we believe are important in achieving successful deployment of such applications:

- Density (4, 8, 16 or more HD encodes per chip)

- Low end-to-end latency (usually sub 80ms)

- Good video quality

- Lower cost per encode

- Reduced carbon emissions

Density is an important factor in scaling cloud gaming solutions. When packing multiple encoding instances per chip, number of references per frame has a direct impact on the density. In most cases, two reference frames per picture is used. However, for some low-cost or high-density encoding chips, one reference frame per picture could be used.

Below six configurations were tested against the anchor (LDB-4Ref) to demonstrate the impact of number of reference frames on the overall performance of an encoder:

- LDB-2Ref

- LDB-1Ref-symmetric

- LDB-1Ref-asymmetric

- LDP-4Ref

- LDP-2Ref

- LDP-1Ref

Tests were performed on VTM-15.0 for four different class of sequences namely, class CTC, class TGM, class gaming [4] and class SA4 gaming. Results are plotted on a BD-rate vs encoder time graph for each class of sequences and are presented in the subsequent section. Additionally, detailed comparison of results can be found in the attached xls.

# Summary of tests:

Figure 1, 2 and 3 shows the snapshot of the coding structure of the LDB-2Ref, LDB-1Ref-symmetric and proposed LDB-1Ref-asymmetric respectively.

Please note that the number of active reference pictures in list 0 and list 1 are same for LDB-1Ref-symmetric and LDB-1Ref-asymmetric. The only difference is that in LDB-1Ref-asymmetric, reference is made to a different frame in the past in list 0 and list 1. Whereas, in LDB-1Ref-symmetric, reference is made to the same frame i.e., the previous frame in list 0 and list 1.

Figure 1: LDB-2Ref: (snapshot from encoder_lowdelay_vtm.cfg after changes)

Figure 2: LDB-1Ref-symmetric: (snapshot from encoder_lowdelay_vtm.cfg after changes)

Figure 3: Proposed LDB-1Ref-asymmetric: (snapshot from encoder_lowdelay_vtm.cfg after changes)

The following table reports BD-rate variations versus the anchor (VTM-15.0 LDB 4Ref):

| Config | Class | BDR Y | BDR U | BDR V | Enc | Dec |
| --- | --- | --- | --- | --- | --- | --- |
| LDB-2Ref (1IP) | Class CTC | 5.52 | 5.97 | 5.93 | 86.4 | 98.1 |
|  | Class TGM | 8.05 | 7.91 | 7.75 | 92.4 | 100.0 |
|  | Class Gaming | 1.61 | 3.12 | 3.33 | 84.5 | 102.0 |
|  | Class SA4 gaming | 3.64 | 4.61 | 4.52 | 83.9 | 100.0 |
| LDB-1Ref-symmetric (1IP) | Class CTC | 18.36 | 21.92 | 22.92 | 82.6 | 102.0 |
|  | Class TGM | 23.28 | 25.50 | 24.54 | 101.0 | 107.0 |
|  | Class Gaming | 9.10 | 16.17 | 17.16 | 74.3 | 94.4 |
|  | Class SA4 gaming | 12.54 | 16.10 | 16.18 | 77.6 | 98.1 |
| Proposed LDB-1Ref-asymmetric (1IP) | Class CTC | 6.32 | 6.88 | 6.64 | 81.5 | 102 |
|  | Class TGM | 9.13 | 8.95 | 8.65 | 94.4 | 110.0 |
|  | Class Gaming | 2.30 | 4.13 | 4.88 | 76.3 | 103.0 |
|  | Class SA4 gaming | 4.62 | 5.41 | 5.14 | 79.3 | 101.0 |

The following figure plots the operating points for the six configurations compared against LDB-4Ref denoted by blue dot.

Following key observations are made based on the above figure:

- LDP-2Ref presents a good BD-rate and encoder time trade-off between LDP-1Ref and LDP-4Ref.

- LDB-1Ref-symmetric (where reference is made to same frame in the past) performed better than LDP-1Ref but at the cost of higher encoding time. The trade-off is not interesting.

- LDB-1Ref-asymmetric (where reference is made to different frame in the past) performed much better than LDB-1Ref-symmetric (where reference is made to same frame in the past) at minimal increase in encoder time, providing a good trade-off.

- LDB-1Ref-asymmetric provides a better trade-off point in gaming sequences compared to the LDB-2Ref

# Proposal

It is proposed to change the LDB-2ref configuration to the LDB-1Ref-asymmetric configuration for at least class TGM, F and the newly gaming class proposed in JVET-Y0041 [1].

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] G. Martin-Cocher, M. badawi, T. Poirier, S. Puri, K. Naser (Interdigital), “AhG-7 Gaming sequences for a new class”, ISO/IEC JTC1/SC29/WG11 JVET-Y0041, Jan 2022

[2] "VTM-cfg," [Online]. https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/master/cfg

[3] G. Martin-Cocher, S. Puri, T. Poirier, K. Naser (InterDigital), J. Xu (Bytedance), D. Nicholson (Ektacom), M. Sychev (Huawei), L. Wang (Nokia), S. Liu, W. Yang (Tencent), J. M. Tiesse (VITEC), M. Karczewicz (Qualcomm), “AHG7: LLCC Scenarios and baseline configurations”, ISO/IEC JTC1/SC29/WG11 JVET-Y0043, Jan 2022

[4] G. Martin-Cocher, M. badawi, T. Poirier, S. Puri, K. Naser (Interdigital), “AhG-7 Gaming sequences for a new class”, ISO/IEC JTC1/SC29/WG11 JVET-Y0041, Jan 2022