| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0117-v2 |
| --- | --- |

| Title: | EE2-2.6: Long-tap interpolation filtering on chroma components |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xi Xie1, Kai Zhang2, Li Zhang2, Junru Li1, Meng Wang1, Shiqi Wang1 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | xixie.jensen@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the test results of EE2-2.6 wherein long-tap interpolation filters are applied on chroma components in motion compensation. On top of ECM-4.0, experimental results are reported as below:

Test #2.6c (6-tap filtering):

RA: {-0.04%, -1.02%, -1.03%}, LB: {}.

# Introduction

In ECM-4.0, a 4-tap interpolation filter is applied on chroma components while a 12-tap interpolation filter is applied on the luma component in the motion compensation process. It was proposed to apply long-tap interpolation filters on chroma components in JVET-Y0172 [1] and the method was decided to be studied in EE2-2.6.

# Proposed methods

This contribution reports the test results of EE2-2.6 wherein long-tap interpolation filters are applied on chroma components in motion compensation. The 6-tap filter coefficients in EE2-2.6c are shown in Table 1. A flag is signaled in SPS to indicate whether the 6-tap interpolation filter or the 4-tap chroma interpolation filter is used. Since the 6-tap filter is more efficient on low-resolution sequences, the original 4-tap chroma interpolation filter is applied for sequences in class A1/A2 while the proposed 6-tap interpolation filter is applied for sequences in other classes, configured by the encoder in the EE tests.

Discrete cosine transform interpolation filter(DCT-IF) has proven to be an effective interpolation filter for motion compensation in H.265 and H.266. In this proposal, the 6-tap filter coefficients is also derived by using DCT-IF for chroma components:

Dk=2Size∙cosπ∙k∙(2∙l-2∙Center+Size)2∙Size

Wk=cosπ∙k∙2∙α-2∙Center+Size2∙Size ; W0=12 ; 1≤k≤Size-1

Wkα=Wkα1+σ∙k2;          σ≥0

Filterlα=k=0Size-1Wkα∙Dlk

Size=Mmin+Mmax-1

Center=Mmin+Mmax2

where Filterlα indicate the filter coefficient for fractional position α. Mmin and Mmax indicate the range of neighboring integer-position samples involved in the interpolation process. Size is the number of reference samples used in the interpolation filter. N is smoothing window size, which is not necessarily an integer. σ is smoothing parameter. And the smoothing parameters are optimized for different phases in this proposal.

(a). Compare with ECM anchor          (b). magnitude response of different phases

Figure 1. The magnitude response

(a). ECM anchor (DCT-IF 4-tap)              (b). VTM anchor (DCT-IF 8-tap)

Figure 2. The phase response of ECM and VTM

Figure 3. The phase response of Our proposal

The frequency response of the interpolation filter is shown in the figure above. As shown in Figure 1, the magnitude of the interpolation filter of this proposal is better than that of the ECM4.0 interpolation filter. Figure 2 shows that for the phase response there are misalignments between the VTM and ECM interpolation filters and the ideal filter. Figure 3 shows that the phase response of the proposed 6-tap interpolation filter is better than the phase response of the VTM and ECM.

# Experimental results

The proposed method is tested on top of ECM-4.0 [2], according to the common test conditions [3]. The simulation results are summarized in Table 2, respectively. The performance of the interpolation filter on class A is also tested. And the simulation results are summarized in Table 3.

It is verified by an internal viewing test that the proposed method shows no obvious impact on the subjective quality. The simulation results of the subjective quality tests are shown from Figures 4 to 15.

# Conclusions

This contribution proposes a 6-tap interpolation filter on chroma components. It is recommended to adopt Test #2.6c (6-tap filter).

Table 1. The coefficients of the 6-tap interpolation filter for chroma components

| Fractional position | Coefficients (6 taps) |
| --- | --- |
| 1/32 | {0, 0, 256, 0, 0, 0}, |
| 2/32 | {1, -6, 256, 7, -2, 0}, |
| 3/32 | {2, -11, 253, 15, -4, 1}, |
| 4/32 | {3, -16, 251, 23, -6, 1}, |
| 5/32 | {4, -21, 248, 33, -10, 2}, |
| 6/32 | {5, -25, 244, 42, -12, 2}, |
| 7/32 | {7, -30, 239, 53, -17, 4}, |
| 8/32 | {7, -32, 234, 62, -19, 4}, |
| 6/32 | {8, -35, 227, 73, -22, 5}, |
| 7/32 | {9, -38, 220, 84, -26, 7}, |
| 8/32 | {10, -40, 213, 95, -29, 7}, |
| 9/32 | {10, -41, 204, 106, -31, 8}, |
| 10/32 | {10, -42, 196, 117, -34, 9}, |
| 11/32 | {10, -41, 187, 127, -35, 8}, |
| 12/32 | {11, -42, 177, 138, -38, 10}, |
| 13/32 | {10, -41, 168, 148, -39, 10}, |
| 14/32 | {10, -40, 158, 158, -40, 10}, |
| 15/32 | {10, -39, 148, 168, -41, 10}, |
| 16/32 | {10, -38, 138, 177, -42, 11}, |
| 17/32 | {8, -35, 127, 187, -41, 10}, |
| 18/32 | {9, -34, 117, 196, -42, 10}, |
| 19/32 | {8, -31, 106, 204, -41, 10}, |
| 20/32 | {7, -29, 95, 213, -40, 10}, |
| 21/32 | {7, -26, 84, 220, -38, 9}, |
| 22/32 | {5, -22, 73, 227, -35, 8}, |
| 23/32 | {4, -19, 62, 234, -32, 7}, |
| 24/32 | {4, -17, 53, 239, -30, 7}, |
| 25/32 | {2, -12, 42, 244, -25, 5}, |
| 26/32 | {2, -10, 33, 248, -21, 4}, |
| 27/32 | {1, -6, 23, 251, -16, 3}, |
| 28/32 | {1, -4, 15, 253, -11, 2}, |
| 31/32 | {0, -2, 7, 256, -6, 1}, |

Table 2. Performance of 6-tap chroma interpolation filter (Test #2.6c)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |  |  |  |  |  |
| Class B | -0.07% | -1.15% | -1.46% | 101% | 104% |  |  |  |  |  |
| Class C | -0.04% | -2.38% | -2.05% | 101% | 103% | -0.11% | -1.94% | -2.66% | 101% | 102% |
| Class E |  |  |  |  |  | -0.05% | 0.40% | 0.76% | 101% | 102% |
| Overall | -0.04% | -1.02% | -1.03% | 100% | 102% |  |  |  |  |  |
| Class D | -0.04% | -3.51% | -3.77% | 100% | 103% | -0.13% | -2.62% | -4.14% | 100% | 103% |
| Class F | -0.02% | -1.07% | -1.18% | 100% | 102% |  |  |  |  |  |

Table 3. Performance of 6-tap chroma interpolation filter (use 6-tap IF for class A)

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.17% | -0.20% | 101% | 104% |  |  |  |  |  |
| Class A2 | -0.05% | -0.92% | -0.90% | 101% | 104% |  |  |  |  |  |
| Class B | -0.07% | -1.15% | -1.46% | 100% | 103% |  |  |  |  |  |
| Class C | -0.04% | -2.38% | -2.05% | 101% | 102% | -0.11% | -1.94% | -2.66% | 101% | 102% |
| Class E |  |  |  |  |  | -0.05% | 0.40% | 0.76% | 101% | 102% |
| Overall | -0.05% | -1.23% | -1.25% | 101% | 103% |  |  |  |  |  |
| Class D | -0.04% | -3.51% | -3.77% | 103% | 104% | -0.13% | -2.62% | -4.14% | 100% | 103% |
| Class F | -0.02% | -1.07% | -1.18% | 101% | 102% |  |  |  |  |  |

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 4. RaceHorses (qp-37, class D)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 5. RaceHorses (qp-22, class D)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 6. BlowingBubbles (qp-37, class D)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 7. BlowingBubbles (qp-22, class D)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 8. BasketballPass (qp-37, class D)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 9. BasketballPass (qp-22, class D)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 10. BQMall(qp-37, class C)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 11. BQMall(qp-22, class C)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 12. BasketballDrive(qp-37, class B)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 13. BasketballDrive(qp-37, class B)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 14. MarketPlace (qp-11, class B)

(a). ECM anchor                           (b). Our proposal(6-tap)

Figure 15. MarketPlace (qp-37, class B)

# References

- X. Xie, K. Zhang, L. Zhang, Junru Li, Meng Wang, Shiqi Wang, “Non-EE2: Long tap interpolation filtering on chroma components,” JVET-Y0172, Jan. 2022.

- ECM-4.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).