| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 26th Meeting, by teleconference, 20–29 April 2022 | Document: JVET-Z0118-r2 |
| --- | --- |

| Title: | AHG 7: GDR Implementation for ECM 4.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seungwook Hong Limin Wang Krit Panusopone | Email: | seungwook.hong@nokia.com limin.2.wang@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution presents a GDR implementation for ECM 4.0 [1]. With this GDR implementation, there is no need to impose constraints on coding tools, which helps to reduce the code size for ECM significantly as compared to VTM-like GDR [3]. In addition, this new GDR implementation provides great flexibility in future tool development for ECM. Specifically, adding of additional new coding tools in ECM code will not affect part of GDR code.

Simulations were conducted under low-delay B configuration specified by LLCC AhG [4]. The partial results (excluding class B Sequence) demonstrate that the overall loss of the new GDR over anchor is only 3.685%,

# Introduction

Versatile Video Coding (VVC) is a new international video coding standard. One of functionalities that VVC supports is Gradual Decoding Refresh (GDR). As the latest video coding standard, VVC employs many new and advanced coding tools. Many of them however may cause leaks for GDR. Hence, constraints have to be imposed on those coding tools, which not only increases the complexity of coding process, but also affects the overall coding performance.

Enhanced Compression Model (ECM) introduces many new coding features and tools on top of VVC. It becomes even more challenging to enable GDR on ECM because whenever new coding tools are adopted into ECM, GDR code has to be modified with adding new code for imposed necessary constraints on those new coding tools.

It is desirable to have a future-proof GDR implementation for ECM, which can immunize from other coding tools. A new GDR without needing imposing of constraints on coding tools was implemented on ECM 3.0. The results (with new ECM 3.0 in-loop filter features off) were reported at the last JVET meeting, Jan 2022. Now, the new GDR is implemented on ECM 4.0, and it can run with all the new ECM 4.0 coding tools, including the new ECM 4.0 in-loop filter features.

# Proposed Solution

Below is a short summary of new GDR implementation.

In the new GDR implementation,

- All types of partitions within a current CTU are allowed, but no CU should span both the clean and dirty areas,

- For CUs in clean (refreshed) area of a current picture, the reconstructed pixels in dirty (non-refreshed) areas of the current picture and reference pictures are considered as “not available”, and if needed, they are set to 2BD-1 , where BD is bit depth, which will give prediction for CUs in clean area of the current picture more freedom over the current picture and reference pictures.

- For CUs in clean area of a current picture, coding information in dirty areas of the current picture and reference pictures are considered “not available”, which will prevent CUs in clean area of the current picture from using coding information in dirty area of the current picture and reference pictures.

- CUs in dirty area of the current picture are allowed to use the reconstructed pixels and the coding information of both clean and dirty areas of the current picture and reference pictures.

- In-loop filters including new ECM 4.0 filter features are performed in both clean and dirty areas, but not across virtual boundaries.

- Decoder can start to decode a bitstream at any GDR picture within a bitstream [5].

As compared to GDR code on ECM 3.0 [2], the new GDR implementation on ECM 4.0 enables all the new ECM in-loop filter tools, including bilateral filter, CCSAO, longer CCALF and alternative band classifier ALF. But, coding information on one side of a virtual boundary are not used for the other side of the virtual boundary. Specifically,

- Bilateral filter (BIF) for both luma and chroma: if pixels on one side of a virtual boundary are affected by the other side of the virtual boundary, BIF is performed for those pixels with padding.

- CCSAO: if pixels on one side of a virtual boundary are affected by the other side of the virtual boundary CCSAO is disabled for the one pixel on each side of the virtual boundary.

- Longer CCALF: if pixels on one side of a virtual boundary are affected by the other side of the virtual boundary, longer CCALF is performed for those pixels with padding.

- Alternative band classifier for ALF (ABC-ALF): if pixels on one side of a virtual boundary are affected by the other side of the virtual boundary, ABC-ALF is performed for those pixels with padding.

# Experimental Results

In evaluating the performance of the new GDR implementation on ECM 4.0, Simulations were conducted over JVET test sequences under low-delay B configuration specified by LLCC AhG [2] with the following modifications,

- The first picture is coded as an IDR picture, followed by the first GDR picture at frame rate + 4,

- GDR period (between two consecutive GDR pictures) is set to (frame rate + 4),

- GDR interval (between GDR picture and associated recovery point picture) is set to frame rate,

- Each inter picture refers up to 2 immediately preceding pictures in decoding order,

- IntraQPOffset and QPoffset are set equal to 0.

All the GDR bitstreams were checked, and none of them shows leak.

For comparison purpose, the anchor bitstreams were also generated under low-delay B configuration specified by LLCC AhG [2], but with the following modifications,

- The first picture is coded as an IDR picture,

- A CRA picture is inserted every (frame rate + 4) pictures,

- Each inter picture refers up to 2 immediately preceding pictures in decoding order,

- IntraQPOffset and QPoffset are set equal to 0.

Note that with the above settings, CRA and GDR pictures in the comparison tests are sync.

Table 1 shows the performance comparison of the new GDR over the anchor. The overall performance loss of the new GDR against the anchor is 3.685%. Note that a loss for GDR should be expected because CUs in clean area do not use any coding information from dirty areas.

Table 1. Simulation Results of new GDR over anchor

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 1.50% | 3.04% | 3.35% | 104% | 109% |
| Class E | 5.87% | 9.76% | 10.79% | 107% | 115% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 2.31% | 5.10% | 6.37% | 102% | 110% |
| Class F | 1.69% | 3.57% | 4.97% | 105% | 98% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Conclusions

The new GDR was implemented on ECM 4.0. Constraints on coding tools are no longer needed for the new GDR implementation, which helps reduce the code size and provide the flexibility in future development of ECM, as compared to VTM-like GDR. Adding of additional new ECM coding tools in the future will not affect the part of GDR code. The partial simulation results demonstrate that the overall loss of the new GDR over the anchor is only 3.685%. It is proposed to integrate this new GDR into ECM code.

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-4.0.

- L. Wang, S. Hong and K. Panusopone, “AHG 7: GDR without encoder constraints for ECM”, JVET-Y0163, Jan. 2022.

- S. Hong, L. Wang, K. Panusopone, “GDR Software”, JVET-U0097, Jan. 2021.

- G. Martin-Cocher, S. Puri, T. Poirier, K. Naser, J. Xu, D. Nicholson, M. Sychev, L. Wang, S. Liu, W. Yang, J.M. Tiesse, and M. Karczewicz, “AHG-7, LLCC Scenarios and baseline configurations”, JVTE -Y0043, Jan. 2022.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/merge_requests/2026

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).